***Sub Engineer***

1. Pick up the correct statement from the following :

a. the lines of sight while observing back sight and fore  sight lie in the same horizontal plane

b. the   staff readings are measurements made vertically downwards from a horizontal plane

c. the horizontal plane with reference to which staff readings are taken, coincides with the level surface through the telescope axis

d. all the above.

2. Pick up the correct statement from the following :

a. The horizontal angle between magnetic meridian and true meridian at a place is called magnetic declination or variance of the compass

b. the imaginary lines which pass through points at which the magnetic declinations are equal at a given time are called isogonic lines

c. the isogonic lines through places at which the declination is zero are termed agonic lines

d. all the above.

3. Surveys which are carried out to provide a national grid of control for preparation of accurate maps of large areas, are known

a. plane surveys

b. geodetic surveys

c. geographical surveys

d. topographical surveys.

4. Keeping the instrument height as 1.5 m, length of staff 4 m, the slope of the ground as 1 in 10, the sight on the down-slope, must be less than

a. 30 m

b. 25 m

c. 20 m

d. 15 m

5. The presence of original rounded surface on the manufactured piece of timber, is called

a. Wane

b. Torn grain

c. Diagonal grain

d. Chipmark.

6. Distemper is

a. a paint consisting of powdered chalk, pigments, and water

b. a water proofing agent

c. a paint consisting of coloured cement and water

d. a drying agent.

7. The most durable varnish is

a. spirit varnish

b. turpentine varnish

c. oil varnish

d. none of these.

8. Pick up the correct statement from the following:

a. The distinct plane of division along which a stone can easily be split, is called natural bed of stone

b. The natural bed of sedimentary rocks is along the planes of stratification

c. The natural bed of igneous rocks is not defined

d. All the above.

9. In plastic analysis, the shape factor for a circular section, is

a. 1.5

b. 1.6

c. 1.7

d. 1.75

10. The locus of reaction of a two hinged semi-circular arch, is

a. straight line

b. parabola

c. circle

d. hyperbola.

11. The section modulus of a rectangular section is proportional to

a. area of the section

b. square of the area of the section

c. product of the area and depth

d. product of the area and width

12. The following assumption is not true in the theory of pure torsion :

a. The twist along the shaft is uniform

b. The shaft is of uniform circular section throughout

c. Cross-section of the shaft, which is plane before twist remains plane after twist

d. All radii get twisted due to torsion.

13. An ideal fluid is

a. incompressible

b. compressible

c. compressible and non-viscous

d. slightly affected by surface torque.

14. Hydraulic radius is equal to

a. area divided by the square of wetted perimeter

b. area divided by wetted perimeter

c. wetted perimeter divided by area

d. square root of the area.

15. Pick up the correct statement from the following :

a. In incompressible flow the density of a fluid remains constant

b. In compressible flow, the density of a fluid changes from point to point

c. In uniform flow, the velocity of a fluid does not change with respect to length of flow direction

d. All the above.

16. The pressure rise due to water hammer depends upon

a. the length of pipe

b. time taken to close the valve

c. the elasticity of the pipe material

d. All of above.

17. A pycnometer is used to determine

a. voids ratio

b. dry density

c. water content

d. density index.

18. The direct shear test suffers from the following disadvantage:

a. Pore water pressure cannot be measured

b. Shear stress on the failure plane is not uniform.

c. The area under the shear and vertical loads does not remain constant throughout the test

d. All the above.

19. The equation τ = C + σ tan φ is given by

a. Rankine

b. Coulomb

c. Culaman

d. Mohr.

20. The relationship between void ratio (e) and porosity ratio (n) is :

a. n =

b. e =

c. n =

d. e = n/(1 - n)

21. The maximum ratio of span to depth of a cantilever slab, is

a. 10

b. 12

c. 14

d. 16

22. A ribbed slab is provided for

a. a plain ceiling

b. thermal insulation

c. acoustic insulation

d. all the above.

23. The design of a retaining wall assumes that the retained earth

a. is free from moisture

b. is not cohesives

c. consists of granular particles

d. all the above.

24. Side face reinforcement shall be provided in the beam when depth of the web in a beam exceeds

a. 50 cm

b. 75 cm

c. 100 cm

d. 120 cm

25. A pointed arch which forms isosceles or equilateral triangle, is generally known as

a. three centred arch

b. two centred arch

c. Lancet arch

d. Bull's eye arch.

26. Rotary drilling

a. is not suitable for deposits containing very coarse gravel

b. hinders the ground water observations and permeability test

c. is not economical for holes of less than 10 cm.

d. all the above.

27. The process of proper and accurate measurement of concrete ingredients for uniformity of proportion, is known

a. curing

b. mixing

c. batching

d. none of these.

28. An aggregate is said to be flaky, if its least dimension is less than

a. 3/4 mean dimension

b. 3/5 mean dimension

c. 5/8 mean dimension

d. none of these.

29. m and n are monsoon duration factor and catchment factor respectively. If P is yearly rainfall in cm, runoff can be calculated by

a. velocity area method

b. weir or spillway method

c. use of venturi-meter

d. using power plant consumption

30. The four major water supply distribution systems, are

a. dead end, tree, grid iron and reticulation

b. dead end, tree, grid iron and circular

c. tree, grid iron, ring and radial

d. tree, reticulation, circular and ring.

31. Aerobic bacterias

a. flourish in the presence of free oxygen

b. consume organic matter as their food

c. oxidise organic matter in sewage

d. All the above.

32. The sewer pipes

a. carry sewage as gravity conduits

b. are designed for generating self-cleansing velocities at different discharge

c. should resist the wear and tear caused due to abrasion

d. all the above.

33. The field capacity of a soil depends upon

a. capillary tension in soil

b. porosity of soil

c. both (a) and (b)

d. neither (a) nor (b).

34. Pick up the incorrect statement from the following

a. Side walls of a venturi head flume are splayed out from the end of the throat at 1 : 10 for a length of 4.5 m

b. Length of side walls should be such that the width of the flume is made equal to 2/3rd the bed width of the distributary

c. Once the width of the flume becomes 2/3rd of the width of the distributary, the splayed walls are increased to 1 in 3 to get full bed width

d. None of these.

35. The top soil of a water logged field becomes more alkaline and infertile if its pH value is

a. 8

b. 9

c. 10

d. 11

36. Side slopes generally kept for canals in loam soil, are :

a. 1 : 1 in cutting and  : 1 in filling

b.  : 1 in cutting and 2 : 1 in filling

c. Neither (a) nor (b)

d. Both (a) and (b)

37. For a poorly graded sub-grade soil, thickness of sub-base, is

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm

38. The wall constructed for the stability of an excavated portion of a road on the hill side, is known as

a. retaining wall

b. breast wall

c. parapet wall

d. all the above.

39. Reinforcement in cement concrete slab of road pavements, is placed

a. transversely

b. longitudinally and transversely

c. in the form of welded wire mesh

d. none of these.

40. Design of flexible pavements is based on

a. mathematical analysis

b. empirical formulae

c. a compromise of pure theory and pure empirical formula

d. none of these.

41. The expected out turn of 2.5 cm cement concrete floor per manson per day

a. 2.5 sqm

b. 5.0 sqm

c. 7.5 sqm

d. 10 sqm

42. Pick up the incorrect statement from the following:

a. The built up covered area at the floor level of any storey of a building is called plinth area

b. The usable covered area of the rooms of any storey of a building is called carpet area

c. The carpet area of a building along with area of its kitchen, pantry, store, lavatory, bath room and glazed verandah, is called floor area

d. None of these.

43. The correct prismoidal formula for volume is

a. D [first area + last area + ∑ Even area + 2 ∑ odd areas]

b.  [first area + last area + 4 ∑ Even area + 2 ∑ odd areas]

c.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas]

d.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas].

44. If tensile stress of a steel rod of diameter D is 1400 kg/cm2 and bond stress is 6 kg/cm2, the required bond length of the rod is

a. 40 D

b. 50 D

c. 53 D

d. 59 D

45. Frederick W. Taylor introduced a system of working known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

46. The main advantage of line organisation, is :

a. defined responsibilities at all levels

b. rigid discipline in the organisation

c. ability of quick decision at all levels

d. all the above.

47. A goiden rule for the procurement of construction stones, suggests

a. 67% at the site and 33% under procurement

b. 50% at the site and 50% under procurement

c. 33% at the site and 67% under procurement

d. none of these.

48. Pick up the correct statement from the following:

a. The duration of time by which an activity can be delayed without affecting the succeeding activity, is called free float

b. The float which affects neither the processor nor the successor activities, is called independent float

c. The difference between total float and free float, is called interfering float

d. All the above.

49. The height of the pilot's eye above the runway surface is assumed

a. 1 m

b. 3 m

c. 4 m

d. 5 m

50. For the proposed runway stated in if the aerodrome reference temperature is 17°.2, the net designed length of the runway is

a. 2716 m

b. 2816 m

c. 2916 m

d. 3016 m