***Sub Engineer***

1. The construction of optical square is based, on the principle of optical

a. reflection

b. refraction

c. double refraction

d. double reflection.

2. If S is the length of a subchord and R is the radius of simple curve, the angle of deflection between its tangent and sub-chord, in minutes, is equal to

a. 573 R/S

b. 171.9 S/R

c. 1718.9 R/S

d. 1718.9 S/R.

3. Probable systematic error in precise levelling as recommended by International Geodetic Association should not exceed (where k is in kilometers.)

a. ± 0.1 k mm

b. ± 0.2 k mm

c. ± 0.1 k

d. 0.2 k mm.

4. If the length of a transition curve to be introduced between a straight and a circular curve of radius 500 m is 90 m, the maximum perpendicular offset for the transition curve, is

a. 1.70 m

b. 2.70 m

c. 3.70 m

d. 4.70 m

5. The steel which contains fissures and cavities, is manufactured by

a. Cementation process

b. Crucible process

c. Bessemer process

d. Open hearth process.

6. The commonly used raw material in the manufacture of cement, is

a. slate

b. sand stone

c. lime stone

d. basalt.

7. For obtaining vinyl chloride acetate, the method used, is

a. addition polymerization

b. condensation polymerization

c. co-polymerization

d. none of these.

8. The hardest rock is

a. marble

b. diamond

c. talc

d. quartz.

9. A steel rod 1 metre long having square cross section is pulled under a tensile load of 8 tonnes. The extension in the rod was 1 mm only. If Esteel = 2 x 106 kg/cm2, the side of the rod, is

a. 1 cm

b. 1.5 cm

c. 2 cm

d. 2.5 cm

10. Pick up the incorrect statement from the following : The torsional resistance of a shaft is directly proportional to

a. modulus of rigidity

b. angle of twist

c. reciprocal of the length of the shaft

d. moment of inertia of the shaft section.

11. A joint of a frame is subjected to three tensile force P, Q and R equally inclined to each other. If P is 10 tonnes, the other forces will be

a. R + 10 tonnes and Q = zero

b. Q + R = 10 tonnes

c. Q - R = zero

d. Q and R each is equal to 10 tonnes.

12. The ratio of the flexural strengths of two square beams one placed with its two sides horizontal and the other placed with one diagonal vertical, diagonal, is

a. 2

b. 2

c. 5

d. 7

13. In C.G.S. system the unit of visocity is

a. joule

b. poise

c. Newton

d. none of these.

14. Maximum efficiency of transmission of power through a pipe, is

a. 25%

b. 33.3%

c. 50%

d. 66.67%.

15. Molecules of fluids get attracted due to

a. surface tension

b. adhesion

c. cohesion

d. none of these.

16. When two layers of a fluid separated by dy move over the other with a difference of velocity dv, causes a shearing stress , where μ is known as

a. absolute viscosity

b. dynamic viscosity

c. viscosity

d. all the above.

17. The minimum centre to centre distance of friction piles of 1 m diameter, is

a. 2 m

b. 2 m to 3 m

c. 3 m to 4 m

d. 5 m

18. The maximum value of effective stress in the past divided by the present value, is defined as over consolidation ratio (OCR). The O.C.R. of an over consolidated clay is

a. less than 1

b. 1

c. more than 1

d. None of these.

19. The ratio of settlement at any time 't' to the final settlement, is known as

a. co-efficient of consolidation

b. degree of consolidation

c. consolidation index

d. consolidation of undisturbed soil.

20. The maximum water content at which a reduction in water content does not cause a decrease in volume of a soil mass, is known

a. liquid limit

b. plastic limit

c. shrinkage limit

d. permeability limit.

21. The maximum area of tension reinforcement in beams shall not exceed

a. 0.15%

b. 1.5%

c. 4%

d. 1%

22. The effective width of a column strip of a flat slab, is

a. half the width of the panel

b. radius of the column

c. diameter of the column

d. none of these.

23. The weight of a foundation is assumed as

a. 5% of wall weight

b. 7% of wall weight

c. 10% of wall weight

d. 12% of wall weight

24. To ensure that the hogging bending moment at two points of suspension of a pile of length L equals the sagging moment at its centre, the distances of the points of suspension from either end, is

a. 0.107 L

b. 0.207 L

c. 0.307 L

d. 0.407 L

25. Pile foundation is generally provided if soil is

a. compressible

b. water logged

c. made up

d. all the above.

26. The triangular portion between any two adjacent arches and the tangent to their crowns, is

a. haunch

b. spandril

c. soffit

d. rise.

27. According to I.S. : 456, the number of grades of concrete mixes, is

a. 4

b. 5

c. 6

d. 7

28. The diameter of the Vicat plunger is 10 mm and its length varies from

a. 30 mm to 40 mm

b. 40 mm to 50 mm

c. 50 mm to 60 mm

d. none of these.

29. Most important method for calculating discharge for planning a water supply project, is

a. velocity area method

b. weir or spillway method

c. use of venturi-meter

d. using power plant consumption

30. By adding 1 ml of orthotolidine solution to 100 ml chlorinated water taken after contact period, the residual chlorine makes the colour of solution

a. yellowish

b. greenish

c. bluish

d. reddish

31. If γw is the unit weight of water, r the hydraulic mean depth of the sewer and S the bed slope of the sewer, then the tractive force exerted by flowing water, is

a. γw . r . S

b. γw r1/2 . S

c. γw r S

d. γw r2/3 S

32. Pick up the correct statement from the following :

a. The screenings are disposed off either by burning or by burial or by dumping

b. The process of burning the screenings, is called incineration

c. The process of burning the screenings, is known as compositing

d. All the above.

33. Solution of Laplacian equation in three dimensions  of water in a syphon, is done by

a. analytical method

b. Khosla's method

c. method of relaxation

d. Unwin's method.

34. Useful soil moisture for plant growth, is

a. capillary water

b. gravity water

c. hygroscopic water

d. chemical water

35. According to Khosla, the exist gradient of surface flow

a. depends upon the b/d ratio

b. is independent of the b/d ratio

c. is independent of the depths of d/s cut off walls

d. none of these.

36. The ratio of the rate of change of discharge of an outlet and parent channel, is known as

a. efficiency

b. sensitivity

c. flexibility

d. modular limit.

37. The minimum vertical clearance of overhanging cliffs or any other structure above the surface of a hill road, should be

a. 4 m

b. 5 m

c. 6 m

d. 4.5 m

38. Minimum radius of a simple circular curve deflecting through 5°, is

a. 1618.9 m

b. 1816.9 m

c. 1718.9 m

d. 1817.9 m

39. The camber on pavements, is provided by

a. parabolic method

b. straight line and parabolic at crown

c. elliptical method

d. circular method.

40. For the movement of vehicles at an intersection of two roads, without any interference, the type of grade separator generally preferred to, is

a. delta

b. trumpet

c. diamond interchange

d. clover leaf.

41. Pick up the correct statement from the following:

a. In a gully trap, a water seal of 6 to 7.5 cm is provided

b. The gully trap collects waste water from the kitchen, sink, wash basins, etc.

c. The gully trap disconnects the sullage drain from the main drainage system

d. The grating provided over gully traps is 23 cm square.

42. The item of steel work which is measured in sq.m, is

a. Rolling shutters

b. Steel doors

c. Ventilators and glazing.

d. All the above.

43. The expected out turn of cement concrete 1 : 2 : 4 per mason per day is

a. 1.5 m3

b. 2.5 m3

c. 3.5 m3

d. 5.0 m3

44. The weight of an item is measured correct to nearest

a. 0.50 kg

b. 0.75 kg

c. 1.00 kg

d. 5 kg

45. Pick up the correct statement from the following:

a. Latest occurrence time is denoted by TL

b. Contractual obligation time is denoted by Ts

c. Latest occurrence time is taken as contractual obligation time

d. All the above.

46. Bar charts are suitable for

a. minor works

b. major works

c. large projects

d. all the Above.

47. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

48. Critical path lies along the activities having total float

a. positive

b. negative

c. zero

d. same.

49. According to I.C.A.O. the slope of transitional surface at right angles to the centre line of runway, is kept

a. 1 in 5

b. 1 in 6

c. 1 in 7

d. 1 in 10

50. The maximum value of the angle of turning of the nose gear large jet aircrafts, is limited to

a. 20°

b. 30°

c. 45°

d. 60°