***Sub Engineer***

1. Which one of the following statements is correct ?

a. when the axes of rotation of the graduated circle and the verniers are not coincident, the instrument possesses eccentricity

b. the mean of the readings of the two verniers gives correct reading free from the eccentricity

c. one vernier may be used if the readings of two verniers differ by a constant

d. all the above.

2. The operation of making the algebraic sum of latitudes and departures of a closed traverse, each equal to zero, is known

a. balancing the sights

b. balancing the departures

c. balancing the latitudes

d. balancing the traverse.

3. The correction to be applied to each 30 metre chain length along θ° slope, is

a. 30 (sec  θ - 1) m

b. 30 (sin  θ - 1) m

c. 30 (cos  θ - 1) m

d. 30 (tan  θ - 1) m

4. Pick up the correct statement from the following :

a. a back sight is the first sight taken after setting up the instrument in any position

b. the first sight on each change point is a fore sight

c. the second sight on each change point is a back sight

d. all the above.

5. Cast steel is manufactured by

a. Cementation process

b. Crucible process

c. Bessemer process

d. Open hearth process.

6. The steel used in R.C.C. work is

a. stainless steel

b. mild steel

c. high carbon steel

d. wrought iron.

7. The commonly used base for iron and steel work, is

a. red lead

b. zinc white

c. white lead

d. titanium white.

8. The filler used in plastic bitumen, is

a. talc powder

b. asbestos powder

c. plastic powder

d. none of these.

9. The ratio of maximum and average shear stresses on a rectangular section, is

a. 1.25

b. 1.5

c. 2.0

d. 2.5

10. Pick up the correct statement from the following:

a. For I sections, the shear centre coincides with the centroid of the cross section of the beam

b. For channels, the shear centre does not concide its centroid

c. Bending loads should pass through the shear centre to avoid twisting

d. All the above.

11. The cross sections of the beams of equal length are a circle and a square whose permissible bending stress are same under same maximum bending. The ratio of their flexural weights is,

a. 1.118

b. 1.338

c. 1.228

d. 1.108

12. The length of a column, having a uniform circular cross-section of 7.5 cm diameter and whose ends are hinged, is 5 m. If the value of E for the material is 2100 tonnes/cm2, the permissible maximum crippling load will be

a. 12.88

b. 128.8 tonnes

c. 288.0

d. none of these.

13. The length of hydraulic jump is roughly

a. 2 to 3 times its height

b. 3 to 5 times its height

c. 5 to 7 times its height

d. None of these.

14. The shape of fire hose nozzle is generally kept

a. divergent

b. convergent

c. convergent divergent

d. cylindrical.

15. The side slope of Cipolletti weir is generally kept

a. 1 to 4

b. 1 to 3

c. 1 to 2

d. 1 : 5

16. The flow is called rotational if its velocity normal to the plane of area is equal to

a. angular velocity vector

b. twice the angular velocity vector

c. thrice the angular velocity vector

d. none of these.

17. A failure wedge develops if a retaining wall

a. moves away from the backfill

b. moves towards the backfill

c. sinks downwards

d. stresses equally by vertical and horizontal forces.

18. Rankine's theory of active earth pressure assumes

a. ground surface is a plane which may be horizontal or inclined

b. back of the wall is vertical and smooth

c. wall yields about the base

d. all the above.

19. Pick up the correct statement from the following :

a. To an agriculturist, soil is the substance existing on the earth's surface, which grows and develops plants

b. To a geologist, soil is the material in a relatively thin surface zone within which roots occur, and rest of the crust is termed as rock irrespective of hardness

c. To an engineer, soil is the unaggregated and uncemented deposits of minerals and organic particles covering the earth's crust

d. All the above.

20. Accurate determination of water content, is made by

a. calcium carbide method

b. sand bath method

c. alcohol method

d. oven-drying method.

21. Total pressure on the vertical face of a retaining wall of height h acts parallel to free surface and from the base at a distance of

a. h/4

b. h/3

c. h/2

d. 2h/3

22. If Ac, Asc and A are areas of concrete, longitudinal steel and section of a R.C.C. column and m and σc are the modular ratio and maximum stress in the configuration of concrete, the strength of column is

a. σcAc + m σcAsc

b. σc(A - Asc) + m σcAsc

c. σc[A + (m - 1)ASC]

d. all the above.

23. According to I.S. : 456, slabs which span in two directions with corners held down, are assumed to be divided in each direction into middle strips and edge strips such that the width of the middle strip, is

a. two-third of the width of the slab

b. three-fourth of the width of the slab

c. four-fifth of the width of the slab

d. three-fifth of the width of the slab.

24. According to I.S. : 456 specifications, the safe diagonal tensile stress for M 150 grade concrete, is

a. 5 kg/cm2

b. 10 kg/cm2

c. 15 kg/cm2

d. 20 kg/cm2

25. According to National Building Code, the hydrants in water mains is provided at minimum interval of

a. 50 m

b. 60 m

c. 75 m

d. 90 m.

26. The form work from the underside of slabs, can be removed only after

a. 1 day

b. 4 days

c. 7 days

d. 14 days.

27. Construction joints are generally provided in concrete

a. retaining walls

b. lining of canals

c. lining of tunnels

d. all the above.

28. Efflorescence in cement is caused due to an excess of

a. iron oxide

b. silica

c. alkalis

d. magnesium oxide.

29. Disinfection of drinking water, is done to remove

a. odour

b. bacterias

c. turbidity

d. colour.

30. Pick up the correct statement from the following :

a. The amount of rainfall in one year, is known as annual rainfall

b. The rain cycle period in India is taken as 35 years

c. The ratio of the actual rainfall in the year to the normal rainfall at a place, is called index of the wetness

d. All the above.

31. Pick up the correct statement from the following :

a. The circular sewers are provided for separate sewerage system

b. The circular sewers work efficiently if the sections run at least half full

c. Two sewers of different shapes are said to be hydraulically equivalent if they discharge at the same rate while running full on the same grade

d. All the above.

32. The sewer pipes

a. carry sewage as gravity conduits

b. are designed for generating self-cleansing velocities at different discharge

c. should resist the wear and tear caused due to abrasion

d. all the above.

33. Water-shed line is abondened for aligning an irrigation canal if

a. water shed forms a sharp loop

b. canal has to take off from a river

c. towns and villages are located on the water shed line

d. all the above.

34. To control the silt entry into a distributary at head regulator, King's vanes are provided which are the walls

a. of height l/3rd to l/4th the depth of water in the main canal

b. spaced at interval of  times their heights

c. extending about one metre beyond a line drawn at an inclination of 2 : 1 to the axis of main canal

d. all the above.

35. Pick up the correct sequence of the part of a canal system from the following

a. Head work-distributary-branch canal-minor

b. Head works-main canal-branch canal-dis-tributary-minor

c. Head works-main canal-branch canal-minor-distributary

d. Heads works-branch canal-main canal distributary, minor.

36. The ratio of the discharge over a trapezoidal crest to a rectangular crest of Sarda falls of identical parameters, is

a. 1.084

b. 1.074

c. 1.064

d. 1.054

37. The absolute minimum sight distance required for stopping a vehicle moving with a speed of 80 km ph, is

a. 120 m

b. 200 m

c. 640 m

d. none of these.

38. Full amount of extra width of a pavement on the curve, is provided at

a. beginning of the transition curve

b. centre of the transition curve

c. beginning of the circular curve

d. centre of the circular curve.

39. To prevent compressive stresses in a rigid concrete pavement, the joint prvoided, is

a. expansion joint

b. contraction joint

c. hinged joint

d. all the above.

40. Customers prefer parking of their vehicles at

a. 85° to aisles

b. 80° to aisles

c. 75° to aisles

d. 60° to aisles

41. The expected out turn for earth work in excavation in ordinary soil per mazdoor per day is

a. 1.00 cubic meter

b. 2.00 cubic meter

c. 3.00 cubic meter

d. 4.00 cubic meter

42. Anti-siphonage pipe is connected to

a. Main soil pipe

b. Bottom of P trap W.C.

c. Top of P trap W.C.

d. Side of water closet.

43. The expected out turn of 2.5 cm cement concrete floor per manson per day

a. 2.5 sqm

b. 5.0 sqm

c. 7.5 sqm

d. 10 sqm

44. If tensile stress of a steel rod of diameter D is 1400 kg/cm2 and bond stress is 6 kg/cm2, the required bond length of the rod is

a. 40 D

b. 50 D

c. 53 D

d. 59 D

45. Pick up the incorrect statement from the following:

a. The difference between the maximum time available for the job and actual time it consumes, is called total float

b. The difference between the latest start time and earliest start time of an activity, is called total float

c. The difference between the earliest finish time of an activity and the earliest start time of its successor activity, is called free float of the activity

d. None of these.

46. Pick up the correct statement from the following:

a. The duration of time by which an activity can be delayed without affecting the succeeding activity, is called free float

b. The float which affects neither the processor nor the successor activities, is called independent float

c. The difference between total float and free float, is called interfering float

d. All the above.

47. Pick up the incorrect statement from the following:

a. The beginning and end of a job, are called events

b. The activity which consumes maximum time, is called a node

c. Logically and sequentially connected activities and events form a network

d. None of these.

48. Critical Path Net Work helps an engineer

a. to concentrate his attention on critical activities

b. to divert the resources from non-critical advanced activities to critical activities

c. to be cautious for avoiding any delay in the critical activities to avoid delay of the whole project

d. all the above.

49. Pick up the correct statement from the following:

a. Landing speed is directly proportional to the wing loading

b. Wing loading remaining constant, the take off distance is directly proportional to the powder loading

c. Neither (a) nor (b)

d. Both (a) and (b)

50. The best direction of a runway is along the direction of

a. longest line on wind rose diagram

b. shortest line on the wind rose diagram

c. line clear of wind rose diagram

d. none of these.