***Sub Engineer***

1. Whole circle bearing of a line is preferred to a quadrantal bearing merely because

a. bearing is not completely specified by an angle

b. bearing is completely specified by an angle

c. Sign of the correction of magnetic declination is different in different quadrants

d. its trigonometrical values may be extracted from ordinary tables easily.

2. For taking offsets with an optical square on the right hand side of the chain line, it is held

a. by right hand upside down

b. by left hand upright

c. by right hand upright

d. by left hand up side down.

3. Pick up the correct statement from the following :

a. spherical aberration may be minimised by replacing the single lens by a combination of the lenses.

b. in telescope objectives, a combination of convex lens and concave lens is used.

c. in eyepieces, two plano-convex lenses placed at a certain distance apart are used

d. all the above.

4. Pick up the correct statement from the following :

a. it is difficult to eliminate an error completely at first trial

b. instability of the instrument makes it almost impossible to adjust it satisfactorily

c. adjustment screws must be left bearing firmly but should never be forced

d. all the above.

5. Pick up the correct statement from the following:

a. The heating of a material to redness in contact with air, is known as calcination

b. The property of lime by which it sets or hardens in damp places having no free circulation of air is called setting

c. The product that remains after calcination of limestone, is called lime

d. All the above.

6. Formula for quick lime, is

a. CaCO3

b. CaO

c. CO3CO2

d. none to these.

7. Lacquer is

a. oil paint

b. distemper

c. spirit varnish

d. none to these.

8. Pick up the polymineralic rock from the following:

a. Pure gypsum

b. Magnesite

c. Granite

d. None of these.

9. The normal component of a force inclined through θ° is obtained by multiplying the force by

a. cos θ

b. tan θ

c. sin θ cos θ

d. sin2θ

10. A body is said to be in equilibrium if

a. it moves horizontally

b. it moves vertically

c. it rotates about its C.G.

d. none of these.

11. In a shaft shear stress intensity at a point is not

a. directly proportional to the distance from the axis

b. inversely proportional to the distance from the axis

c. inversely proportional to the polar moment of inertia

d. directly proportional to the applied torque.

12. A 8 metre long simply supported rectangular beam which carries a distributed load 45 kg/m. experiences a maximum fibre stress 160 kg/cm2. If the moment of inertia of the beam is 640 cm4, the overall depth of the beam is

a. 10 cm

b. 12 cm

c. 15 cm

d. 16 cm

13. The discharge over a Cipolletti weir of length 2.185 m when the head over the weir is 1 m, is

a. 2.0 m3

b. 2.5 m3

c. 3.0 m3

d. 4.0 m3.

14. Total head of a liquid particle in motion is the sum of

a. potential head and kinetic head

b. kinetic head and pressure head

c. potential head and pressure head

d. potential head, kinetic head and pressure head.

15. The best side slope for most economical trapezoidal section, is

a. 30°

b. 45°

c. 60°

d. None of these.

16. When a body is totally or partially immersed in a fluid, it is buoyed up by a force equal to

a. weight of the fluid displaced by the body

b. weight of the body and fluild displaced by the body

c. difference of weights of the fluid displaced and that of the body

d. none of these.

17. The active earth pressure of a soil is proportional to (where φ is the angle of friction of the soil)

a. tan (45° - φ)

b. tan2 (45° + φ/2)

c. tan2 (45° - φ/2)

d. tan (45° + φ)

18. In non-cohesive soil in passive state of plastic equilibrium

a. major principal stress is horizontal

b. minor principal stress is vertical

c. major principal stress is vertical

d. minor and major principal stresses are equally inclined to the horizontal.

19. A phreatic line is defined as the line within a dam section below which there are

a. positive hydrostatic pressure

b. negative hydrostatic pressure

c. negative equipotential lines

d. none of these.

20. For shear strength, triaxial shear test is suitable because

a. precise measurement of the pore pressure and volume change during the test is possible

b. stress distribution on the failure plane, is uniform

c. state of stress within the specimen during any stage of the test as well as at failure, is completely determined

d. all the above.

21. The weight of reinforced concrete, is generally taken as

a. 2300 kg/m3

b. 2400 kg/m3

c. 2500 kg/m3

d. 2800 kg/m3

22. In a prestressed member it is advisable to use

a. high strength concrete only

b. low strength concrete but high tensile steel

c. high strength concrete and high tensile steel

d. high strength concrete but low tensile steel

23. The live load to be considered for an inaccessible roof, is

a. Nil

b. 75 kg/m2

c. 150 kg/cm2

d. 200 kg/m2

24. A continuous beam shall be deemed to be a deep beam if the ratio of effective span to overall depth, is

a. 2.5

b. 2.0

c. less than 2

d. less than 2.5

25. The formWork including the props can be removed from beams, only after

a. 3 day

b. 7 days

c. 14 days

d. 21 days.

26. An arch constructed with finely dressed stones, is known

a. ashlar arch

b. rubble arch

c. gauged arch

d. axed arch.

27. Pick up the incorrect statement from the following:

a. Cement bags should preferably be piled on wooden planks

b. Cement bags should be placed such that bags of one layer does not touch the bags of the adjacent layer

c. Width and height of the pile should not exceed 3 m and 2.70 m respectively

d. None of these.

28. For given workability the grading requiring the least amount of water is one that gives

a. greatest surface area for the given cement and aggregates

b. least surface area for the given cement and aggregates

c. least weight for the given cement and aggregates

d. greatest weight for the given cement and aggregates

29. Asbestos pipes are joined by means of

a. flexible joint

b. dresser coupling joint

c. simplex joint

d. socket and spigot joint.

30. Ground water from artesian wells

a. contains dissolved salts

b. may be saltish and hard

c. generally require lesser treatment

d. all the above.

31. For drains up to 15 cumecs, the depth d and width B are related by

a. d = 0.2 B

b. d = 0.5 B

c. B = 0.2 d

d. B = 0.5 d.

32. The coagulant widely used for sewage treatment, is

a. alum

b. ferric chloride

c. ferric sulphate

d. chlorinated copperas.

33. The structure constructed to allow drainage water to flow under pressure through an inverted syphon below a canal, is called

a. super passage

b. aqueduct

c. super-aqueduct

d. syphon aqueduct.

34. Lacey's equation for calculating flood discharge in rivers, is

a. V = 10.8 R1/2 S1/2

b. V = 10.8 R2/3 S1/2

c. V = 10.8 R2/3 S1/3

d. V = 10.8 R1/3 S2/3

35. Pick up the incorrect statement from the following

a. A bank in the cross-section of a canal is provided on the right

b. A bank in the cross-section of a canal is provided on the left

c. A 'dowla' in the cross-section of a canal is provided on the left

d. None of these.

36. Pick up the correct statement from the following

a. The capacity of escapes should not be less than the capacity of the canal at its location

b. The escapes must lead the surplus water to natural drainages

c. The escapes are aligned to take advantage of contours of lower values

d. All the above.

37. Driving vehicles on wet surfaced roads, is dangerous because it may

a. skid

b. slip

c. overturn

d. all the above.

38. When load is applied on concrete pavement

a. away from edges, the maximum bending moment is negative

b. away from edges, the maximum bending moment cause compression

c. on the edges, the maximum stress is parallel to the edge of the slab

d. none of these.

39. The usual width of side drains along Highways in hilly region, is

a. 60 cm

b. 70 cm

c. 80 cm

d. 100 cm

40. Design of flexible pavement, involves

a. intensity of traffic

b. climate of the region

c. sub-grade conditions

d. all the above.

41. According to ISI method of measurement, the order of the sequence is

a. Length, breadth, height

b. Breadth, length, height

c. Height, length, breadth

d. None of these.

42. Pick up the incorrect statement from the following:

a. No deduction is made for the openings upto 0.1 sq.m

b. No deduction is made for volumes occupied by pipes, not exceeding 100 sq.cm in cross-section

c. No deductions are made for the ends of dissimilar structures up to 500 sq.cm cross-sectional area

d. None of these.

43. While preparing a detailed estimate

a. Dimension should be measured correct to 0.01 m

b. Area should be measured correct to 0.01 sqm

c. Volume should be measured correct to O.Olcum

d. All the above.

44. The volume is measured correct to the nearest

a. 0.01 cum

b. 0.02 cum

c. 0.03 cum

d. 0.04 cum

45. Residential buildings are treated as

a. light construction

b. heavy construction

c. industrial construction

d. private construction.

46. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

47. A critical ratio scheduling

a. determines the status of each activity

b. adjusts automatically changes in activity progress

c. is a dynamic system

d. none of these.

48. Pick up the correct network for the activities of pouring concrete, erection of form work, removal of form work and curing of concrete from the following:

a. 1. Pouring of Concrete  2. Erection of form work 3. Curing of concrete 4. Removal of form work

b. 1. Erection of form work 2. Pouring of concrete 3. Curing of concrete 4. Removal of form work

c. 1. Removal of form work  2. Erection of form work 3. Pouring of concrete 4. Curing of concrete

d. 1. Pouring of concrete 2. Curing of concrete 3. Erection of form work 4. Removal of form work

49. The lift off distance is the distance along the centre of the runway between the starting point and

a. end of the runway

b. end of stopway

c. point where air craft becomes air borne

d. point where air craft attains a height of 10.7 m

50. According to I.C.A.O. all markings on the runways are painted white and on taxiways

a. black

b. red

c. yellow

d. green