***Sub Engineer***

1. If Δ is the angle of deflection of a simple curve of radius R, the length of its long chord, is

a. R cos

b. 2R cos

c. R sin

d. 2R sin .

2. The first reading from a level station is

a. foresight

b. intermediate sight

c. back-sight

d. any sight.

3. To orient a plane table at a point with two inaccessible points, the method generally adopted, is

a. intersection

b. resection

c. radiation

d. two point problem.

4. It is more difficult to obtain good results while measuringhorizontal distance by stepping

a. up-hill

b. down-hill

c. in low undulations

d. in plane areas.

5. A bull nose brick is not used for

a. rounding off sharp corners

b. pillars

c. decoration purpose

d. arches.

6. The main ingredient of a good quality brick earth, is

a. magnesia

b. lime

c. silica

d. alumina.

7. The process of decarbonising the pig iron completely and then adding proper percentage of carbon for manufacturing steel, is called

a. Cementation process

b. crucible process

c. Bessemer process

d. Open hearth process.

8. Pick up the most favourable condition for the rapid growth of fungus for dry rot from the following:

a. dampness

b. presence of sap

c. stagnant air

d. All the above,

9. The shape factor of standard rolled beam section varies from

a. 1.10 to 1.20

b. 1.20 to 1.30

c. 1.30 to 1.40

d. 1.40 to 1.50

10. A steel bar 5 m x 50 mm is loaded with 250, 000 N. If the modulus of elasticity of the material is 0.2 MN/mm2 and Poisson's ratio is 0.25, the change in the volume of the bar is :

a. 1.125 cm3

b. 2.125 cm3

c. 3.125 cm3

d. 4.125 cm2

11. If all the dimensions of a bar are increased in the proportion n : 1, the proportion with which the maximum stress produced in the prismatic bar by its own weight, will increase in the ratio

a. n : 1

b. 1 :

c.  : 1

d. 1 : n.

12. In a three hinged arch, the shear force is usually

a. maximum at crown

b. maximum at springings

c. maximum at quarter points

d. varies with slope.

13. To measure very low pressure, we use

a. barometers

b. piezometers

c. manometers

d. differential manometers.

14. The pressure rise due to water hammer depends upon

a. the length of pipe

b. time taken to close the valve

c. the elasticity of the pipe material

d. All of above.

15. Cappoletti weir is a

a. rectangular weir whose length is kept 3 times the height of the water above sill

b. triangular weir whose notch angle is 90°

c. trapezoidal weir, whose sides slope 1 horizontal to 2 verticals

d. a combination of rectangular and triangular weirs.

16. The ratio of the percentage error in the discharge and percentage error in the measurement of head, over rectangular notch, is

a. 1/2

b. 2/3

c. 3/2

d. 3/4

17. The Terzaghi's general bearing capacity equation for a continuous footing is given by (where Nc, 2V& and Ny are bearing capacity factors.)

a. qf = cNc + γDNq + 0.5γBNγ

b. qf = cNc - γDNq + 0.5γBNγ

c. qf = cNc + γDNq - 0.5γBNγ

d. qf = cNc - γDNq - 0.5γBNγ

18. The bearing capacity of a soil depends upon

a. shape of the particles

b. cohesive properties of particles

c. internal frictional resistance of particles

d. all the above.

19. You are given a sample of soil containing coarse grains to determine its water content, you will use

a. pycnometer

b. oven-drying method

c. calcium carbide method

d. alcohol method.

20. Pick up the incorrect definition from the following:

a. The rotation of soil particles into stable state while remoulding, is known as the thiostropy of soil

b. The water content at which a soil changes from the liquid state to solid state, is known liquid limit of the soil

c. The water content at which a soil flows, is known plastic limit of the soil

d. None of these.

21. The width of the flange of a L-beam, should be less than

a. one-sixth of the effective span

b. breadth of the rib + four times thickness of the slab

c. breadth of the rib + half clear distance between ribs

d. least of the above.

22. Pick up the incorrect statement from the following:

a. In the stem of a retaining wall, reinforcement is provided near the earth side

b. In the toe slab of a retaining wall, rein forcement is provided at the bottom of the slab

c. In the heel slab of a retaining wall, rein forcement is provided at the top of the slab

d. None of these.

23. The effective width of a column strip of a flat slab, is

a. half the width of the panel

b. radius of the column

c. diameter of the column

d. none of these.

24. Piles are usually driven by

a. diesel operated hammer

b. drop hammer

c. single acting steam hammer

d. all the above.

25. Stability of an existing structure may be disturbed by

a. vibrations caused by traffic movements

b. mining in the neighbourhood

c. excavation in the neighbourhood

d. all the above.

26. The sill of a common wooden partition is

a. vertical wooden member on either end

b. lower horizontal wooden member

c. upper horizontal wooden member

d. intermediate horizontal wooden member.

27. For concreting tunnel linings, tran-portation of concrete is done by

a. wheel borrows

b. containers

c. pumps

d. belt conveyors.

28. After casting, an ordinary cement concrete on drying

a. expands

b. mix

c. shrinks

d. none of these.

29. Slow sand filter is used if maximum turbidity of raw water is less than

a. 20 gm/litre

b. 30 gm/litre

c. 40 gm/litre

d. 50 gm/litre.

30. Average annual rainfall at any station is the average of annual rainfall over a period of

a. 14 years

b. 21 years

c. 28 years

d. 35 years.

31. A manhole is generally provided at each

a. junction

b. change of gradient

c. change of sewer diameter

d. all the above.

32. In trickling filter, B.O.D. is reduced to

a. 40 to 60%

b. 60 to 80%

c. 80 to 90%

d. 95%.

33. Side slopes generally kept for canals in loam soil, are :

a. 1 : 1 in cutting and  : 1 in filling

b.  : 1 in cutting and 2 : 1 in filling

c. Neither (a) nor (b)

d. Both (a) and (b)

34. A land is said to be water-logged if its soil pores within

a. a depth of 40 cm are saturated

b. a depth of 50 cm are saturated

c. root zone of the crops are saturated

d. all the above.

35. Which one of the following statements is correct ?

a. The canal system and the drainage system are complementary

b. The canal system runs on ridges, gets divided into smaller channels and spreads its water on the land

c. The drainage system runs along the lowest line, collects drainage water from the slopes and joins the main stream

d. All the above.

36. The state of the soil when plants fail to extract sufficient water for their requirements, is

a. maximum saturated point

b. permanent wilting point

c. ultimate utilisation point

d. none of these.

37. Volume of traffic which is due to improvement carried out in adjacent area, is known as

a. development traffic

b. generated traffic growth

c. normal traffic growth

d. current traffic.

38. Traffic engineering only includes

a. design and application of control devices

b. analysis of traffic characteristics

c. traffic opeation

d. all the above.

39. Minimum number of 50 kg cement bags per cubic metre of concrete for a mix corresponding to crushing strength 280 kg/cm2 at 28 days, are

a. 6.5

b. 7.0

c. 7.5

d. 8.0

40. A gradient along which the vehicle does not require any tractive effort to maintain a specified speed, is known as

a. ruling gradient

b. pushing gradient

c. floating gradient

d. minimum gradient

41. The 'centre line method' is specially adopted for estimating

a. Hexagonal buildings

b. Octagonal buildings

c. Other geometrical shaped buildings

d. All the above.

42. In the mid-section formula

a. The area of mid-sections is calculated by using mean depth

b. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two sections

c. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two original sections

d. (a), (b) and (c) of the above.

43. Pick up the correct statement in case of water supply.

a. Cutting through walls and floors are included with the item

b. Pipes are classified according to their sizes and quality

c. In laying pipes, the method of jointing and fixing is specifically specified

d. All the above.

44. The plinth area of a building not includes

a. area of stair cover

b. internal shaft for sanitary installations up to 2 sq m. in area

c. lift and wall including landing

d. area of cantilevered porch.

45. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

46. A goiden rule for the procurement of construction stones, suggests

a. 67% at the site and 33% under procurement

b. 50% at the site and 50% under procurement

c. 33% at the site and 67% under procurement

d. none of these.

47. Railway projects are treated as

a. light construction

b. heavy construction

c. industrial construction

d. none of these.

48. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

49. An aircraft is flying in an atmosphere of 30°C with a speed of 1260 km ph. Its speed is known as

a. subsonic

b. sonic

c. super-sonic

d. Mach.

50. The lift off distance is the distance along the centre of the runway between the starting point and

a. end of the runway

b. end of stopway

c. point where air craft becomes air borne

d. point where air craft attains a height of 10.7 m