***Sub Engineer***

1. In a lemniscate curve the ratio of the angle between the tangent at the end of the polar ray and the straight, and the angle between the polar ray and the straight, is

a. 3

b. 4

c. 5

d. 3/2

2. There are two stations A and B. Which of the following statements is correct :

a. the fore bearing of AB is AB

b. the back bearing of AB is BA

c. the fore and back bearings of AB differ by 180°

d. all the above.

3. For setting out a simple curve, using two theodolites.

a. offsets from chord produced are required

b. offsets from long chord are required

c. deflection angles from Rankine's formula are required

d. none of these.

4. Locating the position of a plane table station with reference to three known points, is known as

a. intersection method

b. radiation method

c. resection method

d. three point problem.

5. The size of modular bricks, is

a. 10 x 10 x 9 cm

b. 19 x 9 x 9 cm

c. 22.5 x 10 x 8.5 cm

d. 22.5 x 8.0 x 9 cm.

6. A pug mill is used for

a. moulding brick earth

b. tempering brick earth

c. providing brick earth

d. all the above.

7. The rocks in which argil (or clay) predominates, are called

a. sillicious rocks

b. argillaceous rocks

c. calcareous rocks

d. igneous rocks.

8. Dextrin is

a. starch glue

b. albumin glue

c. rubber based adhesive

d. none to these.

9. The general expression for the B.M. of a beam of length l is the beam carries

a. a uniformly distributed load w/unit length

b. a load varying linearly from zero at one end to w at the other end

c. an isolated load at mid span

d. none of these.

10. The horizontal thrust on the ends of a two hinged semicircular arch of radius R carrying

a. a uniformly distributed load ω per unit run over its right half span, is

b. a uniformly distributed load ω per unit run over its entire span is

c. a distributed load varying from zero at the left end to ω per unit horizontal run at the right end, is

d. all the above.

11. The property of a material by which it can be beaten or rolled into thin plates, is called

a. malleability

b. ductility

c. plasticity

d. elasticity.

12. Pick up the incorrect statement

a. A welded joint develops strength of its parent metal

b. Welded joints provide rigidity

c. Welded joints have better finish

d. Welding takes more time than riveting.

13. The discharge through a 100 mm diameter external mouth piece fitted to the side of a large vessel is 0.05948 m3/s. The head over the mouth piece is

a. 2 m

b. 2.5 m

c. 3.0 m

d. 4.0 m.

14. The ratio of frictional factor and coefficient of friction used in general equation for a head loss in a pipe, is

a. 1

b. 2

c. 3

d. 4

15. In flow, the liquid particles may possess

a. potential energy

b. kinetic energy

c. pressure energy

d. all the above.

16. The acceleration f required to accelerate a rectangular tank containing water horizontally so that the slope of its free surface is 45°, is

a. g

b. 2 g

c. 2.5 g

d. 3 g.

17. In a liquid limit test, the moisture content at 10 blows was 70% and that at 100 blows was 20%. The liquid limit of the soil, is

a. 35%

b. 50%

c. 65%

d. none of these.

18. For determining the ultimate bearing capacity of soil, the recommended size of a square bearing plate to be used in load plate test should be 30 to 75 cm square with a minimum thickness of

a. 10 mm

b. 15 mm

c. 20 mm

d. 25 mm

19. Fundamental relationship between dry density (γd), specific gravity (G), water content (ω) and percentage of air voids (na) is :

a. γd =

b. γd =

c. γd =

d. γd =

20. Transporting and redepositing soils, is done by

a. glacier

b. gravity

c. wind

d. all the above.

21. Pick up the correct statement from the following:

a. Lateral reinforcement prevents the shearing of concrete on diagonal plane

b. Lateral reinforcement stops breaking away of concrete cover, due to buckling

c. Lateral reinforcement in R.C.C. columns, is kept not less than 5 mm diameter

d. All the above.

22. If a rectangular prestressed beam of an effective span of 5 meters and carrying a total load 3840 kg/m, is designed by the load balancing method, the central dip of the parabolic tendon should be

a. 10 cm

b. 15 cm

c. 20 cm

d. 25 cm

23. The diameter of the column head support a flat slab, is generally kept

a. 0.25 times the span length

b. 0.25 times the diameter of the column

c. 4.0 cm larger than the diameter of the column

d. 5.0 cm larger than the diameter of the column

24. Design of R.C.C. simply supported beams carrying U.D.L. is based on the resultant B.M. at

a. supports

b. mid span

c. every section

d. quarter span.

25. The maximum bearing capacity of soil is that of

a. loose fine sandy soil

b. dry coarse sandy soil

c. hard rocks

d. soft clay soil.

26. The raft slab is projected beyond the outer walls of the structure by

a. 15 to 20 cm

b. 25 to 30 cm

c. 30 to 45 cm

d. 60 cm.

27. The maximum thickness of concrete floor of a cement warehouse, is

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm

28. For quality control of Portland cement, the test essentially done is

a. soundness

b. tensile strength

c. consistency

d. all the above.

29. Silt storage is the same as

a. dead storage

b. live storage

c. effective storage

d. none of these.

30. After cleaning a slow sand filter, the filtered water is not used for

a. 12 hours to 18 hours

b. 18 hours to 24 hours

c. 24 hours to 36 hours

d. 48 hours.

31. Chlorination of water is done for the removal of

a. bacterias

b. suspended solids

c. sediments

d. hardness.

32. Clogging of sewers, is caused due to

a. low discharge

b. greasy and oily matters

c. domestic wastes thrown in manholes

d. all the above.

33. The down stream expansion head of a guide bank is extended to subtend an angle at the centre, equal to

a. 30°

b. 45°

c. 60°

d. 90°

34. The sensitivity of a rigid module, is

a. 1.50

b. 1.00

c. 0.50

d. zero

35. An outlet is said to be proportional if its flexibility, is

a. zero

b. less than one

c. more than one

d. one

36. If average particle size of the silt in millimetres is m, the Lacey's silt factor f is proportional to

a. m3

b. m1/2

c. m1/3

d. m2/3

37. Customers prefer parking of their vehicles at

a. 85° to aisles

b. 80° to aisles

c. 75° to aisles

d. 60° to aisles

38. The maximum radial distance of a Lemniscate curve, having maximum polar angle a, is

a. 3 R sin2α

b. 3 R sin 2α

c. 3 R sin α/2

d. none of these.

39. During last phase of the reconnaissance, details of the grade line is recorded on 2 metre poles to indicate

a. direction of the proposed alignment

b. distance between the previous and forward pegs

c. relative elevations of pegs

d. all the above.

40. The minimum ratio of the radii of two circular curves of a compound curve, is kept

a. 1.25

b. 1.5

c. 1.75

d. 2.0

41. The 'centre line method' is specially adopted for estimating

a. Hexagonal buildings

b. Octagonal buildings

c. Other geometrical shaped buildings

d. All the above.

42. While preparing a detailed estimate

a. Dimension should be measured correct to 0.01 m

b. Area should be measured correct to 0.01 sqm

c. Volume should be measured correct to O.Olcum

d. All the above.

43. A cement concrete road is 1000 m long, 8 m wide and 15 cm thick over the sub-base of 10 cm thick gravel. The box cutting in road crust is

a. 500 m3

b. 1000 m3

c. 1500 m3

d. 2000 m3

44. The unit of measurement is per quintal for the following:

a. Collapsible gates with rails

b. Rolling shutters

c. Expanded metal wire netting

d. M.S. reinforcement of R.C.C. works.

45. Railway projects are treated as

a. light construction

b. heavy construction

c. industrial construction

d. none of these.

46. Pick up the correct statement from the following:

a. PERT analysis is event oriented

b. CPM does not make any allowance for the uncertainties in the duration of time

c. In CPM, the time is related to cost

d. All the above.

47. A construction schedule is prepared after collecting

a. output of labour

b. output of machinery

c. quantity of various items

d. all the above.

48. Pick up the correct statement from the following:

a. Pessimistic time estimate refers to activities

b. Most likely time estimate refers to activities

c. Expected time estimate refers to activities

d. All the above.

49. If lift off distance of an craft is 2000 m, the clear way at the end of the runway should not be less than

a. 145 m

b. 152.5 m

c. 162.5 m

d. 172.5 m

50. In instrument landing system

a. L.O.M. and L.M.M. are installed on one side and Localizer antenna on the other side

b. L.O.M. and Localizer are installed on one side and L.M.M. on the other side

c. Localizer and L.M.M. are installed on one side and L.O.M. on the other side

d. None of these.