***Sub Engineer***

1. Pick up the correct statement from the following:

a. the theodolite in which telescope can be rotated in vertical plane is called a transit

b. when the vertical circle is to the left of the telescope during observation, it is called to be in left face

c. when the vertical circle is to the right of the telescope during observation, it is called to be in right face

d. all the above.

2. Perpendicular offset from a tangent to the junction of a transition curve and circular curve is equal to

a. shift

b. twice the shift

c. thrice the shift

d. four times the shift.

3. Pick up the correct statement from the following :

a. if the image of the object does not fall on the plane of the cross-lines, parallax exists

b. parallax has nothing to do with the eyepiece

c. the eyepiece is adjusted for clear vision of the cross hairs

d. all the above.

4. In tangential tacheometry, an ordinary level staff is used

a. leaning towards the instrument for inclined sights upward

b. leaning away from the instrument for inclined sights downwards

c. vertical in all cases

d. none of these.

5. The wedging is adopted for quarrying costly stratified rock such as

a. marble

b. limestone

c. sandstone

d. All the above.

6. The rocks which are formed due to pouring of magma at the earth's surface are called

a. Plutonic rocks

b. Hypabyssal rocks

c. Volcanic rocks

d. Igneous rocks

7. Which one of the following is an air binding material ?

a. Gypsum

b. Acid-resistant cement

c. Quick lime

d. All of these.

8. Which one of the following is acid resistant asbestos:

a. amosite asbestos

b. anthophylite asbestos

c. crocidolite asbestos

d. All the above.

9. The locus of the moment of inertia about inclined axes to the principal axis, is

a. straight line

b. parabola

c. circle

d. ellipse

10. A material is said to be perfectly elastic if

a. it regains its original shape on removal of the load

b. It regains its original shape partially on removal of the load

c. it does not regain its original shape at all

d. none of these.

11. A joint of a frame is subjected to three tensile force P, Q and R equally inclined to each other. If P is 10 tonnes, the other forces will be

a. R = 10 tonnes and Q = zero

b. Q + R = 10 tonnes

c. Q - R = zero

d. Q and R each is equal to 10 tonnes.

12. Stress in members of statically determinate simple frames, can be determined by

a. method of joints

b. method of sections

c. graphical solution

d. all the above.

13. 'Flow net' can be drawn only if the flow is

a. turbulent

b. rotational

c. distortion

d. none of these.

14. Discharge over an ogee weir remains the same as that of

a. sharp crested weir

b. triangular weir

c. cippoletti weir

d. drowned weir.

15. In an open tube, free surface of mercury remains

a. horizontal

b. curved upwards

c. curved downwards

d. none of these.

16. The dimensionless parameter not applicable to flowing liquids, is

a. Weber number

b. Pressure coefficient

c. Kinematic viscosity

d. Friction factor.

17. Rankine's theory of active earth pressure assumes

a. ground surface is a plane which may be horizontal or inclined

b. back of the wall is vertical and smooth

c. wall yields about the base

d. all the above.

18. Pick up the correct statement from the following:

a. At critical hydraulic gradient, the saturated sand becomes quick

b. The critical gradient depends on the void ratio and the specific gravity

c. The quick sand occurs more in fine sand and silt than coarse material

d. All the above.

19. A clay subjected to pressure in excess to its present over-burden, is said to be

a. pre-compressed

b. pre-consolidated

c. over-consolidated

d. all the above.

20. The method of the slices is applicable to

a. homogenous soils

b. stratified soils

c. saturated soils

d. non-uniform slopes

21. If diameter of a reinforcement bar is d, the anchorge value of the hook is

a. 8d

b. 12d

c. 16d

d. none of these.

22. In case the factor of safety against sliding is less than 1.5, a portion of slab is constructed downwards at the end of the heel slab, which is known as

a. a key

b. a cut-off wall

c. a rib

d. all the above.

23. In a singly reinforced beam, the effective depth is measured from its compression edge to

a. tensile edge

b. tensile reinforcement

c. neutral axis of the beam

d. longitudinal central axis.

24. An R.C.C. beam of 6 m span is 30 cm wide and has a lever arm of 55 cm. If it carries a U.D.L. of 12 t per m and allowable shear stress is 5 kg/cm2, the beam

a. is safe in shear

b. is safe with stirrups

c. is safe with stirrups and inclined bars

d. needs revision of section

25. In jack arch floor, the rise is kept

a. 1/8th of the span

b. 1/10th of the span

c. 1/12th of the span

d. 1/15th of the span.

26. The entrained concrete is used in lining walls and roofs for making

a. heat insulated

b. sound insulated

c. neither (a) nor (b)

d. both (a) and (b).

27. Workability improved by adding

a. foaming agent

b. oily-agent

c. aluminium compound

d. all the above.

28. The preliminary test is repeated if the difference of compressive strength of three test specimens, exceeds

a. 8 kg/cm2

b. 10 kg/cm2

c. 12 kg/cm2

d. 15 kg/cm2

29. The intake opening is generally covered by a screen to prevent entry of debris etc. and its level is kept

a. at the level of water of the source

b. at the bottom of water of the source

c. at about 2.5 m above the bottom

d. none of these.

30. Surface water may

a. contain large amount of impurities

b. be contaminated by impurities

c. contain disease producing bacterias

d. all the above.

31. The first stage of neutral process of sludge digestion, is

a. acid fermentation

b. acid regression

c. alkaline fermentation

d. none of these.

32. R.M.O. expenses include

a. running expenses

b. maintenance expenses

c. operation expenses

d. all the above.

33. Groynes are generally built

a. perpendicular to the bank

b. inclined up stream up to 30°

c. inclined down stream upto 30°

d. all the above.

34. In a concrete canal the approximate permissible velocity of water should not exceed

a. 1 m/sec

b. 1.5 m/sec

c. 2 m/sec

d. 2.5 m/sec

35. In Montague type fall

a. a straight glacis is provided

b. a circular glacis is provided

c. a parabolic glacis is provided

d. no glacis is provided.

36. A fall in a canal bed is generally provided, if

a. ground slope exceeds the designed bed slope

b. designed bed slope exceeds the ground slope

c. ground slope is practically the same as the designed bed slope

d. none of these.

37. If d is the thickness of a concrete pavement, the equivalent radius b of resisting section for an interior loading, is

a. b = 0.6 + d2 - 0.675 d

b. b = 1.6 + d2 + 0.675 d

c. b = 1.6 + d2 - 3.675 d

d. b = 1.6 + d2 - 0.675 d

38. At intersection of roads, the traffic volume study is carried out to ascertain the number of vehicles

a. moving along straights

b. turning left

c. turning right

d. all the above.

39. The gradients of trace cuts for hilly roads, are kept

a. equal to ruling gradient

b. 10 tO 20 per cent easier than ruling gradients

c. 10 to 20 per cent steeper than ruling gradients

d. none of these.

40. The minimum value of camber provided for thin bituminous surface hill roads, is

a. 2.2%

b. 2.5%

c. 3.0%

d. 3.5%

41. The area is measured correct to the nearest

a. 0.01 sqm

b. 0.02 sqm

c. 0.03 sqm

d. 0.04 sqm

42. The concrete work for the following part of the building of specified thickness is measured in square metres

a. Floors

b. D.P.C.

c. Wall panels

d. All the above.

43. Pick up the correct statement from the following:

a. In order to check up the average depth of excavation, 'Dead mans' are left at the mid-widths of borrow pits

b. The earthwork calculation in excavation is made from the difference in levels obtained with a level

c. The earth work in excavation to form the road embankment includes the formation of correct profile and depositing the soil in layers

d. All the above.

44. According to Indian Standards Institute, the actual size of modular bricks is

a. 23 cm x 11.5 cm x 7.5 cm

b. 25 cm x 13 cm x 7.5 cm

c. 19 cm x 9 cm x 9 cm

d. 20 cm x 10 cm x 10 cm

45. Pick up the correct statement from the following:

a. Latest occurrence time is denoted by TL

b. Contractual obligation time is denoted by Ts

c. Latest occurrence time is taken as contractual obligation time

d. All the above.

46. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

47. Pick up the correct statement from the following:

a. Backward pass is used for calculating the latest occurence time

b. Maximum value of earliest expected time is used if there are more than one value of any event

c. Minimum value of latest occurrence time is used if there are more than one value of any event

d. All the above.

48. Frequency distribution curves

a. having a single lump, are called uninodal curves

b. if symmetrical, are called normal curves

c. if not symmetrical, are called skew curves

d. all the above.

49. The length of a runway under standard atmospheric conditions is 1800 m. If the actual reduced level of the site is 1200 m, the design length of the runway is

a. 2360 m

b. 2460 m

c. 2560 m

d. 2660 m

50. From the end of an instrumental runway, the approach surface rises outwards

a. 1 in 20

b. 1 in 30

c. 1 in 40

d. 1 in 50