***Sub Engineer***

1. The method of reversal

a. is usually directed to examine whether a certain part is truly parallel or perpendicular to another

b. makes the erroneous relationship between parts evident

c. both (a) and (b)

d. neither (a) nor (b).

2. Resolving power of a telescope depends on

a. the diameter of the aperture

b. the pupil aperture of the eye

c. the diameter of the object glass

d. all the above.

3. Pick up the correct statement from the following :

a. spherical aberration may be minimised by replacing the single lens by a combination of the lenses.

b. in telescope objectives, a combination of convex lens and concave lens is used.

c. in eyepieces, two plano-convex lenses placed at a certain distance apart are used

d. all the above.

4. Orientation of a plane table by solving two point problem is only adopted when

a. saving of time is a main factor

b. better accuracy is a main factor

c. given points are inaccessible

d. none of these.

5. Pick up the correct statement from the following:

a. Styrene resin is light in weight

b. Styrene resin transmits ultraviolet waves of light

c. Styrene resin is used to manufacture utensils which are unaffected by chemicals

d. All the above.

6. The softest rock is

a. marble

b. diamond

c. talc

d. quartz.

7. The cracks caused by shrinkage of the exterior surface of the wood exposed to atmosphere, are called:

a. radial shakes

b. heart shakes

c. wind cracks

d. twisted fibres.

8. Pick up the correct statement from the following:

a. The polymer consists of thousands of monomers joined together

b. The polymer molecule is called macro-molecule

c. A polymetric material consists of a large number of long-chain molecules

d. All the above.

9. A close coil helical spring when subjected to a moment M having its axis along the axis of the helix

a. it is subjected to pure bending

b. its mean diameter will decrease

c. its number of coils will increase

d. all the above.

10. A rolled steel joist is simply supported at its ends and carries a uniformly distributed load which causes a maximum deflection of 10 mm and slope at the ends of 0.002 radian. The length of the joist will be,

a. 12 m

b. 14 m

c. 16 m

d. 18 m

11. Columns of given length, cross-section and material have different values of buckling loads for different end conditions. The strongest column is one whose

a. both ends are hinged or pin jointed

b. one end is fixed and the other end entirely free

c. both the ends are fixed

d. none of the these.

12. In a tension test, the yield stress is 300 kg/cm2, in the octa hedral shear stress at the point is:

a. 100 2 kg/cm2

b. 150 2 kg/cm2

c. 200 2 kg/cm

d. 250 2 kg/cm2.

13. The radius of gyration of the water line of a floating ship is 4 m and its metacentric height is 72.5 cm. The period of oscillation of the ship, is

a. 2π

b. 3π

c. 4π

d. π/2

14. The shear stress distribution in viscous fluid through a circular pipe is :

a. maximum at the centre

b. maximum at the inside of surface

c. same throughout the section

d. none of these.

15. The pressure rise due to water hammer depends upon

a. the length of pipe

b. time taken to close the valve

c. the elasticity of the pipe material

d. All of above.

16. The velocity distribution of viscous fluid through a circular/pipe is :

a. hyperbolic

b. circular

c. parabolic

d. elliptical.

17. The shearing strength of a cohesion-less soil depends upon

a. dry density

b. rate of loading

c. confining pressure

d. nature of loading.

18. The maximum shear stress occurs on the filament which makes an angle with the horizontal plane equal to

a. 30°

b. 45°

c. 60°

d. 90°

19. On wetting, cohesive soils,

a. loose permeability

b. gain shear strength

c. loose elasticity

d. decrease their shear strength.

20. The Mohr's straight theory is based on the following fact :

a. Material fails essentially by shear

b. Ultimate strength of the material is determined by the stress in the plane of slip

c. Failure criterion is independent of the intermediate principal stress

d. All the above.

21. A very comfortable type of stairs is

a. straight

b. dog legged

c. geometrical

d. open newel.

22. The stresses developed in concrete and steel in reinforced concrete beam 25 cm width and 70 cm effective depth, are 62.5 kg/cm2 and 250 kg/cm2 respectively. If m = 15, the depth of its neutral axis is

a. 25 cm

b. 30 cm

c. 35 cm

d. 40 cm

23. Minimum spacing between horizontal parallel reinforcement of different sizes, should not be less than

a. one diameter of thicker bar

b. sum of the diameters of ihinner and thicker bars

c. twice the diameter of thinner bar

d. none of these.

24. If At is the gross area of steel in tension, d is the effective depth of the beam and y is the depth of the centre of gravity of the resultant compression, the moment of resistance M of the beam, is given by

a. M = Atf(d - y)

b. M = Atf(d + y)

c. M =

d. none of these.

25. Under reamed piles are generally used for

a. factory buildinp

b. transmission linetowers

c. tall structures.

d. All the above.

26. Weep holes are provided in retaining and breast walls

a. to drain off the water from the filling

b. to ventilate the stone masonry

c. to add architectural beauty

d. to reduce the weight of the earth retained

27. If aggregates completely pass through a sieve of size 75 mm and are retained on a sieve of size 60 mm, the particular aggregate will be flaky if its minimum dimension is less than

a. 30.5 mm

b. 40.5 mm

c. 50.5 mm

d. none of these.

28. The commonly used material in the manufacture of cement is

a. sand stone

b. slate

c. lime stone

d. graphite.

29. The population of a city in 2000 is 50, 000. The average per decade of the previous records of population is 5000 and average percentage per decade is 20%. The population of the city based on geometrical increase method, in the year 2020 will be

a. 60, 000

b. 64, 000

c. 70, 000

d. 72, 000

30. Pressure relief valves are provided in water mains

a. to reduce the pressure

b. at low points

c. upstream of sluice

d. all the above.

31. In the activated sludge process

a. aeration is continued till stability

b. aeration is done with an admixture of previously aerated sludge

c. sludge is activated by constant stirring

d. water is removed by centrifugal action.

32. Pick up the in-correct statement from the following :

a. Manholes are provided in sewer pipes at suitable intervals

b. Catch basins are generally provided in sewers for carrying drainage discharge

c. Inlets are generally provided in all sewers

d. None of these.

33. The structure constructed to allow drainage water to flow under pressure through an inverted syphon below a canal, is called

a. super passage

b. aqueduct

c. super-aqueduct

d. syphon aqueduct.

34. If D is the depth of water upstream of the throat above its sill, B is the width of the throat, to achieve critical flow in an open venturi flume, the theoretical maximum flow Q, is

a. Q = 1.71 BD

b. Q = 1.71 BD3/2

c. Q = 1/71 BD2/3

d. none of these.

35. The useful moisture of soil, is equal to its

a. field capacity

b. saturation capacity

c. moisture content at permanent wilting point

d. difference between filed capacity and permanent wilting point within the root zone of plants.

36. Pick up the correct statement from the following

a. Approach of the water line in a flumed channel section should not be steeper than

b. Departure of the water line from a flumed channel section should not be steeper than 30°

c. Approach of the water line should not be steeper than 30° and departure line not steeper than  in a flumed channel section

d. Approach and departure of the water line in a flumed channel section, should not be steeper than

37. The absolute minimum sight distance required for stopping a vehicle moving with a speed of 80 km ph, is

a. 120 m

b. 200 m

c. 640 m

d. none of these.

38. Excessive camber on pavements may cause

a. deterioration of central portion

b. slip of the speedy vehicles towards the edges

c. erosion of the berms

d. all the above.

39. The normal road land width for a major district road in open area, is

a. 45 m

b. 25 m

c. 15 m

d. 12 m

40. Over taking time required for a vehicle with design speed 50 km ph and overtaking acceleration 1.25 m/sec2 to overtake a vehicle moving at a speed 30 km ph, is

a. 5.0 secs

b. 6.12 secs

c. 225.48 secs

d. 30 secs

41. The detention period in a septic tank is assumed

a. 20 minutes

b. 25 minutes

c. 30 minutes

d. 40 minutes

42. Pick up the item of work not included in the plinth area estimate

a. Room area

b. Verandah area

c. W.C. area

d. Courtyard area.

43. Pick up the excavation where measurements are made in square metres for payment.

a. surface dressing up to 15 cm depths

b. Surface excavation up to 30 cm depths

c. Both (b) and (c)

d. Both (a) and (b)

44. The correct prismoidal formula for volume is

a. D [first area + last area + ∑ Even area + 2 ∑ odd areas]

b.  [first area + last area + 4 ∑ Even area + 2 ∑ odd areas]

c.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas]

d.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas].

45. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

46. The most popular type of organisation used for Civil Engineering Constructions, is

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

47. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

48. If t is the duration of an activity, t1 is the latest finish possible moment of its preceding activity and t2 is the earliest start possible moment, the independent float of the activity is

a. (t1 - t2) - t

b. t - (t1 - t2)

c. (t1 + t2) - t

d. t + (t1 - t2)

49. Pick up the correct statement from the following:

a. Rubberised tar concrete hot blast as well as spillage

b. Epoxy asphalt concrete resists the jet fuels and provides more elastic pavement

c. Epoxy asphalt concrete sets in very small time

d. All the above.

50. Speed of

a. sound varies 2.4 km p.h. per degree centrigrade rise in temperature

b. sound at 0°C is called one Mach

c. an aircraft flying at 2975 km p.h., is called 2.5 Mach

d. All the above.