***Sub Engineer***

1. It is more difficult to obtain good results while measuringhorizontal distance by stepping

a. up-hill

b. down-hill

c. in low undulations

d. in plane areas.

2. Designation of a curve is made by :

a. angle subtended by a chord of any length

b. angle subtended by an arc of specified length

c. radius of the curve

d. curvature of the curve.

3. The area of a plane triangle ABC, having its base AC and perpendicular height h, is

a.  ba sin C

b.  bc sin A

c. S(S - a)(S - b)(S - c) where S is

d. all the above

4. Resolving power of a telescope depends on

a. the diameter of the aperture

b. the pupil aperture of the eye

c. the diameter of the object glass

d. all the above.

5. Mastic asphalt is

a. acid resisting material

b. non-corrosive material

c. corrosive material

d. heating-resisting material.

6. Pozzolana (or surkhi) is used in lime

a. to impart hydraulicity

b. to prevent shrinkage

c. to decrease the cost of construction

d. to decrease the setting time.

7. When a brick is immersed in water for 24 hours and then dried, if

a. no grey or white deposits appear on the surface, the brick is free from soluble salts

b. 10 per cent surface is covered with grey or white deposits, the brick has slight efflorescence

c. 50 per cent surface is covered with grey or white deposits, the brick has serious efflorescence

d. All the above.

8. Pick up the rock which is not  a sedimentary rock from the following:

a. sand stone

b. gypsum

c. dolerite

d. lignite.

9. Stress may be defined as

a. force per unit length

b. force per unit volume

c. force per unit area

d. none of these.

10. In plastic analysis, the shape factor for a circular section, is

a. 1.5

b. 1.6

c. 1.7

d. 1.75

11. Struts are load carrying members of a frame structure which are subjected to

a. axial tension loads

b. axial compressive loads

c. torsional loads

d. transverse loads.

12. A diagram which shows the variations of the axial load for all sections of the span of a beam, is called

a. shear force diagram

b. thrust diagram

c. stress diagram

d. none of these.

13. Equation of continuity of fluids is applicable only if

a. flow is compressive

b. flow is one dimensional

c. velocity is uniform over the cross-sections

d. all the above.

14. The momentum correction factor (β) for the viscous flow through a circular pipe is

a. 1.33

b. 1.50

c. 1.66

d. 2.00

15. The discharge through an internal mouth piece is more if its length is

a. <  diameter

b. < diameter

c. ≥ diameter

d. none of these.

16. For solving the problems is hydraulic engineering, the velocity used is

a. velocity at the centre of pipe section

b. average velocity of flow over a section

c. mean of the velocities at the centre and that along the pipe surface

d. none of these.

17. In a fow net

a. flow lines and equipotential lines cross each other at right angles

b. fields are rectangles whose length is twice the breadth

c. smaller the dimensions of the field, smaller will be the hydraulic gradient and velocity of flow through it

d. for homogeneous soil, the curves are smooth and circular.

18. You are given a sample of soil containing coarse grains to determine its water content, you will use

a. pycnometer

b. oven-drying method

c. calcium carbide method

d. alcohol method.

19. Hydrometer readings are corrected for:

a. meniscus correction

b. dispersing agent correction

c. meniscus and dispersing agent corrections

d. temperature, meniscus and dispersing agent corrections.

20. Coulomb's wedge theory assumes that

a. slip surface is the plane which passes through the heel of the wall

b. sliding wedge itself acts as a rigid body and the value of earth pressure is obtained by considering the limiting equilibrium of the wedge

c. position and direction of the resultant earth pressure, are known

d. all the above.

21. For the design of a simply supported T-beam the ratio of the effective span to the overall depth of the beam is limited to

a. 15

b. 20

c. 25

d. 30

22. If the tendon is placed at an eccentricity e below the centroidal axis of the lon-gitudial axis of a rectangular beam (sectional modulus Z and stressed load P in tendon) the stress at the extreme top edge

a. is increased by

b. is increased by

c. is decreased by

d. remains unchanged.

23. The diameter of the column head support a flat slab, is generally kept

a. 0.25 times the span length

b. 0.25 times the diameter of the column

c. 4.0 cm larger than the diameter of the column

d. 5.0 cm larger than the diameter of the column

24. The design of a retaining wall assumes that the retained earth

a. is free from moisture

b. is not cohesives

c. consists of granular particles

d. all the above.

25. The arrangement of supporting an existing structure by providing supports underneath, is known as

a. shoring

b. underpinning

c. jacking

d. piling

26. The opening provided in sloping roof with its top parallel to the roof surface, is called

a. dormer window

b. sky light window

c. lantern window

d. louvered window.

27. Ordinary concrete is not used for concrete grade

a. M 150

b. M 200

c. M 250

d. M 400

28. Internal friction between the ingredients of concrete, is decreased by using

a. fine aggregates

b. rich mix

c. more water and coarse aggregates

d. none of these.

29. The factor affecting per capita demand, is

a. climatic conditions

b. pressure in water mains

c. cost of water

d. all the above.

30. Most commonly used section in grade aqueducts, is

a. circular

b. rectangular

c. parabolic

d. horse shoe section.

31. If the grit in grit chambers is 4.5 million litres per day, its cleaning is done

a. manually

b. mechanically

c. hydraulically

d. electrically.

32. The most efficient cross-section of sewers in a combined sewerage system is

a. parabolic

b. circular

c. rectangular

d. new egged.

33. The uplift pressure on the roof of an inverted syphon, is maximum when

a. drain in running dry

b. canal is running dry

c. canal is running with F.S.L.

d. drain is running with H.F.L.

34. Groynes are generally built

a. perpendicular to the bank

b. inclined up stream up to 30°

c. inclined down stream upto 30°

d. all the above.

35. Lane's weighted creep theory assumes

a. double weightage to horizontal creep and one weightage to vertical creep

b. triple weightage to horizontal creep and one weightage to vertical creep

c. triple weightage to vertical creep and one weightage to horizontal creep

d. double weightage to vertical creep and one weightage to horizontal creep.

36. To hold hydraulic jumps, baffle walls are provided in

a. Sarda type falls

b. English type falls

c. Montague type falls

d. Vertical type falls.

37. The minimum value of camber provided for thin bituminous surface hill roads, is

a. 2.2%

b. 2.5%

c. 3.0%

d. 3.5%

38. When each particle of aggregates is thinly coated with cement paste, a heterogeneous solid is formed, which is known as

a. hydration

b. gel

c. concrete

d. none of these.

39. To indicate proper control of consistency of a freshly mixed concrete for pavement construction, the slump should be between

a. 4 to 6 cm

b. 5 to 7 cm

c. 7 to 10 cm

d. 10 to 12 cm

40. Following type of pavement is generally known as flexible pavement

a. water-bound macadam roads

b. stabilised soil roads

c. road constructed with various layers of building material well compacted

d. all the above.

41. Pick up the correct statement from the following:

a. Area of canal in cutting = BD + Sd2 where B = bed width, d = depth of cutting and S is the side slope

b. Area of the bank of canal = B1d1 + Sd2 where B1, d1 and S are the width of bank, height of the bank above N.S.L. and side slope respectively

c. If F.S.L. is above N.S.L the canal is called partly in cutting and partly in filling and berms are designed as 2d where d is full supply depth

d. All the above.

42. Pick up the excavation where measurements are made in square metres for payment.

a. surface dressing up to 15 cm depths

b. Surface excavation up to 30 cm depths

c. Both (b) and (c)

d. Both (a) and (b)

43. The measurement is made for stone work in square metre in case of

a. Columns, lintels, copings

b. Building work

c. Dressed stones in Chajjas

d. (a) and (d) of the above.

44. The most reliable estiamte is

a. Detailed estimate

b. Preliminary estimate

c. Plinth area estimate

d. Cube rate estimate

45. Military organisation is known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. none of these.

46. Pick up the incorrect statement from the following:

a. The tail of the arrow indicates the start of the activity

b. The head of the arrow indicates the end of the activity

c. The arrows are drawn to scale from left to right

d. Each activity consumes a given time.

47. Pick up the correct statement from the following:

a. Programme Evaluation and Review Technique, is event oriented

b. Programme Evaluation and Review Technique is not event oriented

c. Critical Path Method is event oriented

d. Critical Path method is event oriented.

48. The main disadvantage of line organisation, is

a. rigid structure

b. extraordinary delay in communications

c. top level executions over work

d. all the above.

49. According to the International Civil Aviation Organisation (I.C.A.O.) the strength of runway pavements, have been coded by

a. Last Seven English alphabets

b. First Seven English alphabets

c. First seven numbers

d. None of these.

50. The distance between main gears is 10 m, and the distance of nose gear from centre of main gears is 30 m. If the angle of turning is 60°, the distance of centre of rotation from the nearer main gear, is

a. 12.30 m

b. 11.30 m

c. 10.30 m

d. 9.30 m