***Sub Engineer***

1. Pick up the correct statement from the following :

a. it is difficult to eliminate an error completely at first trial

b. instability of the instrument makes it almost impossible to adjust it satisfactorily

c. adjustment screws must be left bearing firmly but should never be forced

d. all the above.

2. The sum of the interior angles of a geometrical figure laid on the surface of the earth differs from that of the corresponding plane figure only to the extent of one second for every

a. 100 sq. km of area

b. 150 sq. km of area

c. 200 sq. km of area

d. none of these.

3. Cross hairs in surveying telescopes, are fitted

a. in the objective glass

b. at the centre of the telescope

c. at the optical centre of the eye piece

d. in front of the eye piece.

4. Pick up the correct statement from the following :

a. the lines of sight while observing back sight and fore  sight lie in the same horizontal plane

b. the   staff readings are measurements made vertically downwards from a horizontal plane

c. the horizontal plane with reference to which staff readings are taken, coincides with the level surface through the telescope axis

d. all the above.

5. Invar contains

a. 12% of nickel

b. 24% of nickel

c. 30% to nickel

d. 36% of nickel.

6. Teak wood is suitable for

a. sports articles

b. furnitures

c. railway sleepers

d. all the above.

7. Cast iron is used for

a. structural works in beams

b. small sized water pipes

c. columns and struts

d. none to these.

8. Pick up the constituent of good brick earth whose excess causes the raw bricks shrink and warp during drying and burning, from the following:

a. Alumina

b. Lime

c. Ironoxide

d. Magnesia.

9. The normal and tangential components of stress on an inclined plane through θ° to the direction of the force, will be equal if θ is

a. 45°

b. 30°

c. 60°

d. 90°

10. At any point of a beam, the section modulus may be obtained by dividing the moment of inertia of the section by

a. depth of the neutral axis

b. maximum tensile stress at the section

c. maximum compressive stress at the section

d. none of these.

11. If the width of a simply supported beam carrying an isolated load at its centre is doubled, the deflection of the beam at the centre is changed by

a. 1/2

b. 1/8

c. 2

d. 8

12. The moment diagram for a cantilever carrying a concentrated load at its free end, will be

a. triangle

b. rectangle

c. parabola

d. cubic parabola.

13. Most economical section of a triangular channel, is

a. equilateral triangle

b. right angled triangle

c. isosceles triangle with 45° vertex angle

d. right angled triangle with equal sides.

14. The ratio of maximum velocity to average velocity of viscous fluid through a circular pipe is

a. 0.5

b. 0.75

c. 1.25

d. 2.00

15. With a clinging nappe of a weir, the excess discharge, is

a. 8% to 10%

b. 18% to 20%

c. 25% to 30%

d. 30% to 40%.

16. The width of a weir with end contraction, is

a. equal to the width of the channel

b. less than the width of the channel

c. half the width of the channel

d. none of these.

17. When the seepage pressure becomes equal to the pressure due to submerged weight of a soil, the effective pressure is reduced to zero and the soil particles have a tendency to move up in the direction of flow. This phenomenon is generally known

a. quick condition

b. boiling condition

c. quick sand

d. all the above.

18. Si particles

a. show dilatancy

b. swell when moist

c. possess high strength when dry

d. disintegrate easily.

19. The density of soil can be increased

a. by reducing the space occupied by air

b. by elastic compression of soil grains

c. by expelling water from pores

d. All the above.

20. Under-reamed piles are generally

a. driven piles

b. bored piles

c. precast piles

d. all the above.

21. If C is creep coefficient, f is original prestress in concrete, m is modular ratio, E is Young's modulus of steel and e is shrinkage strain, the combined effect of creep and shrinkage is:

a. (1 - C)mf - eE

b. (C - 1)mf + eE

c. (C - 1)mf - eE

d. (1 - C)mf + eE

22. If the width of the foundation for two equal columns is restricted, the shape of the footing generally adopted, is

a. square

b. rectangular

c. trapezoidal

d. triangular.

23. The design of heel slab of a retaining wall, is based on the maximum bending moment due to:

a. weight of the soil above it,

b. load of the surcharge, if any

c. upward soil reaction

d. all the above.

24. If the loading on a prestressed rectangular beam, is uniformly distributed, the tendon to be provided should be .

a. parabolic with convexity downward

b. parabolic with convexity upward

c. straight above centroidal axis

d. none of these.

25. If the depth of an excavation is 20 metres, number of single stage well points to be installed at various levels, is

a. 4

b. 3

c. 2

d. 6.

26. A cut in frame of a door to receive the shutter, is called

a. louver

b. stop

c. horn

d. rebate.

27. For preparing ordinary concrete, the quantity of water used, is

a. 10% by weight of aggregates plus 10% of weight of cement

b. 5% by weight of aggregates plus 30% of weight of cement

c. 30% by weight of aggregates plus 10% of weight of cement

d. none of these.

28. The final operation of finishing floors, is known as

a. floating

b. trowelling

c. finishing

d. all the above.

29. Disinfection of drinking water, is done to remove

a. odour

b. bacterias

c. turbidity

d. colour.

30. Disinfection of drinking water is done to remove

a. odour

b. colour

c. bacterias

d. all the above.

31. The following is the physical characteristic of sewage

a. colour

b. odour

c. temperature

d. all the above.

32. 'Cowl' is provided at

a. lower end of ventilating column

b. upper end of ventilating column

c. upper end of the manhole

d. first step in manhole.

33. The ratio of the rate of change of discharge of an outlet and parent channel, is known as

a. efficiency

b. sensitivity

c. flexibility

d. modular limit.

34. The length and width of a meander and also the width of the river, vary roughly as

a. square root of the discharge

b. discharge

c. square of the discharge

d. cube of the discharge.

35. Process of meandering is due to

a. sediment load of streams

b. discharge and hydraulic properties of streams

c. relative erodibility of the bed and banks

d. all the above.

36. Pick up the correct statement from the following:

a. The full supply level of a canal should be above ground level

b. According to Lacey, regime conditions require a particular slope for a given discharge and silt factor

c. In case the ground slope is less than the required bed slope, the silt factor must be reduced by permitting the entry of coarse silt

d. All the above.

37. During last phase of the reconnaissance, details of the grade line is recorded on 2 metre poles to indicate

a. direction of the proposed alignment

b. distance between the previous and forward pegs

c. relative elevations of pegs

d. all the above.

38. If d is the thickness of a concrete pavement, the equivalent radius b of resisting section for an interior loading, is

a. b = 0.6 + d2 - 0.675 d

b. b = 1.6 + d2 + 0.675 d

c. b = 1.6 + d2 - 3.675 d

d. b = 1.6 + d2 - 0.675 d

39. In a braking test, a vehicle travelling at 36 km ph was stopped at a braking distance of 8.0 m. The average value of the vehicle's skid resistance (friction coefficient) is

a. 0.64

b. 6.25

c. 0.16

d. none of these

40. Pick up the incorrect statement from the following:

a. Tresguet did not provide the top camber for the drainage of surface water

b. Tresguet provided the top camber for the drainage of surface water

c. Telford provided two layers of stones in the central 5.4 m width and one layer was provided on the sides

d. Macadam provided a camber to the formation at the dug-up state, to drain percolated water.

41. Carpet area does not include the area of

a. verandah, corridor and passage

b. bath room and lavatory

c. kitchen and pantry

d. All the above.

42. The item of steel work which is measured in sq.m, is

a. Rolling shutters

b. Steel doors

c. Ventilators and glazing.

d. All the above.

43. Pick up the excavation where measurements are made in square metres for payment.

a. surface dressing up to 15 cm depths

b. Surface excavation up to 30 cm depths

c. Both (b) and (c)

d. Both (a) and (b)

44. Berms are provided in canals if these are

a. fully in excavation

b. partly in excavtion and partly in embankment

c. fully in embankment

d. All the above.

45. Railway projects are treated as

a. light construction

b. heavy construction

c. industrial construction

d. none of these.

46. Pick up the correct statement from the following:

a. PERT analysis is event oriented

b. CPM does not make any allowance for the uncertainties in the duration of time

c. In CPM, the time is related to cost

d. All the above.

47. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

48. The time by which activity completion time can be delayed without affecting the start of succeeding activities, is known as

a. duration

b. total flat

c. free float

d. interfering float.

49. According to the International Civil Aviation Organisation (I.C.A.O.) the strength of runway pavements, have been coded by

a. Last Seven English alphabets

b. First Seven English alphabets

c. First seven numbers

d. None of these.

50. The maximum value of the angle of turning of the nose gear large jet aircrafts, is limited to

a. 20°

b. 30°

c. 45°

d. 60°