***Sub Engineer***

1. The radius of a simple circular curve is 300 m and length of its specified chord is 30 m. The degree of the curve is

a. 5.73°

b. 5.37°

c. 3.57°

d. 3.75°.

2. If the declination of the needle is 10° W

a. each of the whole circle reckoning has to be micros by 10°

b. in the quadrantal method, the correction is positive in the 1st and 3rd quadrants

c. in the quadrantal method, the corrections is negative in 2nd and 4th quadrants

d. all the above.

3. Two hill tops A and B 20 km apart are intervened by a third top C. If the top most contour of the three hill tops are of the same value, state whether the line of sight AB

a. passes clear of hill top C

b. passes below the hill top C

c. grazes the hill top C

d. none of these.

4. An imaginary line lying throughout on the surface of the earth and preserving a constant inclination to the horizontal, is called

a. contour line

b. contour gradient

c. level line

d. line of gentle scope.

5. Basalt is

a. sedimentary rock

b. metamorphic rock

c. extrusive igneous rock

d. intrusive igneous rock.

6. The rock generally used for roofing, is

a. granite

b. basalt

c. slate

d. pumice.

7. The yield strength and tensile strength of low carbon steel may be improved by the addition of

a. chromium

b. nickel

c. venadium

d. tungsten.

8. Pick up the most favourable condition for the rapid growth of fungus for dry rot from the following:

a. dampness

b. presence of sap

c. stagnant air

d. All the above,

9. Pick up the correct statement from the following:

a. The structural member subjected to compression and whose dimensions are small as compared to its length, is called a stmt

b. The vertical compression members, are generally known as columns or stanchions

c. Deflection in lateral direction of a long column, is generally known as buckling

d. All the above.

10. Pick up the incorrect statement from the following : The torsional resistance of a shaft is directly proportional to

a. modulus of rigidity

b. angle of twist

c. reciprocal of the length of the shaft

d. moment of inertia of the shaft section.

11. If the width of a simply supported beam carrying an isolated load at its centre is doubled, the deflection of the beam at the centre is changed by

a. 1/2

b. 1/8

c. 2

d. 8

12. The weakest section of a diamond riveting, is the section which passes through

a. first row

b. second row

c. central raw

d. one rivet hole of end row.

13. In Newton's viscosity equation , the coefficient (μ) is known as coefficient of

a. absolute viscosity

b. dynamic viscosity

c. simple viscosity

d. all the above.

14. Gauge pressure is

a. absolute pressure - atmospheric pressure

b. absolute pressure + atmospheric pressure

c. atmospheric pressure - absolute pressure

d. none of these.

15. In two dimensional flow the components of velocity are given by u = ax; v = by. The stream lines will be

a. circular

b. parabolic

c. hyperbolic

d. elliptical.

16. When water flows over a rectangular suppressed weir, the negative pressure created beneath the nappe

a. increases the discharge

b. decreases the discharge

c. does not effect the discharge

d. none of these.

17. A soil mass coated with a thin layer of paraffin weighs 460 g. When immersed, it displaces 299 cc of water. The weight of paraffin is 10 g. If specific gravity of solids is 2.5 and that of paraffin 0.9, the void ratio of soil, is

a. 0.55

b. 0.60

c. 0.65

d. 0.70

18. Pick up the correct statement from the following :

a. To an agriculturist, soil is the substance existing on the earth's surface, which grows and develops plants

b. To a geologist, soil is the material in a relatively thin surface zone within which roots occur, and rest of the crust is termed as rock irrespective of hardness

c. To an engineer, soil is the unaggregated and uncemented deposits of minerals and organic particles covering the earth's crust

d. All the above.

19. Fundamental relationship between dry density (γd), bulk density (γ) and water content (ω), is :

a. γ =

b. γd =

c. ω =

d. ω =

20. A sample of saturated soil has 30% water content and the specific gravity of soil grains is 2.6. The dry density of the soil mass in g/cm3, is

a. 1.47

b. 1.82

c. 1.91

d. none of these.

21. A very comfortable type of stairs is

a. straight

b. dog legged

c. geometrical

d. open newel.

22. If depth of slab is 10 cm, width of web 30 cm, depth of web 50 cm, centre to centre distance of beams 3 m, effective span of beams 6 m, the effective flange width of the beam, is

a. 200 cm

b. 300 cm

c. 150 cm

d. 100 cm

23. An R.C.C. column of 30 cm diameter is reinforced with 6 bars 12 mm φ placed symmetrically along the circumference. If it carries a load of 40, 000 kg axially, the stress is

a. 49.9 kg/cm2

b. 100 kg/cm2

c. 250 kg/cm2

d. 175 kg/cm2

24. If the maximum shear stress at the end of a simply supported R.C.C. beam of 6 m effective span is 10 kg/cm2, the share stirrups are provided for a distance x from either end where x is

a. 50 cm

b. 100 cm

c. 150 cm

d. 200 cm

25. The type of arch used for high class buildings where appearance is of prime importance, is known as

a. ashlar arch

b. rubble arch

c. gauged brick arch

d. axed brick arch.

26. Couple roof is used for spans

a. 3.5 m or less

b. 3.5 m but less than 5 m

c. 5 m but less than 6.5 m

d. 6.5 m but less than 8 m.

27. The ratio of various ingredients (cement, sand, aggregates) in concrete of grade M 200, is

a. 1 : 3 : 6

b. 1 :   : 3

c. 1 : 1 : 2

d. 4 : 2 : 1

28. Concrete containing

a. silicious aggregates, has higher co-efficient of expansion

b. igneous aggregates, has intermediate coefficient of expansion

c. lime stones, has lowest co-efficient of expansion

d. All the above.

29. m and n are monsoon duration factor and catchment factor respectively. If P is yearly rainfall in cm, runoff can be calculated by

a. velocity area method

b. weir or spillway method

c. use of venturi-meter

d. using power plant consumption

30. Asbestos pipes are joined by means of

a. flexible joint

b. dresser coupling joint

c. simplex joint

d. socket and spigot joint.

31. Disposal to sewage in large cities, is done in

a. irrigation

b. dilution

c. oxidation

d. putrifaction.

32. The sewer pipe which carries sewage from a building for immediate disposal is

a. house sewer

b. lateral sewer

c. intercepting sewer

d. main sewer.

33. The sinuosity of a meander is the ratio of

a. meander length and the width of meander

b. meander length and half width of the river

c. curved length and the straight distance

d. none of these.

34. For a given discharge in a channel, Blench curves give the relationship between the loss of head (HL) and

a. specific energy up-stream

b. specific energy down-stream

c. critical depth of water down-stream

d. depth of water down-stream

35. The length and width of a meander and also the width of the river, vary roughly as

a. square root of the discharge

b. discharge

c. square of the discharge

d. cube of the discharge.

36. Lacey's equation for calculating flood discharge in rivers, is

a. V = 10.8 R1/2 S1/2

b. V = 10.8 R2/3 S1/2

c. V = 10.8 R2/3 S1/3

d. V = 10.8 R1/3 S2/3

37. The difference in gradients after full super-elevation and the initial alignment of a road, is known as

a. rising gradient

b. compensated gradient

c. differential gradient

d. none of these.

38. Maximum number of passenger cars that can pass a given point on a road during one hour under the most ideal road way and traffic conditions, is known as

a. traffic density

b. basic capacity of traffic lane

c. possible capacity of traffic lane

d. all the above.

39. Minimum thickness of the base of a flexible pavement, is

a. 10 cm

b. 15 cm

c. 20 cm

d. 25 cm

40. Enoscope is used to determine

a. spot speed

b. average speed

c. travel time

d. none of these.

41. Pick up the correct statement from the following:

a. Plastering is measured in sq.m

b. Glazing is measured in sq.m

c. Striking is measured in sq.m

d. All the above.

42. Pick up the incorrect statement regarding a master trap from the following :

a. It is provided a cleaning eye at the top of the trap

b. The height of fresh air inlet pipe fixed vertically with wall is 3 m

c. The mica flap valve which opens inwards only, is fitted at the top of the inlet pipe

d. The water seal is less than that of ordinary traps.

43. Pick up the correct statement from the following:

a. In a gully trap, a water seal of 6 to 7.5 cm is provided

b. The gully trap collects waste water from the kitchen, sink, wash basins, etc.

c. The gully trap disconnects the sullage drain from the main drainage system

d. The grating provided over gully traps is 23 cm square.

44. While estimating a reinforced cement structure, the omitted cover of concrete is assumed

a. at the end of reminforcing bar, not less than 25 mm or twice the diameter of the bar

b. in thin slabs, 12 mm minimum or diameter of the bar whichever is more

c. for reinforcing longitudinal bar in a beam 25 mm minimum or diameter of the largest bar which is more

d. All the above.

45. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

46. Frequency distribution curves

a. having a single lump, are called uninodal curves

b. if symmetrical, are called normal curves

c. if not symmetrical, are called skew curves

d. all the above.

47. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

48. Residential buildings are treated as

a. light construction

b. heavy construction

c. industrial construction

d. private construction.

49. The lift off distance is the distance along the centre of the runway between the starting point and

a. end of the runway

b. end of stopway

c. point where air craft becomes air borne

d. point where air craft attains a height of 10.7 m

50. Two single runways may be arranged so as to have

a. L-shape

b. T-shape

c. X-shape

d. All the above.