***Sub Engineer***

1. Surveys which are carried out to depict mountains, rivers, water bodies, wooded areas and other cultural details, are known as

a. city surveys

b. topographical surveys

c. guide map surveys

d. plane surveys.

2. To avoid large centering error with very short legs, observations are generally made

a. to chain pins

b. by using optical system for centering the theodolite

c. to a target fixed on theodolite tripod on which theodolite may be fitted easily

d. all the above.

3. Greater accuracy in linear measurements, is obtained by

a. tacheometry

b. direct chaining

c. direct taping

d. all the above.

4. Horizontal distances obtained tacheometerically are corrected for

a. slope correction

b. temperature correction

c. refraction and curvature correction

d. all the above.

5. Pick up the metal refractory from the following:

a. Molyblendum

b. Tungsten

c. Zicronium

d. All of these.

6. Wrought iron contains carbon upto

a. 0.25%

b. 1.0%

c. 1.5%

d. 2%.

7. Initial setting of cement is caused due to

a. Tri-calcium silicate

b. Di-calcium silicate

c. Tri-calcium aluminate

d. Tetra calcium alumino ferrite.

8. Geologically, marble is known as

a. sedimentary rock

b. igneous rock

c. metamorphic rock

d. stratified rock.

9. At yield point of a test piece, the material

a. obeys Hooke's law

b. behaves in an elastic manner

c. regains its original shape on removal of the load

d. undergoes plastic deformation.

10. Flat spiral springs

a. are supported at outer end

b. are wound by applying a torque

c. are used in clock-work mechanism

d. all the above.

11. The ratio of the moment of inertia of a circular plate and that of a square plate for equal depth, is

a. equal to one

b. more than one

c. equal to 3π/16

d. none of these.

12. The width b and depth d of a beam cut from a wooden cylindrical log of 100 cm diameter for maximum strength are :

a. b = 57.73 cm d = 81.65 cm

b. b = 81.65 cm d = 57.73 cm

c. b = 50.00 cm d = 50.00 cm

d. b = 40.00 cm d = 80.00 cm

13. If the pressure at the inlet of a pipe is 90 kg/cm2 and pressure drop over the pipe line is 10 kg/cm2, the efficiency of transmission, is

a. 66.6%

b. 77.7%

c. 55.5%

d. 88.8%.

14. The notch angle for maximum discharge over a triangular notch, is

a. 30°

b. 60°

c. 90°

d. 120°

15. Barometres are used to measure

a. difference in pressure at two points

b. atmospheric pressure

c. very low pressure

d. very high pressure.

16. The height of water level in a tank above the centre of a circular hole 2.5 cm in diameter is 50 m. The velocity of water flowing through the hole, is

a. 31.1 m/sec

b. 31.2 m/sec

c. 31.3 m/sec

d. 31.4 m/sec.

17. A suspended particle falls through a height H cm in water in t minutes. If the viscosity of water is η and specific gravity of the particle is G, the diameter of the particle is (where M is a constant factor)

a. 103M

b. 104M

c. 105M

d. 102M

18. The specific gravity of sands, is approximately

a. 2.0

b. 2.2

c. 2.4

d. 2.6

19. The fundamental equation of specific gravity (G), dry density (γd), unit weight of water (γω) and void ratio (e), is

a. e =

b. G =

c. γd =

d. γω =

20. A phreatic line is defined as the line within a dam section below which there are

a. positive hydrostatic pressure

b. negative hydrostatic pressure

c. negative equipotential lines

d. none of these.

21. The horizontal portion of a step in a stairs case, is known as

a. rise

b. flight

c. winder

d. tread.

22. If a rectangular prestressed beam of an effective span of 5 meters and carrying a total load 3840 kg/m, is designed by the load balancing method, the central dip of the parabolic tendon should be

a. 10 cm

b. 15 cm

c. 20 cm

d. 25 cm

23. A very comfortable type of stairs is

a. straight

b. dog legged

c. geometrical

d. open newel.

24. The shear reinforcement in R.C.C. is provided to resist

a. vertical shear

b. horizontal shear

c. diagonal compression

d. diagonal tension.

25. The concrete slump recommended for foundations, is

a. 30 to 125 mm

b. 50 to 100 mm

c. 75 to 125 mm

d. none of these.

26. The platform at the end of a series of steps, is known as

a. relief

b. rest

c. landing

d. stop.

27. Shrinkage in concrete can be reduced by using

a. less cement in the concrete

b. proper concrete mix

c. presaturated aggregates

d. all the above.

28. C.R.R.I. charts are used to obtain a relatioship between strength of concrete and

a. water cement ratio

b. workability

c. grading of aggregate

d. fineness modulus

29. Water is supplied from the source to the treatment plant under gravity in

a. Calcutta

b. Delhi

c. Madras

d. Bombay.

30. In a well planned city, the layout of distribution pipes generally adopted, is

a. grid-iron system

b. interlaced system

c. reticulation system

d. all the above.

31. The drop man holes are generally provided in sewers for

a. industrial areas

b. large town ships

c. hilly town ships

d. cities in plains.

32. The most efficient cross section of sewers in a separate sewerage system is

a. parabolic

b. circular

c. rectangular

d. new egged.

33. According to G.W. Pickles the effect of confining the flood water of a river between levee, is to increase

a. water surface elevation during floods

b. maximum discharge

c. velocity and scouring action

d. surface slope of streams above the leveed portion.

34. Pick up the correct statement from the following:

a. Static level of water in an open well indicates the position of the water table

b. Saturated pores of a soil in the capillary zone above water table have negative pressure

c. At water table, hydrostatic pressure is zero

d. All the above.

35. Solution of Laplacian equation in three dimensions  of water in a syphon, is done by

a. analytical method

b. Khosla's method

c. method of relaxation

d. Unwin's method.

36. Process of meandering is due to

a. sediment load of streams

b. discharge and hydraulic properties of streams

c. relative erodibility of the bed and banks

d. all the above.

37. Shoulders for high traffic volume roads, should

a. be stable throughout the year to be used by vehicles in the case of emergency

b. support and protect the ends of carriage ways

c. not allow entrance of water to sub-grade

d. all the above.

38. Traffic census is carried out for

a. road parking study

b. traffic volume study

c. origin and destination study

d. all the above.

39. Curves in the same direction separated by short tangents, are called

a. compound curves

b. transition curves

c. broken-back curves.

d. None of these.

40. Bottom-most component of a flexible pavement, is

a. subgrade

b. sub-base

c. base

d. base course.

41. Pick up the correct statement from the following:

a. The bent up bars at a support resist the negative bending moment

b. The bent up bars at a support resist the sharing force

c. The bending of bars near supports is generally at 45°

d. All the above.

42. The area is measured correct to the nearest

a. 0.01 sqm

b. 0.02 sqm

c. 0.03 sqm

d. 0.04 sqm

43. Pick up the correct statement from the following:

a. In a gully trap, a water seal of 6 to 7.5 cm is provided

b. The gully trap collects waste water from the kitchen, sink, wash basins, etc.

c. The gully trap disconnects the sullage drain from the main drainage system

d. The grating provided over gully traps is 23 cm square.

44. Size, capacity and materials need be specified for

a. Stop-cocks

b. Ferrules

c. Ball valves

d. All the above.

45. Frederick W. Taylor introduced a system of working known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

46. Pre-tender stage requires

a. selection of site

b. formalisation of alignment of work

c. formalisation of designs and preparation of estimate

d. all the above.

47. If t is the duration of an activity, t1 is the latest finish possible moment of its preceding activity and t2 is the earliest start possible moment, the independent float of the activity is

a. (t1 - t2) - t

b. t - (t1 - t2)

c. (t1 + t2) - t

d. t + (t1 - t2)

48. Final technical authority of a project lies with

a. Assistant Engineer

b. Executive Engineer

c. Superintending Engineer

d. Chief Engineer.

49. For night landing, the thresholds are lighted

a. green

b. red

c. white

d. yellow

50. The reduced levels of the ends A and B of a runway are 3025 m and 3035 m and that of its mid-point is 3015 m. The reduced level of the horizontal surface, is

a. 3060 m

b. 3075 m

c. 3015 m

d. none of these.