***Sub Engineer***

1. Pick up the correct statement from the following

a. the contour lines having the same elevation cannot unite and continue as one line

b. a contour can not end abruptly, but must ultimately close itself not necessarily within the limits of map.

c. the direction of steepest slope at a point on a contour is at right angles to the contour

d. all the above.

2. Straight, parallel and widely spaced contours represent

a. a steep surface

b. a flat surface

c. an inclined plane surface

d. curved surface.

3. Tilt of the staff in stadia tacheometry increases the intercept if it is

a. away from the telescope pointing down hill

b. towards the telescope pointing up-hill

c. away from the telescope pointing up-hill

d. none of these.

4. Under ordinary conditions, the precision of a theodolite traverse is affected by

a. systematic angular errors

b. accidental linear errors

c. systematic linear errors

d. accidental angular errors.

5. The standard size of masonry bricks, is

a. 19 cm x 9 cm x 9 cm

b. 20 cm x 10 cm x 10 cm

c. 21 cm x 11 cm x 11 cm

d. none of these.

6. The method of addition polymerization is used for obtaining:

a. polypropylene

b. polyvinylchloride

c. polystyrene

d. All of these.

7. Knots in timber are

a. defects caused by crushing fibres

b. splits radiating from the centre

c. speckled strains

d. signs of branches cut off.

8. The minimum compressive strength of Ilnd class bricks should be

a. 75 kg/cm2

b. 90 kg/cm2

c. 100 kg/cm2

d. 120 kg/cm2

9. In plastic analysis, the shape factor for a circular section, is

a. 1.5

b. 1.6

c. 1.7

d. 1.75

10. For calculating the allowable stress of long columns. The empirical formula , is known as

a. Straight line formula

b. Parabolic formula

c. Perry's formula

d. Rankine's formula.

11. The under mentioned type is simple strain

a. compressive strain

b. shear strain

c. volumetric strain

d. all the above.

12. As the elastic limit reaches, tensile strain

a. increases more rapidly

b. decreases more rapidly

c. increases in proportion to the stress

d. decreases in proportion to the stress.

13. In Newton's viscosity equation , the coefficient (μ) is known as coefficient of

a. absolute viscosity

b. dynamic viscosity

c. simple viscosity

d. all the above.

14. The flow in open channel is laminar if the Reynold number is

a. less than 500

b. more than 500

c. 1000

d. none of these.

15. The pressure rise due to water hammer depends upon

a. the length of pipe

b. time taken to close the valve

c. the elasticity of the pipe material

d. All of above.

16. A piezometer opening in pipes measures

a. velocity head

b. static pressure

c. total pressure

d. negative static pressure.

17. Pick up the in-correct statement from the following: The soils which contain montmorillonite minerals

a. shrink more when dry

b. possess high plasticity

c. possess high coefficient of internal coefficient

d. None of these.

18. The total weight of a pycnometer with water and oven dried soil 20 (g) is 1600 g. The pycnometer filled with water alone weighs 1500 g. The specific gravity of the soil, is

a. 1.0

b. 1.5

c. 2.0

d. 2.5

19. A structure is erected on an impervious clay whose thickness is 12 m. Drainage is possible both at upper and lower surfaces. Coefficient of consolidation is 0.015 cm2 per minute. For attaining 50% consolidation with a time factor 0.20, the number of days required

a. 3233

b. 3123

c. 33331

d. 3313

20. Over-consolidation of soils is caused due to

a. erosion of over burden

b. melting of ice sheets after glaciation

c. permanent rise of water table

d. all the above.

21. According to I.S. : 456, slabs which span in two directions with corners held down, are assumed to be divided in each direction into middle strips and edge strips such that the width of the middle strip, is

a. two-third of the width of the slab

b. three-fourth of the width of the slab

c. four-fifth of the width of the slab

d. three-fifth of the width of the slab.

22. The horizontal portion of a step in a stairs case, is known as

a. rise

b. flight

c. winder

d. tread.

23. The live load to be considered for an accessible roof, is

a. Nil

b. 75 kg/m3

c. 150 kg/m2

d. 200 kg/cm2

24. In a combined footing if shear stress exceeds 5 kg/cm2, the nominal stirrups provided are:

a. 8 legged

b. 10 legged

c. 12 legged

d. none of these.

25. A pointed arch which forms isosceles or equilateral triangle, is generally known as

a. three centred arch

b. two centred arch

c. Lancet arch

d. Bull's eye arch.

26. Pick up the commonly adopted geophysical method in civil engineering from the following :

a. electrical resistivity method

b. gravitational method

c. magnetic method

d. both (a) and (b) of the above.

27. Pick up the incorrect statement from the following:

a. Tricalcium silicate (C3S) generates more heat of hydration

b. Tricalcium silicate (C3S) develops early strength

c. Tricalcium silicate (C3S) has more resistance to sulphate attack

d. None of these.

28. Gypsum is added for

a. colour

b. strength

c. controlling setting time

d. none of these.

29. A circular gravity aqueduct is not generally preferred to because of

a. its maximum hydraulic mean depth

b. maximum area per unit of wetted perimeter

c. minimum cost of construction

d. its proper support on the ground.

30. The diameter of pipes in bath rooms and lavatories in domestic water supply, is

a. 6 mm

b. 12 mm

c. 18 mm

d. 24 mm.

31. If the side of a square sewer is 1000 mm, the diameter of a hydraulically equivalent circular section, is

a. 1045 mm

b. 1065 mm

c. 1075 mm

d. 1095 mm.

32. Drop manholes at the junctions of sewer lines, are provided if

a. invert level of a branch sewer is more than 60 cm that of the main sewer

b. sewer line runs along a main road

c. ordinary manhole cannot be built

d. two sewer lines intersect.

33. A hydraulic jump is generally formed when a stream moving with

a. a hyper-critical velocity meets a stream moving with a critical velocity

b. a hyper-critical velocity meets a stream moving with a hyper-critical velocity

c. A hyper-critical velocity meets a stream moving with a sub-critical velocity

d. a sub-critical velocity meets a stream moving with a hyper-critical velocity.

34. Garnett's diagrams are used for graphical solution of design equations of a canal by

a. Lacey's theory

b. Kennedy's theory

c. Gibb's theory

d. Lindlay theory.

35. If average particle size of the silt in millimetres is m, the Lacey's silt factor f is proportional to

a. m3

b. m1/2

c. m1/3

d. m2/3

36. For loss of head in a canal inverted syphon barrel, the factor in the Unwin formula  is a coefficient for loss of head due to

a. friction

b. exit

c. entry

d. gradient

37. When a number of hair pin bends are introduced, a minimum intervening distance in between is kept

a. 20 m

b. 40 m

c. 60 m

d. 100 m

38. The camber on pavements, is provided by

a. parabolic method

b. straight line and parabolic at crown

c. elliptical method

d. circular method.

39. Deviation of the alignment of a trace cut may be permitted in areas involving

a. land slides

b. sand dunes

c. dens

d. none of these.

40. In cement concrete pavements, tensile stress is due to :

a. bending or deflection under wheel loads

b. difference in temperature of the top and bottom of pavement

c. contraction of slab during falling temperature

d. all the above.

41. The measurement is made for stone work in square metre in case of

a. Columns, lintels, copings

b. Building work

c. Dressed stones in Chajjas

d. (a) and (d) of the above.

42. The cross-sections for a highway is taken at

a. 30 metres apart

b. intermediate points having abrupt change in gradient

c. the starting end points of the curves

d. All the above.

43. The concrete work for the following part of the building of specified thickness is measured in square metres

a. Floors

b. D.P.C.

c. Wall panels

d. All the above.

44. If the formation level of a highway has a uniform gradient for a particular length, and the ground is also having a longitudinal slope, the earthwork may be calculated by

a. Mid-section formula

b. Trapezoidal formula

c. Prismoidal formula

d. All the above.

45. Frequency distribution curves

a. having a single lump, are called uninodal curves

b. if symmetrical, are called normal curves

c. if not symmetrical, are called skew curves

d. all the above.

46. Pick up the correct statement from the following:

a. The duration of time by which an activity can be delayed without affecting the succeeding activity, is called free float

b. The float which affects neither the processor nor the successor activities, is called independent float

c. The difference between total float and free float, is called interfering float

d. All the above.

47. Pick up the correct statement from the following:

a. If the float is positive and the activity is delayed by a period equal to its total float, the completion of project is not delayed

b. If the float of an activity is negative, delay in its performance is bound to delay the completion of project

c. If the float of an activity is zero, the activity is critical and any delay in its performance will delay the whole project

d. All the above.

48. A Milestone chart

a. shows the interdependencies of various jobs

b. depicts the delay of jobs, if any

c. points outgoing ahead of schedule of jobs, if any

d. none of these.

49. The fuse large includes

a. passengers chamber

b. pilot's cabin

c. tail of aircraft

d. all the above.

50. For the proposed runway stated in if the aerodrome reference temperature is 17°.2, the net designed length of the runway is

a. 2716 m

b. 2816 m

c. 2916 m

d. 3016 m