***Sub Engineer***

1. Pick up the correct statement from the following :

a. The directions of plumb lines suspended at different points in a survey are not strictly parallel

b. In surveys of small extent, the effect of curvature may be ignored and the level surface of the earth is assumed as horizontal

c. In surveys of large extent, the effect of curvature of the earth must be considered

d. All the above.

2. The desired sensitivity of a bubble tube with 2 mm divisions is 30". The radius of the bubble tube should be

a. 13.75 m

b. 3.44 m

c. 1375 m

d. none of these.

3. In chain surveying, perpendiculars to the chain line, are set out by

a. a theodolite

b. a prismatic compass

c. a level

d. an optical square

4. The slope correction may be ignored if

a. the slope of the ground is less than 3°

b. to slope of the ground is say 1 in 19

c. both (a) and (b)

d. neither (a) nor (b)

5. For a 50 kg cement bag water required, is

a. 18.5 litres

b. 20.5 litres

c. 22.5 litres

d. 25 litres.

6. Seasoning is

a. a process of removing sap

b. creosoting

c. painting with sodium silicate

d. coating with tar.

7. For filling cracks in masonry structures, the type of bitumen used, is

a. cut-back bitumen

b. bitumen-emulsion

c. blown bitumen

d. plastic bitumen.

8. According to ISI, bitumen is classified into

a. 4 grades

b. 6 grades

c. 8 grades.

d. 10 grades.

9. A three hinged arch is generally hinged at its supports and

a. at one quarter span

b. at the crown

c. any where in the rib

d. none of these.

10. The ratio of moments of inertia of a triangular section about its base and about a centroidal axis parallel to its base, is

a. 1.5

b. 2.0

c. 2.5

d. 3.0

11. A reinforced concrete beam is assumed to be made of

a. homogeneous material

b. heterogeneous material

c. isotropic material

d. none of these.

12. In a loaded beam, the point of con-traflexture occurs at a section where

a. bending moment is zero or changes sign

b. bending moment is maximum

c. shearing force is maximum

d. shearing force is minimum.

13. The ratio of inertia force of a flowing fluid and the viscous force of the liquid is called :

a. Renold's number

b. Froude's number

c. Euler's number

d. Weber's number.

14. While applying the Bernoulli's equation  = total head, the work any section done on the flow system, if any

a. is added on the right side of the equation

b. is added on the left side of the equation

c. is ignored

d. none of these.

15. The ratio of maximum velocity to average velocity of viscous fluid through a circular pipe is

a. 0.5

b. 0.75

c. 1.25

d. 2.00

16. Weber number is the ratio of inertia force to

a. surface tension

b. gravitational force

c. elasticity

d. viscosity.

17. The active earth pressure of a soil is proportional to (where φ is the angle of friction of the soil)

a. tan (45° - φ)

b. tan2 (45° + φ/2)

c. tan2 (45° - φ/2)

d. tan (45° + φ)

18. Failure of the stability of slopes, generally occurs along

a. slip plane

b. a horizontal surface

c. a curved surface

d. all the surfaces.

19. The consolidation time for soils

a. decreases with increasing permeability

b. increases rapidly with increasing size of soil mass

c. is independent of the magnitude of the stress change.

d. All the above.

20. If the cohesive force, (c), is 1.5 t/m2, the density (γ) of the soil is 2.0 t/m3, factor of safety (F) is 1.5 and stability factor (Sn) is 0.05, the safe height of the §lope, is

a. 5 metres

b. 8 metres

c. 10 metres

d. 12 metres

21. The angle of internal friction of soil mass is the angle whose

a. tangent is equal to the rate of the maximum resistance to sliding on any internal inclined plane to the normal pressure acting on the plane

b. sine is equal to the ratio of the maximum resistance to sliding on any internal inclined plane to the normal pressure acting on the plane

c. cosine is equal to the ratio of the maximum resistance sliding on any internal inclined plane to the normal pressure acting on the plane

d. none of these.

22. The maximum ratio of span to depth of a slab simply supported and spanning in two directions, is

a. 30

b. 35

c. 40

d. 15

23. An R.C.C beam of 25 cm width has a clear span of 5 metres and carries a U.D.L. of 2000 kg/m inclusive of its self weight. If the lever arm of the section is 45 cm., the beam is

a. safe in shear

b. is safe with stirrups

c. is safe with stirrups and inclined members

d. needs revision of the section.

24. According to I.S. : 456, slabs which span in two directions with corners held down, are assumed to be divided in each direction into middle strips and edge strips such that the width of the middle strip, is

a. two-third of the width of the slab

b. three-fourth of the width of the slab

c. four-fifth of the width of the slab

d. three-fifth of the width of the slab.

25. Pick up the correct statements from the following :

a. cracks appear on the plastered surface in the form of hair cracks

b. in brick work, the efflorescence is removed by applying a solution of zinc sulphate and water

c. excessive thermal variations in the backing or plaster causes the plaster to fall

d. all the above.

26. Negative skin friction

a. is a downward drag acting on a pile due to downward movement of the surrounding compressible soil relative to the pile

b. develops due to lowering of ground water

c. both (a) and (b)

d. neither (a) not (b).

27. Too wet concrete may cause

a. excessive laitance

b. segregation

c. lower density

d. all the above.

28. If aggregates completely pass through a sieve of size 75 mm and are retained on a sieve of size 60 mm, the particular aggregate will be flaky if its minimum dimension is less than

a. 30.5 mm

b. 40.5 mm

c. 50.5 mm

d. none of these.

29. Pick up the correct statement from the following :

a. Lead and barium salts have toxic effect

b. Arsenic and selenium are poisonous to human health

c. Higher copper content affects the lungs

d. All the above.

30. The type of pipe commonly used in water supply distribution schemes, is

a. R.C.C. pipes

b. Hume pipes

c. Cast iron pipes

d. G.I. pipes.

31. Pick up the correct statement from the following :

a. In acidic sewage, the pH value is less than 7

b. In alkaline sewage, the pH value is more than 7

c. Fresh sewage is generally alkaline

d. All the above.

32. For a continuous flow type of sedimentation tanks

a. length of the tank is normally kept 4 to 5 times the width

b. maximum horizontal flow velocity is limited to 0.3 m/minute

c. capacity of the tank for 24 hours, is kept equal to maximum daily flow of sewage

d. all the above.

33. The saturation line is the line up to which banks get saturated after the canal runs for some time. The saturation gradient in ordinary loam soil, is generally

a. 3 : 1

b. 4 : 1

c. 5 : 1

d. 6 : 1

34. For a unique design of a channel by Kennedy's theory

a. its breadth must only be known

b. its depth must only be known

c. its breadth and depth ratio must only be known

d. all the above.

35. According to Lacey, in regime conditions

a. silt is kept in suspension by vertical components of eddies

b. entire cross-section of the channel is generated at all points by the forces normal to the wetted perimeter

c. both (a) and (b)

d. neither (a) nor (b).

36. In case of a trapezoidal notch fall

a. top length of the piers should not be less than their thickness

b. splay up stream from the notch section is 45°

c. splay down stream from the notch section is

d. all the above.

37. If the radius of a main curve is 300 m and length of the transition curve is 100 m, the angle with tangent to locate the junction point, is

a. 1° 11'

b. 2° 11'

c. 3° 11'

d. 4° 11'

38. Road width 8.8 m of two lane National highways or State highways in mountainous terrain

a. excludes the width of parapet (0.6 m)

b. excludes the width of side drain (0.6 m)

c. excludes the width of parapet and side drain

d. includes the width of parapet and side drain

39. According to the recommendations of Nagpur Conference, the width formation of an ideal National Highway in hard rock cutting, is

a. 7.9 m

b. 6.9 m

c. 6.5 m

d. 7.5 m

40. The desirable camber for straight roads with thin bituminous surfacing, is

a. 1 in 40 to 1 in 33

b. 1 in 150 to 1 in 140

c. 1 in 160 to 1 in 140

d. none of these.

41. The ground surface slopes 1 in 50 along a proposed railway embankment 150 m in length. The height of the embankment at zero chainage is 0.5 m, the width is 11 m and side slopes 2:1. If the falling gradient of the embankment is 1 in 150, the quantity of the earthwork calculated by prismoidal formula, is

a. 3250 m3

b. 3225 m3

c. 3275 m3

d. 3300 m3

42. The concrete work for the following part of the building of specified thickness is measured in square metres

a. Floors

b. D.P.C.

c. Wall panels

d. All the above.

43. Pick up the incorrect statement from the following:

a. Dimensions are measured to the nearest 0.01 m

b. Areas are measured to the nearest 0.01 sq.m

c. Cubic contents are measured to the nearest 0.1 cm m

d. Weights are measured to the nearest 0.001 tonnes

44. The diameter of a domestic sewer pipe laid at gradient 1 in 100 is recommended

a. 100 mm

b. 150 mm

c. 200 mm

d. 175 mm

45. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

46. Mile Stone charts were invented in the year of

a. 1920

b. 1930

c. 1940

d. 1950

47. In CPM analysis,

a. uncertainties are not allowed

b. activities are represented by arrows

c. beginning and end of an activity, are denoted by nodes

d. all the above.

48. The difference between the time avail-to do a job and the time required to do the job, is known as

a. event

b. float

c. duration

d. constraint.

49. For the taxiways, the following statement is true

a. The maximum longitudinal grade is 3%

b. The permissible rate of change of grade is 1%

c. The permissible transverse grade is 1.5%

d. All the above.

50. The meterological condition which influences the size and location of an air port is

a. air density

b. reduced level

c. wind direction

d. all the above.