***Sub Engineer***

1. Surveys which are carried out to provide a national grid of control for preparation of accurate maps of large areas, are known

a. plane surveys

b. geodetic surveys

c. geographical surveys

d. topographical surveys.

2. A traverse deflection angle is

a. less than 90°

b. more than 90° but less than 180°

c. the difference between the included angle and 180°

d. the difference between 360° and the included angle.

3. If the chain line which runs along N-S direction is horizontal and the ground in E-W direction is sloping

a. it is possible to set offsets correctly on east side

b. it is possible to set offsets correctly on east side

c. it is not possible to set offsets correctly on west side

d. it is possible to set offsets correctly on both sides.

4. In quadrantal bearing system, back bearing of a line may be obtained from its forward bearing, by

a. adding 180°, if the given bearing is less than 180°

b. subtracting 180°, if the given bearing, is more than 180°

c. changing the cardinal points, i.e. substituting N for S and E for W and vice-versa

d. none of these.

5. The base material for distemper, is

a. chalk

b. lime

c. lime putty

d. cement wash.

6. Slump test for concrete is carried out, to determine

a. strength

b. durability

c. workability

d. water content.

7. The term frog means

a. an apparatus to lift the stone

b. a depression on a face of brick

c. vertical joint in a brick work

d. soaking brick in water.

8. Cast iron contains carbon approximately

a. 1.5% to 5.5%

b. 0.05% to 1.75%

c. 0.250%

d. none to these.

9. In plastic analysis, the shape factor for rectangular section, is

a. 1.4

b. 1.5

c. 1.6

d. 1.7.

10. Pick up the correct statement from the following:

a. The structural member subjected to compression and whose dimensions are small as compared to its length, is called a stmt

b. The vertical compression members, are generally known as columns or stanchions

c. Deflection in lateral direction of a long column, is generally known as buckling

d. All the above.

11. Stress in members of statically determinate simple frames, can be determined by

a. method of joints

b. method of sections

c. graphical solution

d. all the above.

12. Pick up the correct statement from the following :

a. In visco-elastic material, stress-strain relation is dependent on time

b. An instropic material has different properties is different directions

c. An orthotropic material has different properties in three mutually perpendicular directions

d. All the above.

13. The flow in open channel is laminar if the Reynold number is

a. less than 500

b. more than 500

c. 1000

d. none of these.

14. Kinematic viscosity of liquids by equation  is determined by :

a. Redwood viscometer

b. Engler viscometer

c. Saybolt universal viscometer

d. None of these.

15. A fluid particle may possess the displacement of

a. translation

b. rotation

c. distortion

d. all the above.

16. The centre of pressure of a vertical plane immersed in a liquid is at

a. centre of higher edge

b. centre of lower edge

c. centroid of the area

d. none of these.

17. Pick up the correct statement from the following:

a. The line which shows moisture content dry density relation for soil containing a constant percentage of air voids, is known as air void line

b. The weight of hammer used for compaction test is 25 kg

c. The free fall of hammer for compaction is 30.5 cm

d. All the above.

18. The soil moisture driven off by heat, is called

a. free water

b. hydroscopic water

c. gravity water

d. none of these.

19. A saturated soil sample has water content of 40% and specific gravity of soil particles 2.7. The void ratio of the soil, is

a. 0.4

b. 0.52

c. 1.08

d. none of these.

20. In a liquid limit test, the moisture content at 10 blows was 70% and that at 100 blows was 20%. The liquid limit of the soil, is

a. 35%

b. 50%

c. 65%

d. none of these

21. According to the steel beam theory of doubly reinforced beams

a. compression is resisted by compression steel

b. stress in tension steel equals the stress in compression steel

c. no stress is developed in compression concrete as well as in tension concrete

d. all the above.

22. Minimum spacing between horizontal parallel reinforcement of the same size should not be less than

a. one diameter

b. 2.5 diameters

c. 3 diameters

d. 3.5 diameters

23. A simply supported beam , 6 m long and of effective depth 50 cm, carries a uniformly distributed load 2400 kg/m including its self weight. If the lever arm factor is 0.85 and permissible tensile stress of steel is 1400 kg/cm2, the area of steel required, is

a. 15 cm2

b. 16 cm2

c. 17 cm2

d. 18 cm2

24. For a circular slab carrying a uniformly distributed load, the ratio of the maximum negative to maximum positive radial moment, is

a. 2

b. 3

c. 4

d. 5

25. The raft slab is projected beyond the outer walls of the structure by

a. 15 to 20 cm

b. 25 to 30 cm

c. 30 to 45 cm

d. 60 cm.

26. Vertical construction joints are provided where the shearing forces are minimum in the case of

a. slabs

b. beams

c. girders

d. all of these.

27. The percentage of the aggregate of F.M. 2.6 to be combined with coarse aggregate of F.M. 6.8 for obtaining the aggregates of F.M. 5.4, is

a. 30%

b. 40%

c. 50%

d. 60%.

28. 'Ware house pack' of cement means

a. pressure exertion of the bags of upper layers

b. pressure compaction of the bags on lower layers

c. packing the ware house

d. none of these.

29. The period of cleaning of a slow sand filter, is usually

a. two weeks to three weeks

b. one month to three months

c. three months to six months

d. one year.

30. Flow through period, in sedimentation tanks, is

a. more than detention period

b. less than detention period

c. detention period divided by displacement efficiency

d. none of these.

31. The value of Chezy's constant is used in

a. Chezy's formula

b. Bazin's formula

c. Kutter's

d. Manning's formula.

32. The pH value of fresh sewage is usually

a. equal to 7

b. more than 7

c. less than 7

d. equal to zero.

33. Process of meandering is due to

a. sediment load of streams

b. discharge and hydraulic properties of streams

c. relative erodibility of the bed and banks

d. all the above.

34. Lane's weighted creep theory assumes

a. double weightage to horizontal creep and one weightage to vertical creep

b. triple weightage to horizontal creep and one weightage to vertical creep

c. triple weightage to vertical creep and one weightage to horizontal creep

d. double weightage to vertical creep and one weightage to horizontal creep.

35. Irrigation canals are generally aligned along

a. ridge line

b. contour line

c. valley line

d. straight line.

36. The saturation line is the line up to which banks get saturated after the canal runs for some time. The saturation gradient in ordinary loam soil, is generally

a. 3 : 1

b. 4 : 1

c. 5 : 1

d. 6 : 1

37. Road makers along roads from the edge of a kerb should not be less than

a. 45 cm

b. 50 cm

c. 55 cm

d. 60 cm

38. Longitudinal pavement lines marked broken in white paint

a. are for the guidance of the drivers

b. are not to be crossed over

c. may be crossed over at the discretion of the driver

d. all the above.

39. The maximum radial distance of a Lemniscate curve, having maximum polar angle a, is

a. 3 R sin2α

b. 3 R sin 2α

c. 3 R sin α/2

d. none of these.

40. The minimum superelevation in rolling terrain in plains, is limited to

a. 5%

b. 6%

c. 7%

d. 10%

41. Pick up the correct statement from the following:

a. Additional supervising staff engaged at work site, is called work charged establishment

b. Detailed specifications specify qualities, quantities and the proportions of materials to be used for a particular item

c. The cost per unit at which the article can be procured, from the open market at a given time, is called 'market rate'

d. All the above.

42. In long and short wall method of estimation, the length of long wall is the centre to centre distance between the walls and

a. breadth of the wall

b. half breadth of wall on each side

c. one fourth breadth of wall on each side

d. None of these.

43. Pick up the correct statement from the following:

a. In order to check up the average depth of excavation, 'Dead mans' are left at the mid-widths of borrow pits

b. The earthwork calculation in excavation is made from the difference in levels obtained with a level

c. The earth work in excavation to form the road embankment includes the formation of correct profile and depositing the soil in layers

d. All the above.

44. The floor area includes the area of the balcony up to

a. 100%

b. 75%

c. 50%

d. 25%

45. Pick up the correct statement from the following:

a. Backward pass is used for calculating the latest occurence time

b. Maximum value of earliest expected time is used if there are more than one value of any event

c. Minimum value of latest occurrence time is used if there are more than one value of any event

d. All the above.

46. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

47. The most popular type of organisation used for Civil Engineering Constructions, is

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

48. Pick up the correct statement from the following:

a. If the float is positive and the activity is delayed by a period equal to its total float, the completion of project is not delayed

b. If the float of an activity is negative, delay in its performance is bound to delay the completion of project

c. If the float of an activity is zero, the activity is critical and any delay in its performance will delay the whole project

d. All the above.

49. Pick up the correct statement from the following:

a. F.I.C. stands for flight information centre

b. Radius of control area is 160 km

c. Radius of control zone is 40 km

d. All the above.

50. Pick up the correct statement from the following:

a. Approach areas are measured in horizontal surfaces

b. The imaginary inclined plane which is directly above the approach area is called approach surface

c. Obstruction clearance surface and approach surface are same

d. All the above.