***Sub Engineer***

1. In levelling operation,

a. The first sight on any change point is a back sight

b. The second sight on any change point is a fore sight

c. The line commences with a fore sight and closes with a back sight

d. The line commences with a back sight and closes with a foresight.

2. The zero of the graduated circle of a prismatic compass is located at

a. north end

b. east end

c. south end

d. west end.

3. Pick up the correct statement from the following :

a. the system of a series of lines which forms a circuit which ends at the starting point, is called a closed traverse

b. the traverse that starts from a point already fixed in some survey system and ends on another such point, is called a controlled traverse

c. the traverse that is not controlled is called a fly traverse

d. all the above.

4. In geodetic surveys higher accuracy is achieved, if

a. curvature of the earth surface is ignored

b. curvature of the earth surface is taken into account

c. angles  between the curved lines are treated as plane angles

d. none of these.

5. The rocks which are formed due to cooling of magma at a considerable depth from earth's surface are called

a. Plutonic rocks

b. Hypabyssal rocks

c. Volcanic rocks

d. Igneous rocks.

6. The rocks having alumina or clay as their major constituents, are known as

a. argillaceous rocks

b. calcareous rocks

c. sedimentary rocks

d. igneous rocks.

7. Bitumen felt

a. is used as water proofing material

b. is used as damp proofing material

c. is made from bitumen and hessian fibres

d. all the above.

8. The lime which contains high percentage of calcium oxide, is generally called

a. fat lime

b. rich lime

c. white lime

d. All of these.

9. If a three hinged parabolic arch, (span l, rise h) is carrying a uniformly distributed load w/unit length over the entire span,

a. horizontal thrust is wl2/8h

b. S.F. will be zero throughout

c. B.M. will be zero throughout

d. all the above.

10. Maximum shear stress theory for the failure of a material at the elastic limit, is known

a. Guest's or Trecas' theory

b. St. Venant's theory

c. Rankine's theory

d. Haig's theory

11. A cast iron T section beam is subjected to pure bending. For maximum compressive stress to be three times the maximum tensile stress, centre of gravity of the section from flange side is

a. h/4

b. h/3

c. h/2

d. 2/3 h.

12. Influence lines are drawn for structures

a. of any type

b. statically determinate

c. pin-jointed truss

d. none of these.

13. For the two dimensional flow, the stream function is given by ψ = 2xy. The velocity at a point (3, 4) is

a. 8 m/sec

b. 10 m/sec

c. 12 m/sec

d. 15 m/sec.

14. The line joining the points to which the liquid rises in vertical piezometer tubes fitted at different cross-sections of a conduit, is known as

a. piezometric line

b. pressure grade line

c. hydraulic grade line

d. all the above.

15. The instrument used for measuring the velocity of flow, is known as

a. venturimeter

b. orifice meter

c. pitot tube

d. none of these.

16. If the Mach number for a fluid flow is less than 1, the flow is

a. sonic

b. supersonic

c. sub-sonic

d. none of these.

17. When the seepage pressure becomes equal to the pressure due to submerged weight of a soil, the effective pressure is reduced to zero and the soil particles have a tendency to move up in the direction of flow. This phenomenon is generally known

a. quick condition

b. boiling condition

c. quick sand

d. all the above.

18. Pick up the incorrect statement from the following:

a. Below the water table, the pore water may be static.

b. The hydrostatic pressure depends on the depth below the water level.

c. The attractive forces between the particles, caused due to negative pressure of water held above the water table is called soil suction.

d. None of these.

19. Flow net is used for the determination of

a. hydrostatic pressure

b. seepage pressure

c. exit gradient

d. all the above.

20. If the failure of a finite slope occurs through the toe, it is known as

a. slope failure

b. face failure

c. base failure

d. toe failure.

21. If permissible working stresses in steel and concrete are respectively 1400 kg/cm2 and 80 kg/cm2 and modular ratio is 18, in a beam reinforced in tension side and of width 30 cm and having effective depth 46 cm, the lever arms of the section, is

a. 37 cm

b. 38 cm

c. 39 cm

d. 40 cm

22. An R.C.C beam of 25 cm width has a clear span of 5 metres and carries a U.D.L. of 2000 kg/m inclusive of its self weight. If the lever arm of the section is 45 cm., the beam is

a. safe in shear

b. is safe with stirrups

c. is safe with stirrups and inclined members

d. needs revision of the section.

23. [A + (m - 1)ASC] known as equivalent concrete area of R.C.C. is given by

a. modular ratio method

b. load factor method

c. ultimate load method

d. none of these.

24. On piles, the drop must be at least

a. 100 cm

b. 120 cm

c. 140 cm

d. 150 cm

25. The process of keeping concrete moist for a certain period after its finishing, is known as

a. curing of concrete

b. placing of concrete

c. compaction of concrete

d. none of these.

26. The steel pile which is generally sunk in soft clay or loose sand of low bearing capacity, is

a. pipe pile

b. screw pile

c. disc pile

d. none of these.

27. The type of aggregates of same nominal size, which contain less voids when compacted, are

a. rounded spherical

b. irregular

c. flaky

d. none of these.

28. A construction joint is provided where

a. bending moment is small

b. shear force is small

c. the member is supported by other member

d. all the above.

29. In distribution pipes, air valves are provided at

a. lower points

b. junction points

c. higher points

d. any where.

30. Acidity in water is caused due to

a. Free CO2

b. Iron sulphate

c. Aluminium sulphate

d. All the above.

31. The spacing of bars for perforations in coarse screens used for the treatment of sewage, is

a. 30 mm

b. 40 mm

c. 50 mm

d. 100 mm.

32. The maximum depth of sedimentation tanks, is kept

a. 3 m

b. 3.5 m

c. 4 m

d. 4.5 m.

33. The ratio of the rate of change of discharge of an outlet and parent channel, is known as

a. efficiency

b. sensitivity

c. flexibility

d. modular limit.

34. The top soil of a water logged field becomes more alkaline and infertile if its pH value is

a. 8

b. 9

c. 10

d. 11

35. An outlet is said to be proportional if its flexibility, is

a. zero

b. less than one

c. more than one

d. one

36. Meandering of a river generally occurs, in

a. rocky stage

b. delta stage

c. boulder stage

d. trough stage.

37. The absolute minimum radius of horizontal curve for a design speed 60 km ph is

a. 131 m

b. 210 m

c. 360 m

d. none of these.

38. Driving vehicles on wet surfaced roads, is dangerous because it may

a. skid

b. slip

c. overturn

d. all the above.

39. Road makers along roads from the edge of a kerb should not be less than

a. 45 cm

b. 50 cm

c. 55 cm

d. 60 cm

40. Longitudinal pavement lines marked broken in white paint

a. are for the guidance of the drivers

b. are not to be crossed over

c. may be crossed over at the discretion of the driver

d. all the above.

41. The weight of an item is measured correct to nearest

a. 0.50 kg

b. 0.75 kg

c. 1.00 kg

d. 5 kg

42. The plinth area of a building not includes

a. area of stair cover

b. internal shaft for sanitary installations up to 2 sq m. in area

c. lift and wall including landing

d. area of cantilevered porch.

43. Brick walls are measured in sq. m if the thickness of the wall is

a. 10 cm

b. 15 cm

c. 20 cm

d. None of these.

44. Cost of fittings and their fixing is specified for the following sanitary fittings

a. Flushing pipes

b. Lavatary basins

c. Sinks

d. All the above.

45. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

46. Pick up the incorrect statement from the following:

a. The tail of the arrow indicates the start of the activity

b. The head of the arrow indicates the end of the activity

c. The arrows are drawn to scale from left to right

d. Each activity consumes a given time.

47. Pick up the correct statement from the following:

a. The difference of latest occurence time and earliest expected time, is called slack

b. The activities connecting the events having zero slack, lie on the critical path

c. The critical path consumes the maximum time

d. All the above.

48. If t is the duration of an activity, t1 is the latest finish possible moment of its preceding activity and t2 is the earliest start possible moment, the independent float of the activity is

a. (t1 - t2) - t

b. t - (t1 - t2)

c. (t1 + t2) - t

d. t + (t1 - t2)

49. The best direction of a runway is along the direction of

a. longest line on wind rose diagram

b. shortest line on the wind rose diagram

c. line clear of wind rose diagram

d. none of these.

50. The height of the pilot's eye above the runway surface is assumed

a. 1 m

b. 3 m

c. 4 m

d. 5 m