***Sub Engineer***

1. For indirect ranging, number of ranging rods required, is

a. 2

b. 3

c. 4

d. 5

2. The method generally preferred to for contouring an undulating area, is

a. chain surveying

b. plane table surveying

c. tacheometrical surveying

d. compass surveying.

3. ABCD is a regular parallelogram plot of land whose angle BAD is 60°. If the bearing of the line AB is 30°, the bearing of CD, is

a. 90°

b. 120°

c. 210°

d. 270°

4. Pick up the correct statement from the following :

a. if the image of the object does not fall on the plane of the cross-lines, parallax exists

b. parallax has nothing to do with the eyepiece

c. the eyepiece is adjusted for clear vision of the cross hairs

d. all the above.

5. Duco paints are

a. cellulose paints

b. emulsion paints

c. bituminous paints

d. oil paints.

6. In order of increasing percentage of silica, the correct sequence is

a. sandy clay, calcareous clay, pure clay

b. calcareous clay, pure clay, sandy clay

c. pure clay, sandy clay, calcareous clay

d. None of these.

7. Mastic asphalt is

a. water proof

b. fire proof

c. elastic

d. all the above.

8. Acrylic sheets

a. are generally unaffected by most household detergents

b. possess the light transmission rate of 93%

c. are available in various shapes

d. all the above.

9. A spring of mean radius 40 mm contains 8 action coils of steel (N = 80000 N/mm2), 4 mm in diameter. The clearance between the coils being 1 mm when unloaded, the minimum compressive load to remove the clearance, is

a. 25 N

b. 30 N

c. 35 N

d. 40 N

10. The locus of the end point of the resultant of the normal and tangential components of the stress on an inclined plane, is

a. circle

b. parabola

c. ellipse

d. straight line.

11. The radius of gyration of a rectangular section is not proportional to

a. square root of the moment of inertia

b. square root of the inverse of the area

c. square root of the moment of inertia divided by area of the section

d. none of these.

12. For a simply supported beam of length L, the bending moment M is described as M = a (x - x3/L2), 0 ≤ x < L; where a is a constant. The shear force will be zero at

a. the supports

b. x = L/2

c. x = L/3

d. x = L/3

13. Weber number is the ratio of inertia force to

a. surface tension

b. gravitational force

c. elasticity

d. viscosity.

14. If the Mach number for a fluid flow is less than 1, the flow is

a. sonic

b. supersonic

c. sub-sonic

d. none of these.

15. For a most economical rectangular channel, the width of the channel must be

a. equal to depth of flow

b. twice the depth of flow

c. half the depth of flow

d. None of these.

16. If velocities of fluid particles vary from point to point in magnitude and direction, as well as from instant to instant, the flow is said to be

a. laminar

b. turbulent flow

c. uniform flow

d. non-uniform flow.

17. Negative skin friction on piles

a. is caused due to relative settlement of the soil

b. is caused in soft clays

c. decreases the pile capacity

d. all of the above.

18. Pick up the correct statement from the following:

a. When stress decreases, void, ratio decreases

b. When stress decreases, coefficient of permeability decreases

c. When stress decreases, coefficient of volume change decreases

d. When stress decreases void ratio, co-effi-cients of per-meability and volume change decrease.

19. A soil mass coated with a thin layer of paraffin weighs 460 g. When immersed, it displaces 299 cc of water. The weight of paraffin is 10 g. If specific gravity of solids is 2.5 and that of paraffin 0.9, the void ratio of soil, is

a. 0.55

b. 0.60

c. 0.65

d. 0.70

20. The specific gravity of Calcite is

a. 2.65

b. 2.72

c. 2.85

d. 2.90

21. In a singly reinforced beam, the effective depth is measured from its compression edge to

a. tensile edge

b. tensile reinforcement

c. neutral axis of the beam

d. longitudinal central axis.

22. If the shear stress in a R.C.C. beam is

a. equal or less than 5 kg/cm2, no shear reinforcement is provided

b. greater than 4 kg/cm2, but less than 20 kg/cm2, shear reinforcement is provided

c. greater than 20 kg/cm2, the size of the section is changed

d. all the above.

23. According to the steel beam theory of doubly reinforced beams

a. compression is resisted by compression steel

b. stress in tension steel equals the stress in compression steel

c. no stress is developed in compression concrete as well as in tension concrete

d. all the above.

24. The thickness of the topping of a ribbed slab, varies between

a. 5 cm to 8 cm

b. 8 cm to 10 cm

c. 12 cm to 15 cm

d. 12 cm to 18 cm

25. To stagger vertical joints in successive courses of a wall, a piece of brick is generally used at the end of the course, which is known as

a. bat

b. header

c. stretcher

d. closer.

26. Cast iron piles

a. are suitable for works under sea water

b. resist shocks or vibrations

c. are suitable for use as batter piles

d. are useful for heavy vertical loads.

27. Expansion joints are provided if the length of concrete structures exceeds

a. 15 m

b. 15 m

c. 35 m

d. 45 m

28. Hardening of cement occurs at

a. rapid rate during the first few days and afterwards it continues to increase at a decreased rate

b. slow rate during the first few days and afterwards it continues to increase at a rapid rate

c. uniform rate throughout its age

d. none of these.

29. Most commonly used pump for lifting water in water supply mains, is

a. axialflow pump

b. reciprocating pump

c. rotary type pump

d. centrifugal pumps

30. Pick up the incorrect statement from the following :

a. Porosity of pure sand is 35%

b. Porosity of sand stone is up to 15%

c. Least porosity is that of granite and quartz

d. None of these.

31. Faculative bacterias survive in

a. the presence of oxygen

b. the absence of oxygen

c. both cases (a) and (b)

d. neither (a) nor (b).

32. The formula  used for determining flow velocities in sewers, is known as

a. Chezy's formula

b. Bazin's formula

c. Kutter's formula

d. Manning's formula.

33. The most suitable location of a canal head work, is

a. boulders stage of the river

b. delta stage of the river

c. rock stage of the river

d. trough stage of the river.

34. If d1 is the depth of cutting, d2 is the height of the bank from bed level r2 : 1 and r1 : 1 are the slopes in filling and cutting respectively, the horizontal distance n between the bed and bank, is

a. x = r1 d1

b. x = r2 d2

c. x = d1 / r1

d. x = d2 / r2

35. A river training work is generally required when the river is

a. meandering

b. aggrading

c. degrading

d. all the above.

36. According to Lacey, depth of scour in a river depends upon the straightness of the reach. If D is the depth of scour in regime flow in a right angled bend, it is

a. 1.25 D

b. 1.50 D

c. 1.75 D

d. 2.00 D

37. The wall constructed for the stability of a back filling portion of a road on the down hill side, is known as

a. retaining wall

b. breast wall

c. parapet wall

d. all the above.

38. The efficiency of the brakes of a vehicle depends upon

a. condition of road surface

b. condition of the tyres

c. presence of the show moisture

d. all the above.

39. In case of a multi-lane road, overtaking is generally permitted

a. from right

b. from left

c. from both sides right and left

d. not at all.

40. The maximum radial distance of a Lemniscate curve, having maximum polar angle a, is

a. 3 R sin2α

b. 3 R sin 2α

c. 3 R sin α/2

d. none of these.

41. The following item of earth work is not measured separately.

a. Site clearence

b. dead men

c. Steps in deep exavaction

d. All the above.

42. The unit of measurement is per quintal for the following:

a. Collapsible gates with rails

b. Rolling shutters

c. Expanded metal wire netting

d. M.S. reinforcement of R.C.C. works.

43. The plinth area of a building not includes

a. area of stair cover

b. internal shaft for sanitary installations up to 2 sq m. in area

c. lift and wall including landing

d. area of cantilevered porch.

44. Anti-siphonage pipe is connected to

a. Main soil pipe

b. Bottom of P trap W.C.

c. Top of P trap W.C.

d. Side of water closet.

45. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

46. Modular co-ordination of construction means proper

a. planning

b. designing

c. execution

d. all the above.

47. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

48. Pick up the incorrect statement from the following:

a. The beginning and end of a job, are called events

b. The activity which consumes maximum time, is called a node

c. Logically and sequentially connected activities and events form a network

d. None of these.

49. For the taxiways, the following statement is true

a. The maximum longitudinal grade is 3%

b. The permissible rate of change of grade is 1%

c. The permissible transverse grade is 1.5%

d. All the above.

50. Pick up the component not applicable to aeroplanes

a. engines

b. air screw

c. fuse lage

d. none of these.