***Sub Engineer***

1. The first reading from a level station is

a. foresight

b. intermediate sight

c. back-sight

d. any sight.

2. Horizontal distances obtained tacheometerically are corrected for

a. slope correction

b. temperature correction

c. refraction and curvature correction

d. all the above.

3. Angles to a given pivot station observed from a number of traverse stations when plotted, the lines to the pivot station intersect at a common point

a. angular measurements are correct and not the linear measurements

b. linear measurements are correct and not the angular measurements

c. angular and linear measurements are correct and not the plotting of traverse

d. angular and linear measurements and also plotting of the traverse are correct.

4.  is the conventional sign of

a. temple

b. mosque

c. idgah

d. church

5. For high grade instruments the steel preferred to, is

a. cast steel

b. bessemer steel

c. mild steel

d. whitworth compressed steel.

6. Pig iron is manufactured from the ores by

a. dressing

b. calcination and roasting

c. smelting

d. all the above.

7. Wrought iron contains carbon about

a. 1.5% to 5.5%

b. 0.5% to 1.75%

c. 0.1% to 0.25%

d. none to these.

8. For the manufacture of stainless steel, steel is mixed with

a. chromium

b. nickel

c. tungsten

d. none of these.

9. The ratio of moments of inertia of a triangular section about its base and about a centroidal axis parallel to its base, is

a. 1.5

b. 2.0

c. 2.5

d. 3.0

10. A steel bar 20 mm in diameter simply-supported at its ends over a total span of 40 cm carries a load at its centre. If the maximum stress induced in the bar is limited to N/mm2, the bending strain energy stored in the bar, is

a. 411 N mm

b. 511 N mm

c. 611 Nmm

d. 711 N mm

11. A cylinder is said to be thin if the ratio of its thickness and diameter, is less than

a. 1/20

b. 1/15

c. 1/10

d. 1/5

12. The maximum resistance against rotation, is offered by the weld at a point

a. most distant

b. least distant

c. at either end

d. centrally located.

13. Pick up the correct statement from the following :

a. The fluids which follow  are known as Newtonian fluids

b. The fluids which do not follow the linear relationship between shear and rate of strain are known as non Newtonian fluids

c. The substances which flow after yield strains, are known as plastics

d. all the above.

14. The following is not a laminar flow

a. Flow in beds in ground water

b. Rise of water in plants through roots

c. Flow of blood in the arteries of human body

d. Flow in water pipe lines.

15. For the most economical trapezoidal open channel,

a. half of the top width must be equal to one of the sloping sides

b. the hydraulic mean depth must be equal to half the depth of flow

c. the semicircle drawn with top width as diameter must touch the three sides of the channel

d. All of these.

16. For a most economical trapezoidal open channel, the half of the top width must be equal to

a. the bed width

b. one sloping side

c. the depth of flow

d. None of these.

17. Pick up the correct statement from the following:

a. The porosity of a soil is defined as the ratio of the volume of voids to the gross volume of the soil

b. The bulk density of a soil is defined as the unit weight of the soil

c. The dry density of a soil is defined as weight of solids to the total volume of the soil

d. All the above.

18. A soil mass is said to be in plastic equilibrium if

a. it is stressed to maximum

b. it is on the verge of failure

c. it is in plastic stage

d. it starts flowing.

19. The consolidation time for soils

a. decreases with increasing permeability

b. increases rapidly with increasing size of soil mass

c. is independent of the magnitude of the stress change.

d. All the above.

20. The critical exist gradient of seepage water in soils, is

a. inversely proportional to the specific gravity

b. directly proportional to the specific gravity

c. inversely proportional to the voids ratio

d. none of these.

21. The minimum thickness of a flat slab is taken

a. L/32 for end panels without drops

b. L/36 for end panels without drops

c. L/36 for interior panels without drop

d. all the above.

22. Piles are usually driven by

a. diesel operated hammer

b. drop hammer

c. single acting steam hammer

d. all the above.

23. The radius of a bar bend to form a hook, should not be less than

a. twice the diameter

b. thrice the diameter

c. four times the diameter

d. five times the diameter

24. The minimum clear cover for R.C.C. columns shall be

a. greater of 40 mm or diameter

b. smaller of 40 mm or diameter

c. greater of 25 mm or diameter

d. smaller of 25 mm or diameter

25. Vertical construction joints are provided where the shearing forces are minimum in the case of

a. slabs

b. beams

c. girders

d. all of these.

26. The service area in a building means the area occupied by

a. stairs

b. toilets

c. light and shafts

d. all the above.

27. Strength of concrete with passage of time

a. increases

b. decreases

c. fluctuates

d. remains constant.

28. Ordinary concrete is not used for concrete grade

a. M 150

b. M 200

c. M 250

d. M 400

29. Specific yield of a well is

a. quantity of water than can be drawn from the well

b. flow of water per unit time

c. total quantity of water available in the well

d. quantity of water per unit time per unit draw-down.

30. In a reservoir the average volume of sediment deposition is 0.15 million cubic per year. If the dead storage and total capacity of the reservoir are 8 million cubic metres and 36 million cubic metres respectively,

a. the reservoir will theoretically be silted in 240 years

b. the reservoir will start reducing after 60 years

c. both (a) and (b)

d. neither (a) nor (b).

31. Drop manholes are the manholes

a. without entry ladders

b. without manhole covers

c. with depths more than 3.5 m

d. having drains at different levels.

32. In primary sedimentation, the 0.2 mm inorganic solids get separated if specific gravity is

a. 2.50

b. 2.55

c. 2.65

d. 2.75

33. Pick up the correct statement from the following:

a. Duty of canal water is usually calculated at the head discharge of a channel

b. Duty of canal water at the head of a channel and at the outlet of a distributary, differ considerably due to progressive losses of water

c. Duty of canal water at the outlet of a distributary, is generally known as outlet factor or outlet discharge factor

d. All the above.

34. Garnett's diagrams are used for graphical solution of design equations of a canal by

a. Lacey's theory

b. Kennedy's theory

c. Gibb's theory

d. Lindlay theory.

35. If straight sides of a triangular section of a lined canal with circular bottom of radius R, make an angle θ with horizontal, the area of its cross-section, is

a. R(θ + cot θ)

b. R2(θ + tan θ)

c. R2(θ + cot θ)

d. R(θ + tan θ)

36. The sinuosity of a meander is the ratio of

a. meander length and the width of meander

b. meander length and half width of the river

c. curved length and the straight distance

d. none of these.

37. Following type of pavement is generally known as flexible pavement

a. water-bound macadam roads

b. stabilised soil roads

c. road constructed with various layers of building material well compacted

d. all the above.

38. When a number of hair pin bends are introduced, a minimum intervening distance in between is kept

a. 20 m

b. 40 m

c. 60 m

d. 100 m

39. The minimum vertical clearance of overhanging cliffs or any other structure above the surface of a hill road, should be

a. 4 m

b. 5 m

c. 6 m

d. 4.5 m

40. If present A.D.T. is 5000 vehicles and annual increase is 10%, the average future flow after 5 years will be

a. 6050 vehicles

b. 7050 vehicles

c. 8050 vehicles

d. 9050 vehicles

41. Referring of given figure, pick up the correct statement from the following:

a. The total length of centre line of four walls is 20 m

b. Length of long wall out-to-out is 6.80 m

c. Length of short walls in-to-in is 3.20 m

d. All the above.

42. Anti-siphonage pipe is connected to

a. Main soil pipe

b. Bottom of P trap W.C.

c. Top of P trap W.C.

d. Side of water closet.

43. The correct prismoidal formula for volume is

a. D [first area + last area + ∑ Even area + 2 ∑ odd areas]

b.  [first area + last area + 4 ∑ Even area + 2 ∑ odd areas]

c.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas]

d.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas].

44. The detention period in a septic tank is assumed

a. 20 minutes

b. 25 minutes

c. 30 minutes

d. 40 minutes

45. In CPM analysis,

a. uncertainties are not allowed

b. activities are represented by arrows

c. beginning and end of an activity, are denoted by nodes

d. all the above.

46. If t is the duration of an activity, t1 is the latest finish possible moment of its preceding activity and t2 is the earliest start possible moment, the independent float of the activity is

a. (t1 - t2) - t

b. t - (t1 - t2)

c. (t1 + t2) - t

d. t + (t1 - t2)

47. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

48. Pick up the incorrect statement from the following:

a. The difference between the maximum time available for the job and actual time it consumes, is called total float

b. The difference between the latest start time and earliest start time of an activity, is called total float

c. The difference between the earliest finish time of an activity and the earliest start time of its successor activity, is called free float of the activity

d. None of these.

49. The lift off distance is the distance along the centre of the runway between the starting point and

a. end of the runway

b. end of stopway

c. point where air craft becomes air borne

d. point where air craft attains a height of 10.7 m

50. The distance between main gears is 10 m, and the distance of nose gear from centre of main gears is 30 m. If the angle of turning is 60°, the distance of centre of rotation from the nearer main gear, is

a. 12.30 m

b. 11.30 m

c. 10.30 m

d. 9.30 m