***Sub Engineer***

1. Pick up the correct statement from the following :

a. The vertical distance between a maximum ordinate and the next forward maximum ordinate represents the whole volume of the embankment

b. The vertical distance between a minimum ordinate and the next forward maximum ordinate represents the whole volume of a cutting

c. The area enclosed by a loop of the curve and balancing line, measures the haul in that direction.

d. all the above.

2. If the plane table is not horizontal in a direction at right angles to the alidade, the line of sight is parallel to the fiducial edge only for

a. horizontal sights

b. inclined sights upward

c. inclined sight downward

d. none of these.

3. A relatively fixed point of known elevation above datum, is called

a. bench mark

b. datum point

c. reduced level

d. reference point.

4. For a closed traverse the omitted measurements may be calculated

a. bearing of one side only

b. both length and bearing of one side

c. length or bearing of adjacent side

d. all the above.

5. Pick up the correct statement from the following:

a. Attrition test is done to find out the rate of wear of stones which are used in road construction

b. Crushing test is done to find out the compressive strength of the stone

c. Impact test is done to determine toughness of a stone

d. All the above.

6. Priming consists of

a. one part of white lead, 8 parts of chalk and four parts of twice boiled linseed oil

b. 8 parts of white lead, one part of chalk and four parts of twice boiled linseed oil

c. one part of white lead, 8 parts of chalk and one part of linseed oil

d. none to these.

7. Pig iron is manufactured from the ores by

a. dressing

b. calcination and roasting

c. smelting

d. all the above.

8. The property by virtue of which lime sets under water, is known as

a. slacking

b. setting

c. hydraulicity

d. calcining.

9. A steel bar 20 mm in diameter simply-supported at its ends over a total span of 40 cm carries a load at its centre. If the maximum stress induced in the bar is limited to N/mm2, the bending strain energy stored in the bar, is

a. 411 N mm

b. 511 N mm

c. 611 Nmm

d. 711 N mm

10. Pick up the correct statement from the following:

a. The pitch of a close coil spring, is very small

b. The angle made by the coil with horizontal, is called the angle of helix

c. In the open coil helical spring, the angle of helix is comparatively large

d. All the above.

11. In rectangular columns (cross-section b x h), the core is a

a. square of length b/2

b. rhombus of length h/2

c. rhombus of diagonals b/3 and h/3

d. none of the these.

12. A three hinged arch is loaded with an isolated load 1000 kg at a horizontal distance of 2.5 m from the crown, 1 m above the level of hinges at the supports 10 metres apart. The horizontal thrust is

a. 1250 kg

b. 125 kg

c. 750 kg

d. 2500 kg

13. The equation  for the viscosity, is suggested

a. Newton

b. Chezy

c. Bezin

d. Helmholtz.

14. An open container filled with water is moved vertically upward with a uniform linear acceleration. The pressure at its bottom will be

a. greater than static pressure

b. equal to static pressure

c. lesser than static pressure

d. none of these.

15. Dimensions of the dynamic viscosity (μ) are

a. MLT-2

b. M-1L-1T-1

c. ML-1T-2

d. ML-1T-1.

16. Discharge with velocity of approach, over a rectangular weir is

a.  Cd x L x 2g [(H + ha)3/2 + ha3/2]

b.  Cd x L x 2g [(H + ha)3/2 - ha3/2]

c.  Cd x L x 2g [(H - ha)3/2 + ha3/2]

d.  Cd x L x 2g [(H - ha)3/2 - ha3/2]

17. A partially saturated soil is classified as

a. one phase soil

b. two phase soil

c. three phase soil

d. four phase soil.

18. A direct shear test possesses the following disadvantage:

a. A relatively thin thickness of sample permits quick drainage

b. A relatively thin thickness of sample permits quick dissipation of pore pressure developed during the test

c. As the test progresses the area under shear, gradually changes

d. none of these.

19. Buoyant unit weight equals the saturated density

a. multiplied by unit weight of water

b. divided by unit weight of water

c. plus unit weight of water

d. minus unit weight of water.

20. An unsaturated 100 cm3 sample of soil weighs 190 g. If its dried weight is 160 g, water content of the soil, is

a. 0.188

b. 0.288

c. 0.388

d. 0.488

21. Minimum spacing between horizontal parallel reinforcement of the same size should not be less than

a. one diameter

b. 2.5 diameters

c. 3 diameters

d. 3.5 diameters

22. Total pressure on the vertical face of a retaining wall of height h acts parallel to free surface and from the base at a distance of

a. h/4

b. h/3

c. h/2

d. 2h/3

23. Dimensions of a beam need be changed if the shear stress is more than

a. 10 kg/cm2

b. 15 kg/cm2

c. 20 kg/cm2

d. 25 kg/cm2

24. If the diameter of longitudinal bars of a square column is 16 mm, the diameter of lateral ties should not be less than

a. 5 mm

b. 6 mm

c. 8 mm

d. 10 mm

25. The type of stone masonry in which stones of same height are laid in layers, is called

a. random rubble masonry

b. course rubble masonry

c. uncoursed rubble masonry

d. ashlar masonry.

26. The depth of concrete bed of the foundation depends upon

a. the projection of the concrete block beyond the footing over it

b. the upward soil pressure

c. the mix of the concrete

d. all the above.

27. The diameter of the Vicat plunger is 10 mm and its length varies from

a. 30 mm to 40 mm

b. 40 mm to 50 mm

c. 50 mm to 60 mm

d. none of these.

28. According to IS : 382-1963, a good aggregate should be

a. chemically inert

b. sufficiently strong

c. hard and durable

d. all the above.

29. Recuperation test is carried out to determine

a. turbidity of water

b. pH value of water

c. yield of well

d. discharge from a well.

30. The lowest outlet sluice in a dam is provided

a. below the dead storage

b. on the top level of dead storage

c. on the top level of useful storage

d. at the centre of the dam.

31. The sewer pipes

a. carry sewage as gravity conduits

b. are designed for generating self-cleansing velocities at different discharge

c. should resist the wear and tear caused due to abrasion

d. all the above.

32. If D is the diameter of a circular sewer and D' is the top horizontal diameter of an equivalent egg shaped section, the relationship which holds good, is

a. D' = 0.64 D

b. D' = 0.74 D

c. D' = 0.84 D

d. D' = 0.94 D.

33. The difference in level between the top of a bank and supply level in a canal, is called

a. berm

b. free board

c. height of bank

d. none of these.

34. According to Lacey, in regime conditions

a. silt is kept in suspension by vertical components of eddies

b. entire cross-section of the channel is generated at all points by the forces normal to the wetted perimeter

c. both (a) and (b)

d. neither (a) nor (b).

35. A hydraulic jump is generally formed when a stream moving with

a. a hyper-critical velocity meets a stream moving with a critical velocity

b. a hyper-critical velocity meets a stream moving with a hyper-critical velocity

c. A hyper-critical velocity meets a stream moving with a sub-critical velocity

d. a sub-critical velocity meets a stream moving with a hyper-critical velocity.

36. If the straight sides of a triangular section of a lined canal with circular bottom of radius D, make an angle θ with horizontal, the hydraulic mean depth is

a. D/2

b. D/3

c. D/4

d. D/5

37. Maximum number of vehicles that can pass a given point on a lane during one hour without creating unreasonable delay, is known as

a. traffic density of lane

b. basic capacity of lane

c. probable capacity of lane

d. practical capacity of lane.

38. If the radii of a compound curve and a reverse curve are respectively the same, the length of common tangent

a. of compound curve will be more

b. of reverse curve will be more

c. of both curves will be equal

d. none of these.

39. As per recommendations of I.R.C., traffic volume study is carried out for rural roads for 7 days continuously during

a. harvesing

b. lean season

c. harvesting and lean season

d. none of these.

40. On earth roads, the camber should preferably be

a. 1 in 20 to 1 in 24

b. 1 in 30 to 1 in 48

c. 1 in 15 to 1 in 20

d. 1 in 10 to 1 in 15

41. The weight of an item is measured correct to nearest

a. 0.50 kg

b. 0.75 kg

c. 1.00 kg

d. 5 kg

42. The ground surface slopes 1 in 50 along a proposed railway embankment 150 m in length. The height of the embankment at zero chainage is 0.5 m, the width is 11 m and side slopes 2:1. If the falling gradient of the embankment is 1 in 150, the quantity of the earthwork calculated by prismoidal formula, is

a. 3250 m3

b. 3225 m3

c. 3275 m3

d. 3300 m3

43. The measurement is made in square metre in case of

a. R.C.C. structure

b. Hollow concrete block wall

c. Concrete fencing posts

d. None of these.

44. Pick up the correct statement from the following:

a. Additional supervising staff engaged at work site, is called work charged establishment

b. Detailed specifications specify qualities, quantities and the proportions of materials to be used for a particular item

c. The cost per unit at which the article can be procured, from the open market at a given time, is called 'market rate'

d. All the above.

45. While scheduling a project by C.P.M.

a. required time for each activity is established

b. sequence of various activities is made according to their importance

c. net work is drawn by connecting the activities and the events

d. All the above.

46. An Executive Engineer may have powers upto

a. Rs. 25,000

b. Rs. 50,000

c. Rs. 100,000

d. Rs. 200,000

47. Power stations are generally treated as

a. light construction

b. heavy construction

c. industrial construction

d. electrical construction.

48. The artificial activity which indicates that an activity following it, cannot be started unless the preceding activity is complete, is known as

a. event

b. free float

c. dummy

d. constant

49. Pick up the incorrect statement from the following:

a. Channelization of pavement is caused by constant use of tri-cycle gears of aircraft

b. Blast pads are used over cohesionless soils to resist erosion due to tremendous speed of the jet blast

c. Over-run areas of at least 300 m length on either side of the runway are provided

d. None of these.

50. Pick up the correct statement from the following:

a. The speed of the aircraft relative to the ground, is called cruising speed

b. The speed of the aircraft relative to wind, is called air speed

c. When wind is blowing the direction of the flight, air speed is less than cruising speed

d. All the above.