***Sub Engineer***

1. Transition curves are introduced at either end of a circular curve, to obtain

a. gradually decrease of curvature from zero at the tangent point to the specified quantity at the junction of the transition curve with main curve

b. gradual increase of super-elevation from zero at the tangent point to the specified amount at the junction of the transition curve with main curve

c. gradual change of gradient from zero at the tangent point to the specified amount at the junction of the transition curve with main curve

d. none of these.

2. If i is the stadia distance, f is the focal length and d is the distance between the objective and vertical axis of the techeometer, the multiplying constant, is

a. f/i

b. i/f

c. (f + d)

d. f/d.

3. A theodolite is said to be in perfect adjustment if

a. rotation axis is vertical to the transit axis

b. transit axis is perpendicular to line of collimation

c. line of collimation sweeps out a vertical plane while the telescope is elevated or depressed

d. all the above.

4. The orthographical projection of a traverse leg upon the reference meridian, is known as

a. departure of leg

b. latitude to the leg

c. co-ordinate of the leg

d. bearing of the leg.

5. The minimum percentage of silica, alumina and ferric oxide in lime for white washing, is

a. 15

b. 10

c. 5

d. 0.

6. The weight of 1 m3 of brick earth, is about

a. 1200 kg

b. 1500 kg

c. 1800 kg

d. 2000 kg.

7. Due to attack of dry rot, the timber

a. cracks

b. shrinks

c. reduces to powder

d. none of these.

8. The softest rock is

a. marble

b. diamond

c. talc

d. quartz.

9. A body is said to be in equilibrium if

a. it moves horizontally

b. it moves vertically

c. it rotates about its C.G.

d. none of these.

10. Pick up the correct statement from the following:

a. Hoop strain of the walls of a cylinder due to liquid is

b. Longitudinal strain in the walls of a cylinder due to liquid is

c. Volumetric change in the cylinderdue to liquid is

d. All the above.

11. The range within which a load can be applied on a rectangular column, to avoid any tensile stress, is

a. one-third of the base

b. one-fourth of the base

c. one-fifth of the base

d. one sixth of the base on either side of centroid.

12. A steel rod of 2 cm diameter and 5 metres long is subjected to an axial pull of 3000 kg. If E = 2.1 x 106, the elongation of the rod will be

a. 2.275 mm

b. 0.2275 mm

c. 0.02275 mm

d. 2.02275 mm.

13. The continuity equation

a. expresses the relationship between work and energy

b. relates the momentum per unit volume between two points on a stream line

c. relates mass rate of flow along a stream line

d. requires that Newton's second law of motion be satisfied at every point in fluid.

14. Flow in pipes is laminar if Reynold number is

a. less than 2100

b. more than 3000

c. between 2100 and 3000

d. none of these.

15. The thickness of a sharp crested weir is kept less than

a. one-half of the height of water on the sill

b. one-fourth of the height of water on the sill

c. two-third of the height of water on the sill

d. none of these.

16. The value of kinetic energy correction factor (a) for a laminar flow through a circular pipe, is

a. 1.0

b. l.5

c. 2.0

d. 2.5

17. Under-reamed piles are generally

a. driven piles

b. bored piles

c. precast piles

d. all the above.

18. In a fow net

a. flow lines and equipotential lines cross each other at right angles

b. fields are rectangles whose length is twice the breadth

c. smaller the dimensions of the field, smaller will be the hydraulic gradient and velocity of flow through it

d. for homogeneous soil, the curves are smooth and circular.

19. The ratio of the weight of given volume of soil solids to the weight of an equal volume of distilled water at the given temperature, is known

a. porosity

b. specific gravity

c. void ratio

d. water content.

20. Through a point in a loaded soil mass, there exists n typical planes mutually orthogonal on which the stress is wholly normal and no shear stress acts, if n is

a. 1

b. 2

c. 3

d. 4

21. If R and T are rise and tread of a stair spanning horizontally, the steps are supported by a wall on one side and by a stringer beam on the other side, the steps are designed as beams of width

a. R + T

b. T-R

c. R2 + T2

d. R-T

22. Enlarged head of a supporting column of a flat slabs is technically .known as

a. top of the column

b. capital

c. drop panel

d. none of these.

23. According to the steel beam theory of doubly reinforced beams

a. compression is resisted by compression steel

b. stress in tension steel equals the stress in compression steel

c. no stress is developed in compression concrete as well as in tension concrete

d. all the above.

24. An R.C.C beam of 25 cm width has a clear span of 5 metres and carries a U.D.L. of 2000 kg/m inclusive of its self weight. If the lever arm of the section is 45 cm., the beam is

a. safe in shear

b. is safe with stirrups

c. is safe with stirrups and inclined members

d. needs revision of the section.

25. The foundations are placed below ground level, to increase

a. strength

b. workability

c. stability of structure

d. all the above.

26. A concrete structure is set on fire and the temperature raises to 1000°C. The strength of concrete as compared to original strength reduces to

a. 10%

b. 15%

c. 20%

d. 25%.

27. Percentage of pozzolanic material containing clay upto 80% used for the manufacture of pozzolana cement, is

a. 30%

b. 40%

c. 50%

d. 60%

28. Proper proportioning of concrete, ensures

a. desired durability

b. water tightness of the structure

c. resistance to water

d. all the above.

29. Perched acquifers are generally found

a. on the surface of the ground

b. below the surface of the ground but above water table

c. below the water table

d. all the above.

30. Water supply system includes

a. digging a well for water

b. construction of dams

c. construction of canals

d. entire arrangement from source to distribution.

31. Design period of 40 to 50 years is adopted for

a. branch sewers

b. main sewers

c. trunk sewers

d. all the above.

32. Stoneware sewers are available in size

a. 15 cm

b. 20 cm

c. 25 cm

d. all the above.

33. Lacey's equation for calculating flood discharge in rivers, is

a. V = 10.8 R1/2 S1/2

b. V = 10.8 R2/3 S1/2

c. V = 10.8 R2/3 S1/3

d. V = 10.8 R1/3 S2/3

34. The useful moisture of soil, is equal to its

a. field capacity

b. saturation capacity

c. moisture content at permanent wilting point

d. difference between filed capacity and permanent wilting point within the root zone of plants.

35. If water table is comparatively high, the irrigation canal becomes useless, due to

a. water logging of the cultivated areas

b. uncertain water demand

c. spread of malaria

d. all the above.

36. For diversion of flood water of rivers, the type of canal constructed, is

a. ridge canal

b. perennial canal

c. inundation canal

d. canal.

37. The absolute minimum radius of horizontal curve for a design speed 60 km ph is

a. 131 m

b. 210 m

c. 360 m

d. none of these.

38. Minimum permissible speed on high speed roads, is decided on the basis of

a. 15 percentile cumulative frequency

b. 20 percentile cumulative frequency

c. 30 percentile cumulative frequency

d. 40 percentile cumulative frequency.

39. In case of cement concrete pavements, pick up the incorrect statement

a. initial cost of construction is high

b. initial cost of construction is low

c. visibility during nights is high

d. none of these.

40. If the coefficient of friction on the road surface is 0.15 and a maximum super-elevation 1 in 15 is provided, the maximum speed of the vehicles on a curve of 100 metre radius, is

a. 42.44 kg/hour

b. 52.44 km/hour

c. 62.44 km/hour

d. 72.44 km/hour

41. Pick up the correct statement from the following:

a. Area of canal in cutting = BD + Sd2 where B = bed width, d = depth of cutting and S is the side slope

b. Area of the bank of canal = B1d1 + Sd2 where B1, d1 and S are the width of bank, height of the bank above N.S.L. and side slope respectively

c. If F.S.L. is above N.S.L the canal is called partly in cutting and partly in filling and berms are designed as 2d where d is full supply depth

d. All the above.

42. The damp proof course (D.P.C.) is measured in

a. Cub.m

b. Sq m

c. Metres

d. None of these

43. Pick up the incorrect statement from the following:

a. Dimensions are measured to the nearest 0.01 m

b. Areas are measured to the nearest 0.01 sq.m

c. Cubic contents are measured to the nearest 0.1 cm m

d. Weights are measured to the nearest 0.001 tonnes

44. According to ISI method of measurement, the order of the sequence is

a. Length, breadth, height

b. Breadth, length, height

c. Height, length, breadth

d. None of these.

45. A CPM family includes

a. CPP (Critical Path Plotted)

b. MCE (Minimum Cost Expenditure)

c. CPS (Critical Path Scheduling)

d. All the above.

46. Military organisation is known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. none of these.

47. Pick up the correct statement from the following:

a. PERT analysis is event oriented

b. CPM does not make any allowance for the uncertainties in the duration of time

c. In CPM, the time is related to cost

d. All the above.

48. Critical Path Net Work helps an engineer

a. to concentrate his attention on critical activities

b. to divert the resources from non-critical advanced activities to critical activities

c. to be cautious for avoiding any delay in the critical activities to avoid delay of the whole project

d. all the above.

49. The air is blowing at a speed of 75 knots in the direction of the aircraft flying at a ground speed of 775 knots. The air speed of the aircraft, is

a. 775 knots

b. 75 knots

c. 850 knots

d. 675 knots

50. Pick up the incorrect statement from the following:

a. L.M.M. is installed at 1.0 km from the threshold

b. Localizer antenna is installed at 300 m from the other end of the runway

c. Glide slope antenna is installed at the centre of the runway about 150 m on one side

d. None of these.