***Sub Engineer***

1. In reciprocal levelling, the error which is not completely eliminated, is due to

a. earth's curvature

b. non-adjustment of line of collimation

c. refraction

d. non-adjustment of the bubble tube.

2. The chord of a curve less than peg interval, is known as

a. small chord

b. sub-chord

c. normal chord

d. short chord.

3. The difference in the lengths of an arc and its subtended chord on the earth surface for a distance of 18.2 km, is only

a. 1 cm

b. 5 cm

c. 10 cm

d. 100 cm.

4. If h1 and h2 are the differences in level between ground and the formation levels, m is the slope of the sloping sides. D is the distance between the cross sections then, prismoidal correction for a level section is

a. D/3m(h1 - h2)

b. D/6m(h1 - h2)2

c. D/6m(h1 - h2)3

d. D/6m(h1 + h2)2

5. The base material for distemper, is

a. chalk

b. lime

c. lime putty

d. cement wash.

6. Lime concrete is generally used for

a. wall foundations

b. flooring at ground level

c. both (a) and (b)

d. neither (a) nor (b).

7. For the manufacture of plywood, veneers are placed so that grains of adjacent veneers

a. run at right angles

b. parallel

c. inclined at 45°

d. inclined at 60°.

8. For a 50 kg cement bag water required, is

a. 18.5 litres

b. 20.5 litres

c. 22.5 litres

d. 25 litres.

9. If a concrete column 200 x 200 mm in cross-section is reinforced with four steel bars of 1200 mm2 total cross-sectional area. Calculate the safe load for the column if permissible stress in concrete is 5 N/mm2 and Es is 15 Ec

a. 274 MN

b. 284 MN

c. 294 MN

d. None of these.

10. The forces acting normally on the cross section of a bar shown in the given figure introduce

a. compressive stress

b. tensile stress

c. shear stress

d. none of these.

11. The ratio of the tensile stress developed in the wall of a boiler in the circumferential direction to the tensile stress in the axial direction, is

a. 4

b. 3

c. 2

d. 1

12. The minimum number of rivets for the connection of a gusset plate, is

a. 1

b. 2

c. 3

d. 4

13. Water displaced by a floating wooden block of density 0.75, 5 m long, 2 m wide and 3 m high, is

a. 17.5 m3

b. 20.0 m3

c. 22.5 m3

d. 25 km3.

14. The unit of the viscosity is

a. kg sec/m2

b. Newton sec per m2

c. Newton-sec2/m3

d. m2 per sec.

15. In flowing liquids pitot tubes are used measure

a. discharge

b. pressure

c. velocity

d. depth.

16. Chezy's constant  is suggested by

a. Bazin

b. Kutter

c. Manning

d. Powell.

17. The coefficient of curvature for a well graded soil, must be between

a. 0.5 to 1.0

b. 1.0 to 3.0

c. 3.0 to 4.0

d. 4.0 to 5.0

18. The specific gravity of Calcite is

a. 2.65

b. 2.72

c. 2.85

d. 2.90

19. Back fill with a sloping surface exerts a total active pressure Pa on the wall of height H and acts at

a. H/4 above the base parallel to base

b. H/2 above the base parallel to base

c. H/3 above the base parallel to base

d. H/5 above the base parallel to base.

20. Pick up the correct statement from the following:

a. The void space between the soil grains, is filled partly with air and partly with water

b. In perfectly saturated soil, the voids are completely filled with water

c. In dry soil, the voids are completely filled with air

d. all the above.

21. The effective width of a column strip of a flat slab, is

a. half the width of the panel

b. radius of the column

c. diameter of the column

d. none of these.

22. Design of a two way slab simply supported on edges and having no provision to prevent the corners from lifting, is made by

a. Marcus formula

b. Rankine Grashoff formula

c. Grashoff formula

d. Rankine-Marcus formula.

23. The angle of repose of a soil is the maximum angle which the outer face of the soil mass makes

a. with the horizontal

b. with the vertical

c. with the perpendicular to. the inclined plane of the soil

d. none of these.

24. If the depth of actual neutral axis of a doubly reinforced beam

a. is greater than the depth of critical neutral axis, the concrete attains its maximum stress earlier

b. is less than the depth of critical neutral axis, the steel in the tensile zone attains its maximum stress earlier

c. is equal to the depth of critical neutral axis, the concrete and steel attain their maximum stresses simultanesouly

d. all the above.

25. The bearing capacity of a water logged soil, may be improved by

a. grouting

b. chemical action

c. drainage

d. compaction.

26. Pick up the correct statement from the following :

a. the pile driven in sand is called sand pile

b. the drilled hole filled with sand is called sand pile

c. the sand piles are used for bearing purposes

d. None of these.

27. Pick up the correct statement from the following i.

a. Higher workability indicates deficiency of sand

b. If the concrete mix is dry, the slump is zero

c. Concrete mix having zero slump, is unsuitable for high strength.

d. All the above.

28. Concrete mainly consists of

a. aggregates

b. admixture

c. water

d. all the above.

29. Pick up the incorrect statement from the following. The underground sources of water, is from

a. springs

b. infiltration wells

c. storage reservoirs

d. none of these.

30. By adding 1 ml of orthotolidine solution to 100 ml chlorinated water taken after contact period, the residual chlorine makes the colour of solution

a. yellowish

b. greenish

c. bluish

d. reddish

31. The sewer which collects the discharge from a collecting system and delivers it to a treatment plant, is known

a. house sewer

b. lateral sewer

c. branch sewer

d. sewer outfall.

32. In a city the ratio of the drainage to sewage is 20, the percentage discharge passing through non-monsoon periods, is

a. 5

b. 10

c. 15

d. 20

33. Pick up the correct statement from the following:

a. Static level of water in an open well indicates the position of the water table

b. Saturated pores of a soil in the capillary zone above water table have negative pressure

c. At water table, hydrostatic pressure is zero

d. All the above.

34. Lacy's regime condition is obtained if

a. silt grade in the channel is variable

b. discharge in the channel is variable

c. silt charge in the channel is variable

d. channel flows in unlimited, incoherent alluvium of the same character as that transported material.

35. If the straight sides of a triangular section of a lined canal with circular bottom of radius D, make an angle θ with horizontal, the hydraulic mean depth is

a. D/2

b. D/3

c. D/4

d. D/5

36. In case of a trapezoidal notch fall

a. top length of the piers should not be less than their thickness

b. splay up stream from the notch section is 45°

c. splay down stream from the notch section is

d. all the above.

37. Reduction of load capacity in a ruling gradient of

a. 1 in 15, is 15%

b. 1 in 20, is 10%

c. 1 in 25, is 25%

d. none of these.

38. The length of the side of warning sign boards of roads is

a. 30 cm

b. 40 cm

c. 45 cm

d. 50 cm

39. Pick up the incorrect statement from the following. The width of the right-of-way is decided so as to accommodate

a. side slopes

b. horizontal curve

c. vertical curve

d. provision of future widening.

40. Floating gradients are generally provided

a. along minimum gradients

b. at summit curves

c. at valley curves

d. every where.

41. In case of laying gullies, siphons, intercepting traps, the cost includes

a. Setting and laying

b. Bed concreting

c. Connection to drains

d. All of these.

42. The slope of the oultet of 'P trap' below the horizontal is kept

a. 8°

b. 10°

c. 12°

d. 14°

43. The most reliable estiamte is

a. Detailed estimate

b. Preliminary estimate

c. Plinth area estimate

d. Cube rate estimate

44. The expected out turn of brick work in cement mortar in foundation and plinth per mason per day, is

a. 1.00 m3

b. 1.25 m3

c. 1.50 m3

d. 1.75 m3

45. Time and progress chart of a construction, is also known as

a. Gantt chart

b. Modified Mile stone chart

c. Critical path method chart

d. All the above.

46. Pick up the correct statement from the following:

a. If the float is positive and the activity is delayed by a period equal to its total float, the completion of project is not delayed

b. If the float of an activity is negative, delay in its performance is bound to delay the completion of project

c. If the float of an activity is zero, the activity is critical and any delay in its performance will delay the whole project

d. All the above.

47. Pick up the correct statement from the following:

a. Backward pass is used for calculating the latest occurence time

b. Maximum value of earliest expected time is used if there are more than one value of any event

c. Minimum value of latest occurrence time is used if there are more than one value of any event

d. All the above.

48. The most popular type of organisation used for Civil Engineering Constructions, is

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

49. The meterological condition which influences the size and location of an air port is

a. air density

b. reduced level

c. wind direction

d. all the above.

50. The fuse large includes

a. passengers chamber

b. pilot's cabin

c. tail of aircraft

d. all the above.