***Sub Engineer***

1. Volume of the earth work may be calculated by

a. end areas

b. Prismoidal formula

c. Trapezoidal

d. all the above.

2. Pick up the correct statement from the following :

a. mistakes arise from inattention, inexperience or carelessness

b. systematic errors persist and have regular effects in the survey performances

c. accidental errors occur inspite of every precaution is taken

d. all the above.

3.  is the conventional sign of

a. temple

b. mosque

c. idgah

d. church

4. If the radius of a simple curve is R, the length of the chord for calculating offsets by the method of chords produced, should not exceed.

a. R/10

b. R/15

c. R/20

d. R/25.

5. The melting point of silica is :

a. 1570°C

b. 1630°C

c. 1730°C

d. 1850°C.

6. The commonly used thinner in oil paints, is

a. turpentine

b. both (a) and (b)

c. neither (a) nor (b)

d. none the these.

7. To give a brilliant finish, the type of varnish used, is

a. spirit varnish

b. turpentine varnish

c. oil varnish

d. none of these.

8. If the furnace is provided with insufficient fuel at low temperatures, the type of pig iron produced, is called

a. Bessemer pig

b. Grey or foundry pig

c. White or forge pig

d. Mottled pig.

9. The assumption in the theory of bending of beams, is :

a. material is isotropic

b. Young's modulus is same in tension as well as in compression

c. each layer is independent to expand or to contract

d. all the above.

10. For a strongest rectangular beam cut from a circular log, the ratio of the width and depth, is

a. 0.404

b. 0.505

c. 0.606

d. 0.707

11. The maximum deflection of

a. a fixed ended beam carrying a distributed load over the span is

b. a fixed ended beam carrying a concentrated load at the mid span is

c. a cantilever beam subjected to a moment M at the free end is

d. All the above.

12. During a tensile test on a ductile material

a. nominal stress at fracture is higher than the ultimate stress

b. true stress at fracture is higher than the ultimate stress

c. true stress at fracture is the same as the ultimate stress

d. none of these.

13. In a centrifugual pump casing, the flow of water leaving the impeller, is

a. rectilinear flow

b. radial flow

c. free vortex motion

d. forced vertex.

14. A fluid particle may possess the displacement of

a. translation

b. rotation

c. distortion

d. all the above.

15. For exerting a pressure of 4.8 kg/cm2, the depth of oil (specific gravity 0.8), should be

a. 41 cm

b. 56 cm

c. 60 cm

d. 76 cm.

16. Orifice-meter is used to measure

a. pressure at the point

b. discharge

c. average speed

d. velocity.

17. Pick up the incorrect statement from the following:

a. Compaction has no effect on the structure of a soil

b. Permeability decreases with increase in the dry density of a compacted soil

c. A wet side compacted soil is more compressible than a dry side compacted soil

d. Dry side compaction soils swell more when given access to moisture

18. The ratio of the weight of water to the weight of solids in a given mass of soil, is known

a. porosity

b. specific gravity

c. void ratio

d. water content.

19. Pile foundations are generally preferred to for

a. bridge foundations

b. sky scrapper buildings

c. residential buildings

d. runways.

20. The weight of a pycnometer containing 400 g sand and water full to the top is 2150 g. The weight of pycnometer full of clean water is 1950 g. If specific gravity of the soil is 2.5, the water content is

a. 5%

b. 10%

c. 15%

d. 20%

21. The width of the flange of a T-beam should be less than

a. one-third of the effective span of the T-beam

b. distance between the centres of T-beam

c. breadth of the rib plus twelve times the thickness of the slab

d. least of the above.

22. The toe projection of foundation slabs is taken

a. as one third of the base

b. as one sixth of overall height of the wall

c. equal to heel slab

d. below ground surface.

23. The horizontal portion of a step in a stairs case, is known as

a. rise

b. flight

c. winder

d. tread.

24. Pick up the assumption for the design of a prestressed concrete member from the following :

a. A transverse plane section remains a plane after bending

b. During deformation limits, Hook's law is equally applicable to concrete as well as to steel

c. Variation of stress in reinforcement due to changes in external loading is negligible

d. All the above.

25. In case of multi-storeyed buildings, the forms to be removed first are

a. sides of beams and girders

b. column forms

c. bottom of beams and girders

d. all the above at the same time.

26. The brick laid with its length parallel to the face of a wall, is a known as

a. header

b. stretcher

c. closer

d. none of these.

27. Bulking of sand is

a. mixing of different sizes of sand particles

b. mixing of lime with sand

c. maximum water with sand

d. swelling of sand when wetted.

28. Curing a concrete for long period ensures better

a. strength

b. water resistance

c. water tightness and durability

d. all the above.

29. A pressure conduit laid under ground, may not be subjected to

a. pressure due to external load

b. longitudinal temperature stress

c. longitudinal stresses due to unbalanced pressure to bends

d. none of these.

30. In a rapid gravity filter

a. raw water from the source is supplied

b. disinfected raw water is supplied

c. raw water passed through coagulation tank is supplied

d. none of these.

31. Which one of the following tests is used for testing sewer pipes :

a. water test

b. ball test

c. mirror test

d. all of these.

32. For the survival of fish in a river stream, the minimum dissolved oxygen is prescribed

a. 3 PPm

b. 4 PPm

c. 5 PPm

d. 10 ppm.

33. For the design of major hydraulic structures on the canals, the method generally preferred to, is based on

a. Bligh's theory

b. Electrical analogy method

c. The relaxation method

d. Khosla's method of independent variables.

34. According to Khosla, the exist gradient of surface flow

a. depends upon the b/d ratio

b. is independent of the b/d ratio

c. is independent of the depths of d/s cut off walls

d. none of these.

35. In Montague type fall

a. a straight glacis is provided

b. a circular glacis is provided

c. a parabolic glacis is provided

d. no glacis is provided.

36. The water face of the guide banks, is protected by

a. one men stone pitching

b. two man stone pitching

c. three man stone pitching

d. four man stone pitching

37. The first stage of deciding the alignment of a hill road, is

a. reconnaissance

b. detailed survey

c. trace-out

d. preliminary survey.

38. The minimum cross fall of shoulders is kept

a. 1.0%

b. 1.5%

c. 2.5%

d. 3%

39. The minimum superelevation in rolling terrain in plains, is limited to

a. 5%

b. 6%

c. 7%

d. 10%

40. If the velocity of moving vehicles on a road is 24 km/per hour, stopping distance is 19 metres and average length of vehicles is 6 metres, the basic capacity of lane, is

a. 500 vehicles per hour

b. 700 vehicles per hour

c. 1000 vehicles per hour

d. 1250 vehicles per hour

41. The total length of a cranked bar through a distance (d) at 45° in case of a beam of effective length L, is

a. L + 0.42 d

b. L + 2 x 0.42 d

c. L - 0.42 d

d. L - 2 x 0.4 d

42. For 100 sq. m cement concrete (1 : 2: 4) 4 cm thick floor, the quantity of cement required, is

a. 0.90 m3

b. 0.94 m3

c. 0.98 m3

d. 1.00 m3

43. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

44. The expected out turn of brick work in cement mortar in foundation and plinth per mason per day, is

a. 1.00 m3

b. 1.25 m3

c. 1.50 m3

d. 1.75 m3

45. Modular co-ordination of construction means proper

a. planning

b. designing

c. execution

d. all the above.

46. The object of technical planning, is

a. preparation of estimates

b. initiating the procurement action of resources

c. taking remedial action for likely bottleneck in the execution

d. all the above.

47. A critical ratio scheduling

a. determines the status of each activity

b. adjusts automatically changes in activity progress

c. is a dynamic system

d. none of these.

48. A CPM family includes

a. CPP (Critical Path Plotted)

b. MCE (Minimum Cost Expenditure)

c. CPS (Critical Path Scheduling)

d. All the above.

49. Pick up the correct statement from the following:

a. Approximate geometric centre of the landing area, is called air port reference point

b. The boundaries of horizontal surface and conical surface are marked with reference to air port reference point

c. The location of the air port on a map is the position of air port reference point

d. All the above.

50. The runway orientation is made so that landing and take off are

a. against the wind direction

b. along the wind direction

c. perpendicular to wind direction

d. none of these.