***Sub Engineer***

1. The sensitiveness of a level tube decreases if

a. radius of curvature of its inner surface is increased

b. diameter of the tube is increased

c. length of the vapour bubble is increased

d. both viscosity and surface tension are increased.

2. The slope correction may be ignored if

a. the slope of the ground is less than 3°

b. to slope of the ground is say 1 in 19

c. both (a) and (b)

d. neither (a) nor (b)

3. The accuracy of measurement in chain surveying, does not depend upon

a. length of the offset

b. scale of the plotting

c. importance of the features

d. general layout of the chain lines.

4. To avoid large centering error with very short legs, observations are generally made

a. to chain pins

b. by using optical system for centering the theodolite

c. to a target fixed on theodolite tripod on which theodolite may be fitted easily

d. all the above.

5. The percentage of alumina and silica in good fire clay vary respectively is

a. 25, 75

b. 30, 70

c. 35, 65

d. All of these.

6. The most durable varnish is

a. spirit varnish

b. turpentine varnish

c. oil varnish

d. none of these.

7. Asbestos is

a. an incombustible fire proof material

b. an organic substance

c. a bad insulator for sound and heat

d. all the above.

8. A stone is rejected if it absorbs water more than

a. 10%

b. 15%

c. 20%

d. 25%.

9. A cantilever of length 2 cm and depth 10 cm tapers in plan from a width 24 cm to zero at its free end. If the modulus of elasticity of the material is 0.2 x 106 N/mm2, the deflection of the free end, is

a. 3 mm

b. 4 mm

c. 5 mm

d. 6 mm

10. The general expression for the B.M. of a beam of length l is the beam carries

a. a uniformly distributed load w/unit length

b. a load varying linearly from zero at one end to w at the other end

c. an isolated load at mid span

d. none of these.

11. The law which states, "within elastic limits strain produced is proportional to the stress producing it", is known as

a. Stress law

b. Hooke's law

c. Poisson's law

d. none of these.

12. A member which is subjected to reversible tensile or compressive stress may fail at a stress lower than the ultimate stress of the material. This property of metal, is called

a. plasticity of the metal

b. elasticity of the metal

c. fatigue of the metal

d. workability of the metal.

13. For solving network problems of pipes, necessary condition is

a. continuity equation

b. energy equation

c. Darcy-Weisbach equation

d. all the above.

14. A rectangular channel 6 m wide and 3 m deep and having a bed slope as 1 in 2000 is running full. If Chezy's constant C = 54.8, pick up the correct specification of the channel from the following :

a. hydraulic mean depth = 1.5 m

b. Velocity of flow = 1.5 m/sec

c. Rate of flow = 27 m3/sec

d. All the above.

15. For exerting a pressure of 4.8 kg/cm2, the depth of oil (specific gravity 0.8), should be

a. 41 cm

b. 56 cm

c. 60 cm

d. 76 cm.

16. A jet projected at an angle of 45θ, 40 m from the foot of a vertical column, just reaches the top of the column. The height of the column is

a. 20 m

b. 30 m

c. 40 m

d. 60 m.

17. If σ0' and σ' represent initial and increased pressure ; e0 and e void ratios corresponding to initial and increased pressure ; and C0 the compression index (dimensionless), then, the virgin compression curve as expressed by Terzaghi empirical formula is

a.  e = e0 + Cc log10 σ'/σ0'

b. e = e0 - Cc log10 σ'/σ0'

c. e0 = e - Cc log10 σ'/σ0'

d. e0 = e + Cc log10 σ0'/σ'

18. Fundamental relationship between dry density (γd), specific gravity (G), water content (ω) and percentage of air voids (na) is :

a. γd =

b. γd =

c. γd =

d. γd =

19. A suspended particle falls through a height H cm in water in t minutes. If the viscosity of water is η and specific gravity of the particle is G, the diameter of the particle is (where M is a constant factor)

a. 103M

b. 104M

c. 105M

d. 102M

20. The shear strength in plastic undrained clay, is due to

a. inter-granular friction

b. internal friction

c. cohesion

d. none of these.

21. The allowable tensile stress in mild steel stirrups, reinforced cement concrete, is

a. 1400 kg/cm2

b. 190 kg/cm2

c. 260 kg/cm2

d. 230 kg/cm2

22. The minimum number of main steel bars provided in R.C.C.

a. rectangular columns is 4

b. circular columns is 6

c. octagonal columns is 8

d. all the above.

23. A foundation is called shallow if its depth, is

a. half of its width

b. three-fourth of its width

c. equal to its width

d. all the above.

24. Distribution reinforcement in a simply supported slab, is provided to distribute

a. load

b. temperature stress

c. shrinkage stress

d. all the above.

25. For a wall carrying heavy load on low bearing capacity soil,

a. thick concrete bed is provided

b. reinforced concrete bed is provided

c. (a) and (c) of the above

d. (b) and (c) of the above.

26. The pile provided with one or more bulles in its vertical shaft, is generally known as

a. under-ream pile

b. friction pile

c. bearing pile

d. sheet pile.

27. The final operation of finishing floors, is known as

a. floating

b. trowelling

c. finishing

d. all the above.

28. To prevent segregation, the maximum height for placing concrete, is

a. 100 cm

b. 125 cm

c. 150 cm

d. 200 cm

29. The maximum pressure to which a pipe is subjected to during its operation, is known

a. working pressure

b. design pressure

c. test pressure

d. pipe pressure

30. The chlorine supply cylinders are generally kept at 38°C to 40°C to prevent

a. conversion into crystals

b. it from burning

c. it from explosion

d. none of these.

31. No treatment of the sewage is given if dilution factor is

a. between 150 to 200

b. between 200 to 300

c. between 400 to 500

d. more than 500

32. A well oxidized sewage contains nitrogen mainly as

a. nitrates

b. nitrites

c. free ammonia

d. none of these.

33. Lacy's regime condition is obtained if

a. silt grade in the channel is variable

b. discharge in the channel is variable

c. silt charge in the channel is variable

d. channel flows in unlimited, incoherent alluvium of the same character as that transported material.

34. Effective precipitation for a crop may be defined as

a. total precipitation minus the loss due to evaporation

b. total precipitation minus the loss due to infiltration

c. total precipitation during the crop period

d. available water stored in soil within root zone of the crop.

35. Lane's weighted creep theory assumes

a. double weightage to horizontal creep and one weightage to vertical creep

b. triple weightage to horizontal creep and one weightage to vertical creep

c. triple weightage to vertical creep and one weightage to horizontal creep

d. double weightage to vertical creep and one weightage to horizontal creep.

36. Disposal of extra excavated earth of canals, is utilised to provide a spoil bank on

a. left side

b. right side

c. both sides

d. all the above.

37. The absolute minimum radius of horizontal curve for a design speed 60 km ph is

a. 131 m

b. 210 m

c. 360 m

d. none of these.

38. The minimum road width is taken

a. 9 m

b. 12 m

c. 16 m

d. 20 m

39. An ideal vertical curve is

a. cubic spiral

b. cubic parabala

c. lamniscate

d. none of these.

40. When a number of hair pin bends are introduced, a minimum intervening distance in between is kept

a. 20 m

b. 40 m

c. 60 m

d. 100 m

41. Pick up the correct statement from the following:

a. Area of canal in cutting = BD + Sd2 where B = bed width, d = depth of cutting and S is the side slope

b. Area of the bank of canal = B1d1 + Sd2 where B1, d1 and S are the width of bank, height of the bank above N.S.L. and side slope respectively

c. If F.S.L. is above N.S.L the canal is called partly in cutting and partly in filling and berms are designed as 2d where d is full supply depth

d. All the above.

42. The inspection pit or chamber is a manhole provided in a base drainage system

a. at every change of gradient

b. at every 30 m intervals

c. at the point where vertical soil pipe joins the house drain

d. All the above.

43. Pick up the correct statement from the following:

a. Plastering is measured in sq.m

b. Glazing is measured in sq.m

c. Striking is measured in sq.m

d. All the above.

44. The volume is measured correct to the nearest

a. 0.01 cum

b. 0.02 cum

c. 0.03 cum

d. 0.04 cum

45. Railway projects are treated as

a. light construction

b. heavy construction

c. industrial construction

d. none of these.

46. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

47. The time which results in the leasi, possible construction cost of an activity, is known

a. normal time

b. slow time

c. crash time

d. standard time.

48. While scheduling a project by C.P.M.

a. required time for each activity is established

b. sequence of various activities is made according to their importance

c. net work is drawn by connecting the activities and the events

d. All the above.

49. The meterological condition which influences the size and location of an air port is

a. air density

b. reduced level

c. wind direction

d. all the above.

50. The distance between main gears is 10 m, and the distance of nose gear from centre of main gears is 30 m. If the angle of turning is 60°, the distance of centre of rotation from the nearer main gear, is

a. 12.30 m

b. 11.30 m

c. 10.30 m

d. 9.30 m