***Sub Engineer***

1. Angles to a given pivot station observed from a number of traverse stations when plotted, the lines to the pivot station intersect at a common point

a. angular measurements are correct and not the linear measurements

b. linear measurements are correct and not the angular measurements

c. angular and linear measurements are correct and not the plotting of traverse

d. angular and linear measurements and also plotting of the traverse are correct.

2. The constant vertical distance between two adjacent contours, is called

a. horizontal equivalent

b. vertical equivalent

c. contour interval

d. contour gradient.

3. The sensitiveness of a level tube decreases if

a. radius of curvature of its inner surface is increased

b. diameter of the tube is increased

c. length of the vapour bubble is increased

d. both viscosity and surface tension are increased.

4. For the construction of highway (or railway)

a. longitudinal sections are required

b. cross sections are required

c. both longitudinal and cross sections are required

d. none of these.

5. Wrought iron contains carbon about

a. 1.5% to 5.5%

b. 0.5% to 1.75%

c. 0.1% to 0.25%

d. none to these.

6. Pick up the correct statement from the following:

a. The lime stone resists fire upto about 800°C and at higher temperature it splits into CaO and CO2

b. The sand stone with silicates resist a fire in a better way

c. The argillaceous stone though poor in strength can resist fire quite weak

d. All the above.

7. Sea sand used in structures causes

a. dampness

b. efflorescence

c. disintegration

d. All of these.

8. Varnish is a transparent or semi-transparent solution of resinuous substances in

a. alcohol

b. linseed

c. turpentine

d. all the above.

9. Pick up the correct statement from the following:

a. The structural member subjected to compression and whose dimensions are small as compared to its length, is called a stmt

b. The vertical compression members, are generally known as columns or stanchions

c. Deflection in lateral direction of a long column, is generally known as buckling

d. All the above.

10. Gradually applied static loads do not change with time their

a. magnitude

b. direction

c. point of application

d. all the above.

11. Rankine-Golden formula accounts for direct as well as buckling stress and is applicable to

a. long columns

b. short columns

c. intermediate columns

d. all the above.

12. The section modulus of a rectangular section is proportional to

a. area of the section

b. square of the area of the section

c. product of the area and depth

d. product of the area and width

13. The horizontal component of the force on a curved surface is equal to

a. weight of liquid vertically below the curved surface

b. force on a vetical projection of the curved surface

c. product of pressure at its centroid and the area

d. weight of liquid retained by the curved area.

14. The equation  for the viscosity, is suggested

a. Newton

b. Chezy

c. Bezin

d. Helmholtz.

15. Barometres are used to measure

a. difference in pressure at two points

b. atmospheric pressure

c. very low pressure

d. very high pressure.

16. The dimensions MLT-2 refers to

a. specific weight

b. force

c. discharge

d. none of these.

17. The ultimate bearing capacity of a soil, is

a. total load on the bearing area

b. safe load on the bearing area

c. load at which soil fails

d. load at which soil consolidates.

18. The plasticity of fine soils may be assessed by means of

a. dry strength test

b. toughness test

c. dilatancy test

d. all of these.

19. The ratio of the volume of voids to the total volume of the given soil mass, is known

a. porosity

b. specific gravity

c. void ratio

d. water content.

20. A fundamental equation of void ratio(e), specific gravity (G), water content (ω) and degree of saturation (Sr) is

a. e =

b. ω =

c. G =

d. Sr =

21. If C is creep coefficient, f is original prestress in concrete, m is modular ratio, E is Young's modulus of steel and e is shrinkage strain, the combined effect of creep and shrinkage is:

a. (1 - C)mf - eE

b. (C - 1)mf + eE

c. (C - 1)mf - eE

d. (1 - C)mf + eE

22. On piles, the drop must be at least

a. 100 cm

b. 120 cm

c. 140 cm

d. 150 cm

23. If W is weight of a retaining wall and P is the horizontal earth pressure, the factor of safety against sliding, is

a. 1.25

b. 1.5

c. 2.0

d. 2.5

24. A circular slab subjected to external loading, deflects to form a

a. semi-hemisphere

b. ellipsoid

c. parabolloid

d. none of these.

25. The angular steps used for changing direction of the stairs, are called

a. angular steps

b. winders

c. radial steps

d. circular steps.

26. The additional piles which are driven to increase the capacity of supporting loads on vertical piles, are known

a. raking piles

b. eccentric piles

c. sinking piles

d. none of these.

27. Segregation is responsible for

a. porous layers in concrete

b. surface scaling in concrete

c. sand streaks in concrete

d. all the above.

28. Curing of pavements, floors, roofs and slabs, is done by

a. ponding method

b. covering surface with bags

c. sprinkling water method

d. shading concrete method.

29. At the socket and spigot joint,

a. normal end of the pipe is called spigot

b. spigot is fitted into the socket

c. molten lead is used for water proofing

d. all the above.

30. The maximum pressure to which a pipe is subjected to during its operation, is known

a. working pressure

b. design pressure

c. test pressure

d. pipe pressure

31. Stoneware sewers are available in size

a. 15 cm

b. 20 cm

c. 25 cm

d. all the above.

32. The settling velocity of the particles larger than 0.06 mm in a settling tank of depth 2.4 is 0.33 m per sec. The detention period recommended for the tank, is

a. 1 hour

b. 1 hour and 30 minutes

c. 2 hours

d. 3 hours.

33. A fall which maintains its depth, is

a. a trapezoidal notch fall

b. a low weir fall

c. a rectangular notch fall

d. all the above.

34. Pick up the correct sequence of the part of a canal system from the following

a. Head work-distributary-branch canal-minor

b. Head works-main canal-branch canal-dis-tributary-minor

c. Head works-main canal-branch canal-minor-distributary

d. Heads works-branch canal-main canal distributary, minor.

35. The ratio of the rate of change of discharge of an outlet and parent channel, is known as

a. efficiency

b. sensitivity

c. flexibility

d. modular limit.

36. The main cause of silting up a channel,

a. inadequate slope

b. defective head regulator

c. defective outlets

d. all the above.

37. Alignment of highways in hilly regions, is decided on

a. number of river crossings

b. natural unstable areas

c. saddles or passes

d. all the above.

38. When a number of hair pin bends are introduced, a minimum intervening distance in between is kept

a. 20 m

b. 40 m

c. 60 m

d. 100 m

39. Transverse joints are provided at distances varying from

a. 12 m to 18 m

b. 16 m to 24 m

c. 17 m to 27 m

d. 25 m to 35 m

40. The wall constructed for the stability of an excavated portion of a road on the hill side, is known as

a. retaining wall

b. breast wall

c. parapet wall

d. all the above.

41. Pick up the incorrect statement regarding a master trap from the following :

a. It is provided a cleaning eye at the top of the trap

b. The height of fresh air inlet pipe fixed vertically with wall is 3 m

c. The mica flap valve which opens inwards only, is fitted at the top of the inlet pipe

d. The water seal is less than that of ordinary traps.

42. The excavation exceeding 1.5 m in width and 10 sq.m in plan area with a depth not exceeding 30 cm, is termed as

a. Excavation

b. Surface dressing

c. Cutting

d. Surface excavation.

43. The expected out turn for earth work in excavation in ordinary soil per mazdoor per day is

a. 1.00 cubic meter

b. 2.00 cubic meter

c. 3.00 cubic meter

d. 4.00 cubic meter

44. The brick work is not measured in cu m in case of

a. One or more than one brick wall

b. Brick work in arches

c. Reinforced brick work

d. Half brick wall.

45. Pick up the correct statement from the following:

a. The duration of time by which an activity can be delayed without affecting the succeeding activity, is called free float

b. The float which affects neither the processor nor the successor activities, is called independent float

c. The difference between total float and free float, is called interfering float

d. All the above.

46. CPM is

a. is built of activities oriented programme

b. is based on time estimate

c. is used for repetitive works

d. all the above.

47. Railway projects are treated as

a. light construction

b. heavy construction

c. industrial construction

d. none of these.

48. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

49. According to I.C.A.O. the slope of transitional surface at right angles to the centre line of runway, is kept

a. 1 in 5

b. 1 in 6

c. 1 in 7

d. 1 in 10

50. A gradient of + 0.08% is followed by a gradient of - 0.07%. If the permissible rate of change of grade is 0.003 per 30 metres, the length of the transition curve, is

a. 150 m

b. 140 m

c. 160 m

d. 175 m