***Sub Engineer***

1. One of the Lehmann's rules of plane tabling, is

a. location of the instrument station is always distant from each of the three rays from the known points in proportion to their distances

b. when looking in the direction of each of the given points, the instrument station will be on the right side of one and left side of the other ray

c. when the instrument station is outside the circumscribing circle its location is always on the opposite side of the ray to the most distant point as the inter-section of the other two rays

d. none of these.

2. Pick up the correct statement from the following :

a. if the image of the object does not fall on the plane of the cross-lines, parallax exists

b. parallax has nothing to do with the eyepiece

c. the eyepiece is adjusted for clear vision of the cross hairs

d. all the above.

3. The difference of level between a point below the plane of sight and one above, is the sum of two staff readings and an error would be produced equal to

a. the distance between the zero of gradient and the foot of the staff

b. twice the distance between the zero of graduation and the foot of the staff

c. thrice the distance between the zero of graduation and the foot of the staff

d. none of the above.

4. Pick up the correct statement from the following :

a. the lines of sight while observing back sight and fore  sight lie in the same horizontal plane

b. the   staff readings are measurements made vertically downwards from a horizontal plane

c. the horizontal plane with reference to which staff readings are taken, coincides with the level surface through the telescope axis

d. all the above.

5. Stones used for ornamental work must be

a. soft

b. hard

c. light

d. heavy.

6. Based on its dry weight, a freshly felled tree may contain water

a. 25%

b. 50%

c. 75%

d. 100%.

7. Chemically, marble is known as

a. metamorphic rock

b. argillaceous rock

c. calcareous rock

d. silicious rock.

8. Elastomers can extend upto

a. five times their original dimensions

b. seven times their original dimensions

c. ten times their original dimensions

d. three times their original dimensions.

9. The ratio of crippling loads of a column having both the ends fixed to the column having both the ends hinged, is

a. 1

b. 2

c. 3

d. 4

10. By applying the static equations i.e. ΣH = 0, ΣV = 0 and ΣM = 0, to a determinate structure, we may determine

a. shear forces only

b. bending moments only

c. internal forces only

d. all the above.

11. Strain energy of a member may be equated to

a. average resistance x displacement

b.  stress x strain x area of its cross-section

c.  stress x strain x volume of the member

d.  (stress)2 x volume of the member + Young's modulus E.

12. In a loaded beam, the point of con-traflexture occurs at a section where

a. bending moment is zero or changes sign

b. bending moment is maximum

c. shearing force is maximum

d. shearing force is minimum.

13. The discharge through a V-notch weir varies as

a. 1/H

b. H3/2

c. H7/2

d. H5/2.

14. A cylindrical vessel 40 cm high is revolved about its vertical axis so that the water touches the bottom when it just spills out. If the radius of the cylinder is 5 cm, the angular velocity of rotation, is

a. 2 rad/sec

b. 3 rad/sec

c. 4 rad/sec

d. 5 rad/sec

15. Water belongs to

a. Newtonian fluids

b. non-Newtonian fluids

c. compressible fluid

d. none of these.

16. For steady flow in open channels, which one of the following does not change :

a. depth of flow

b. velocity of flow

c. rate of flow

d. All of these.

17. Pick up the correct statement from the following:

a. The ratio of shear strength in natural state to the remoulded shear strength under undrained conditions, is called degree of sensitivity.

b. The difference between the undisturbed shear strength and remoulded shear strength is known remoulding loss

c. The tendency of dense sand to expand on application of shearing load, is known as dilatancy

d. All the above.

18. A soil sample of mass specific gravity 1.92, has a moisture content 30%. If the specific gravity of solids is 2.75, the void ratio, is

a. 0.858

b. 0.860

c. 0.862

d. 0.864

19. Pick up the correct statement from the following:

a. At low value of water content most soils tend to be stiff.

b. At high water content, the dry density decreases with an increase of water content.

c. An airless soil will have maximum possible value of dry density for the given water content

d. All the above.

20. Negative skin friction on piles

a. is caused due to relative settlement of the soil

b. is caused in soft clays

c. decreases the pile capacity

d. all of the above.

21. A short column 20 cm x 20 cm in section is reinforced with 4 bars whose area of cross section is 20 sq. cm. If permissible compressive stresses in concrete and steel are 40 kg/cm2 and 300 kg/cm2, the Safe load on the column, should not exceed

a. 4120 kg

b. 41, 200 kg

c. 412, 000 kg

d. none of these.

22. On piles, the drop must be at least

a. 100 cm

b. 120 cm

c. 140 cm

d. 150 cm

23. The spacing of transverse reinforcement of column is decided by the following consideration.

a. The least lateral dimension of the column

b. Sixteen times the diameter of the smallest longitudinal reinforcing rods in the column

c. Forty-eight times the diameter of transverse reinforcement

d. All the above.

24. The maximum shear stress (qmax) in a rectangular beam is

a. 1.50 times the average

b. 1.75 times the average

c. 2.0 times the average

d. 2.5 times the average.

25. The foundation which consists of a thick reinforced cement slab covering whole area to support heavy concentrated structural loads, is known as

a. combined footing

b. strap footing

c. raft footing

d. none of these.

26. The opening provided in sloping roof with its top parallel to the roof surface, is called

a. dormer window

b. sky light window

c. lantern window

d. louvered window.

27. Pick up the correct statement from the following:

a. Cement should be mixed for at least one minute

b. 10% of water is placed in the rotating drum before adding dry material

c. 10% of water is added after placing the other ingredients in the drum

d. All the above.

28. To obtain cement dry powder, lime stones and shales or their slurry, is burnt in a rotary kiln at a temperature between

a. 1200° and 1300°C

b. 1300° and 1400°C

c. 1400° and 1500°C

d. 1500° and 1600°C

29. Pick up the correct statement from the following :

a. At zero discharge, discharge valve remaining closed, the head developed is maximum which is known as shut-off head

b. At a particular speed the discharge at which efficiency of a pump is maximum, is known as normal discharge

c. The brake horse power required from the primemover to drive the pump, increases with the increase of discharge

d. all the above.

30. 45 litres of water per person per day, is provided in

a. office buildings

b. hotels

c. hostels

d. nurse's homes.

31. In primary sedimentation, the 0.2 mm inorganic solids get separated if specific gravity is

a. 2.50

b. 2.55

c. 2.65

d. 2.75

32. No treatment of the sewage is given if dilution factor is

a. between 150 to 200

b. between 200 to 300

c. between 400 to 500

d. more than 500

33. When a canal flowing under pressure is carried below a natural drainage such that its F.S.L. does not touch the underside of the supporting structure, the structure so provided, is called

a. syphon

b. aqueduct

c. super passage

d. syphon-aqueduct.

34. The level of the fioor of a syphon aqueduct can be obtained

a. by subtracting the depth of the culvert from the canal bed level

b. by subtracting the thickness of culvert plus the depth of the culvert from the canal bed level

c. both (a) and (b)

d. none of these.

35. Regime conditions in a channel may occur if

a. discharge is constant

b. channel flows uniformly in incoherent alluvium as that transported in suspension

c. silt grade and silt charge are constant

d. all the above.

36. A counter berm is

a. a horizontal benching provided on the inside slope

b. a horizontal benching provided on the outside slope

c. a vertical benching provided on the outer edge of the bank

d. a vertical benching provided on the inner edge of the bank.

37. The normal road land width for a National or State highway, in open areas should be

a. 45 m

b. 30 m

c. 24 m

d. 20 m

38. Traffic engineering only includes

a. design and application of control devices

b. analysis of traffic characteristics

c. traffic opeation

d. all the above.

39. The inventor of road making as a building science, was

a. Sully

b. Tresguet

c. Telford

d. Macadam.

40. If the rate of change of the super-elevation along a curved portion of a 7 metre wide road is 1 in 150 and the maximum super-elevation allowed is 1 in 15, the maximum length of the transition curve to be provided at either end, is

a. 70 m

b. 75 m

c. 80 m

d. 90 m

41. The main factor to be considered while preparing a detailed estimate, is

a. Availability of materials

b. Transportation of materials

c. Location of site and local labour charges

d. All the above.

42. The minimum width of a septic tank is taken

a. 70 cm

b. 75 cm

c. 80 cm

d. 90 cm

43. The following item of earth work is not measured separately.

a. Site clearence

b. dead men

c. Steps in deep exavaction

d. All the above.

44. The measurement is made for stone work in square metre in case of

a. Columns, lintels, copings

b. Building work

c. Dressed stones in Chajjas

d. (a) and (d) of the above.

45. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

46. Modular co-ordination of construction means proper

a. planning

b. designing

c. execution

d. all the above.

47. Military organisation is known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. none of these.

48. Which one of the following represents an event ?

a. concrete cured

b. fixing of door

c. plastering of walls

d. selecting sites

49. For the proposed air port, the survey project provides

a. topograhic plan

b. grading plan

c. drainage plan

d. all the above.

50. According to the International Civil Aviation Organisation (I.C.A.O.), the runway lengths of aerodromes, have been coded by

a. Seven English alphabets

b. Last Seven English alphabets

c. First Seven English alphabets

d. First seven natural numbers.