***Sub Engineer***

1. You have to observe an included angle with better accuracy than what is achievable by a vernier, you will prefer the method of

a. repetition

b. reiteration

c. double observations

d. exactness.

2. Cross hairs in surveying telescopes, are fitted

a. in the objective glass

b. at the centre of the telescope

c. at the optical centre of the eye piece

d. in front of the eye piece.

3. The operation of making the algebraic sum of latitudes and departures of a closed traverse, each equal to zero, is known

a. balancing the sights

b. balancing the departures

c. balancing the latitudes

d. balancing the traverse.

4. The slope correction for a 3° slope for a length of 100 m, is

a. -0.11 m

b. -0.12 m

c. -1.87 m

d. -0.137 m

5. Cast iron is used for

a. structural works in beams

b. small sized water pipes

c. columns and struts

d. none to these.

6. Plastic

a. consists of natural or synthetic binders

b. finished products are rigid and stable at normal temperature

c. is capable of flow when necessary heat and pressure are applied

d. All the above.

7. The steel used for the manufacture of rails, is

a. Bessemer steel

b. mild steel

c. cast steel

d. stainless steel.

8. Stainless steel contains

a. 18% of chromuim and 8% nickel

b. 8% of chromium and 18% of nickel

c. 12% of chromium and 36% of nickel

d. 36% of chromium and 12% of nickel.

9. The tangential component of stress on an plane inclined θ° to the direction of the force, may be obtained by multiplying the normal stress by

a. cos θ

b. tan θ

c. sin θ cos θ

d. sin2θ

10. The locus of the moment of inertia about inclined axes to the principal axis, is

a. straight line

b. parabola

c. circle

d. ellipse.

11. For a simply supported beam of length L, the bending moment M is described as M = a (x - x3/L2), 0 ≤ x < L; where a is a constant. The shear force will be zero at

a. the supports

b. x = L/2

c. x = L/3

d. x = L/3

12. The reaction at support A of the beam shown in below figure, is

a. zero

b. 5 T

c. 10 T

d. 1 T

13. If velocities of fluid particles vary from point to point in magnitude and direction, as well as from instant to instant, the flow is said to be

a. laminar

b. turbulent flow

c. uniform flow

d. non-uniform flow.

14. The shape of fire hose nozzle is generally kept

a. divergent

b. convergent

c. convergent divergent

d. cylindrical.

15. In flow, the liquid particles may possess

a. potential energy

b. kinetic energy

c. pressure energy

d. all the above.

16. For a most economical rectangular channel, the width of the channel must be

a. equal to depth of flow

b. twice the depth of flow

c. half the depth of flow

d. None of these.

17. A soil sample of mass specific gravity 1.92, has a moisture content 30%. If the specific gravity of solids is 2.75, the void ratio, is

a. 0.858

b. 0.860

c. 0.862

d. 0.864

18. Pick up the correct statement from the following:

a. The dry density reduces by addition of water after attaining optimum moisture content

b. The line joining the peak of three moisture content graphs obtained by using three compactive energies, is called line of optimus

c. Well graded coarse grained soils can be compacted to a very high density as compared to fine grained soils

d. All the above. .

19. During seepage through a soil, direction of seepage is always

a. parallel to equipotential lines

b. perpendicular to stream lines

c. perpendicular to equipotential lines

d. none of these.

20. Pick up the correct statement from the following:

a. The line which shows moisture content dry density relation for soil containing a constant percentage of air voids, is known as air void line

b. The weight of hammer used for compaction test is 25 kg

c. The free fall of hammer for compaction is 30.5 cm

d. All the above.

21. Design of a two way slab simply supported on edges and having no provision to prevent the corners from lifting, is made by

a. Marcus formula

b. Rankine Grashoff formula

c. Grashoff formula

d. Rankine-Marcus formula.

22. The maximum ratio of span to depth of a slab simply supported and spanning in two directions, is

a. 30

b. 35

c. 40

d. 15

23. The shear reinforcement in R.C.C. is provided to resist

a. vertical shear

b. horizontal shear

c. diagonal compression

d. diagonal tension.

24. An R.C.C beam of 25 cm width has a clear span of 5 metres and carries a U.D.L. of 2000 kg/m inclusive of its self weight. If the lever arm of the section is 45 cm., the beam is

a. safe in shear

b. is safe with stirrups

c. is safe with stirrups and inclined members

d. needs revision of the section.

25. If a is the offset of concrete bed in cms, and d is the depth of concrete bed in cms, then

a. d = 0.445 a

b. 0.557 a

c. d = 0.775 a

d. none of these.

26. In grillage foundations, distance between flanges of grillage beams, is kept

a. 40 cm

b. equal to flange width

c. twice the flange width

d. maximum of (a), (b) and (c).

27. If a grading curve is horizontal bet-wen the portions of 20 mm I.S. Sieve and 4.75 mm I.S. Sieve, the graded aggregates do not contain

a. 20 mm particles

b. 10 mm particles

c. 4.75 mm particles

d. all the above.

28. Gypsum is added for

a. colour

b. strength

c. controlling setting time

d. none of these.

29. Pick up the incorrect statement from the following :

a. Porosity of pure sand is 35%

b. Porosity of sand stone is up to 15%

c. Least porosity is that of granite and quartz

d. None of these.

30. Well treated water is generally supplied for

a. commercial use

b. public use

c. waste water

d. all the above.

31. In septic tanks,

a. the baffles or tees are extended up to top level of scum

b. the clear space between the baffle top and covering slab is about 7.5 cm

c. the inlet is kept 30 cm below the sewage line and outlet is kept at a level 40% depth of the sewage

d. all the above.

32. The sewerage system consists of

a. lateral sewer

b. branch sewer

c. main sewer

d. all of these.

33. The ratio of the rate of change of discharge of an outlet and parent channel, is known as

a. efficiency

b. sensitivity

c. flexibility

d. modular limit.

34. Water-shed line is abondened for aligning an irrigation canal if

a. water shed forms a sharp loop

b. canal has to take off from a river

c. towns and villages are located on the water shed line

d. all the above.

35. For the stability of a structure against seepage pressure according to Khosla's creep theory, the critical gradient is

a. 0.25

b. 0.50

c. 0.75

d. 1.00

36. Groynes are generally built

a. perpendicular to the bank

b. inclined up stream up to 30°

c. inclined down stream upto 30°

d. all the above.

37. For a properly designed vehicle, the resistance generally ignored, is

a. rolling resistance

b. grade resistance

c. axle resistance

d. none of these.

38. Pick up the correct statement from the following:

a. Borrow pits are located out side the right of way

b. Borrow pits may be located on either side of the right of way

c. Spoil bank is located on one side of the right of way

d. All the above.

39. Pick up the incorrect statement from the following. On highways circular curves may be

a. compound curves

b. reverse curves

c. vertical curves

d. all the above.

40. Thickness of broken line markings on multi-lane road for lanes is generally kept

a. 10 cm

b. 12 cm

c. 15 cm

d. 18 cm

41. The total length of a cranked bar through a distance (d) at 45° in case of a beam of effective length L, is

a. L + 0.42 d

b. L + 2 x 0.42 d

c. L - 0.42 d

d. L - 2 x 0.4 d

42. The measurement is made for stone work in square metre in case of

a. Columns, lintels, copings

b. Building work

c. Dressed stones in Chajjas

d. (a) and (d) of the above.

43. The measurement is made in square metre in case of

a. R.C.C. structure

b. Hollow concrete block wall

c. Concrete fencing posts

d. None of these.

44. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

45. An Executive Engineer may have powers upto

a. Rs. 25,000

b. Rs. 50,000

c. Rs. 100,000

d. Rs. 200,000

46. Modular co-ordination of construction means proper

a. planning

b. designing

c. execution

d. all the above.

47. The estimated time required to perform an activity, is known as

a. event

b. dummy

c. duration

d. float.

48. Bar charts are suitable for

a. minor works

b. major works

c. large projects

d. all the Above.

49. The strength of winds is measured with the help of

a. Benfort scale

b. Wind indicator

c. Barometres

d. None of these.

50. The reduced level of the proposed site of an air port is 2500 m above M.S.L. If the recommended length by I.C.A.O. for the runway at sea level is 2500 m, the required length of the runway is

a. 2500 m

b. 3725 m

c. 3000 m

d. 3250 m