***Sub Engineer***

1. The orthographical projection of a traverse leg upon the reference meridian, is known as

a. departure of leg

b. latitude to the leg

c. co-ordinate of the leg

d. bearing of the leg.

2. Subtense tacheometry is generally preferred to if ground is

a. flat

b. undulating

c. mountaineous

d. deserts.

3. The total change in level along the line is equal to total back sights

a. minus total fore sights

b. the total rises minus total falls

c. the reduced level of last point minus reduced level of the first point

d. all the above.

4. Pick up the correct statement from the following :

a. the handles are on swivel joints to prevent twisting of the chain

b. the fifth tag from either end of the chain is numbered 5

c. the length of the Gunter's chain is 66 ft.

d. all the above.

5. Acrylic is the name of

a. cellulose resin

b. alkyd resin

c. methyl metha crylate

d. cumarone-indene.

6. Pick up the correct statement from the following:

a. Quick lime is obtained by burning pure lime stone:

b. Hydraulic lime is obtained by burning lime stone containing clay 5% to 30%

c. Poor lime is obtained by burning lime stone containing impurities more than 5%

d. All the above.

7. Whitworth compressed steel is obtained when molten steel is subjected to a pressure of

a. 9 kg/mm2

b. 13 kg/mm2

c. 15 kg/mm2

d. 10 kg/mm2.

8. Slacking of lime is affected by

a. keeping it exposed to air

b. immersing the lime in water

c. crushing the lime lumps

d. none of these.

9. Pick up the incorrect statement from the following : The torsional resistance of a shaft is directly proportional to

a. modulus of rigidity

b. angle of twist

c. reciprocal of the length of the shaft

d. moment of inertia of the shaft section.

10. The greatest load which a spring can carry without getting permanently distorted, is called

a. stiffness

b. prof resilience

c. proof stress

d. proof load.

11. A solid cube is subjected to equal normal forces on all its faces. The volumetric strain will be x-times the linear strain in any of the three axes when

a. x = 1

b. x = 2

c. x = 3

d. x = 4

12. Pick up the correct statement from the following :

a. In the standard rolled channels, the shear centre is on the horizontal line passing through and away from the C.G. beyond web

b. In equal angles, the shear centre is on the horizontal plane and away from the C.G., outside of the leg projection

c. In T-sections, the shear centre is at the C.G. of the section

d. All the above.

13. For a most economical trapezoidal open channel, the half of the top width must be equal to

a. the bed width

b. one sloping side

c. the depth of flow

d. None of these.

14. Mercury is generally used in barometers because

a. the height of the barometer will be less

b. it is a best liquid

c. both (a) and (b) above

d. both (b) and (c) above.

15. The discharg formula Q = Cd 2gH x A is used for rectangular

a. small orifices only

b. large orifices only

c. small and large orifices only

d. for all types of orifices.

16. Water flows through a convergent mouthpiece of diameter 4 cm at convergence under a head of 3 metres. If the maximum vacuum pressure is 9 metres of water, the maximum diameter of divergence, to aviod separation of flow, is

a. 4 cm

b. 6 cm

c. 2 cm

d. 23 cm.

17. When drainage is permitted under initially applied normal stress only and full primarily consolidation is allowed to take place, the test is known as

a. quick test

b. drained test

c. consolidated undrained test

d. none of these.

18. Pick up the correct statement from the following:

a. The ratio of shear strength in natural state to the remoulded shear strength under undrained conditions, is called degree of sensitivity.

b. The difference between the undisturbed shear strength and remoulded shear strength is known remoulding loss

c. The tendency of dense sand to expand on application of shearing load, is known as dilatancy

d. All the above.

19. Chemical weathering of soil is caused due to

a. carbonation

b. hydration

c. leaching

d. all the above.

20. The ratio of volume of air voids to the volume of total voids, is known as

a. air content

b. percentage air voids

c. percentage voids

d. porosity.

21. Top bars are extended to the projecting parts of the combined footing of two columns L distance apart for a distance of

a. 0.1 L from the outer edge of column

b. 0.1 L from the centre edge of column

c. half the distance of projection

d. one-fourth the distance of projection.

22. Side face reinforcement shall be provided in the beam when depth of the web in a beam exceeds

a. 50 cm

b. 75 cm

c. 100 cm

d. 120 cm

23. High strength concrete is used in prestressed member

a. to ovecome bursting stresses at the ends

b. to provide high bond stresses

c. to overcome cracks due to shrinkage

d. all the above.

24. If the sides of a slab simply supported on edges and spanning in two directions are equal, the maximum bending moment is multiplied by

a. 0.3

b. 0.4

c. 0.5

d. 0.7

25. The vertical member running through middle of a shutter frame, is

a. style

b. reveal

c. mullion

d. post.

26. The Auger borings are not common

a. in soils that require lateral support

b. in cohesive soils

c. in soft soils

d. none of the above.

27. Higher workability of concrete is required if the structure is

a. made with cement concrete

b. thick and reinfored

c. thin and heavily reinforced

d. thick and heavily reinfored.

28. The size of fine aggregates does not exceed

a. 3.00 mm

b. 3.75 mm

c. 4.75 mm

d. 5.75 mm

29. Perched acquifers generally occur

a. above water table

b. in acquicludes

c. in artesian acquifers

d. none of these.

30. If Q is discharge is cubicmetres per sec and D is the economical diameter of the pipe. According to Lea

a. D = 0.67 to 0.87 Q

b. D = 0.77 to 0.97 Q

c. D = 0.97 to 1.22 Q

d. D = 1.22 to 1.33 Q.

31. Lead acetate test in sewer manhole is done to test the presence of

a. Methane gas

b. Hydrogen sulphide

c. Carbondioxide gas

d. Diesel vapours.

32. The asbestos cement sewers are

a. light in weight

b. not structurally strong to bear large compressive stress

c. susceptible to corrosion by sulphuric acid

d. All the above.

33. The structure constructed to allow drainage water to flow under pressure through an inverted syphon below a canal, is called

a. super passage

b. aqueduct

c. super-aqueduct

d. syphon aqueduct.

34. If the straight sides of a triangular section of a lined canal with circular bottom of radius R, make an angle θ with horizontal, the perimeter of the canal is

a. 2R(θ + tan θ)

b. R(θ + cos θ)

c. 2R(θ + cos θ)

d. none of these.

35. Lacy's regime condition is obtained if

a. silt grade in the channel is variable

b. discharge in the channel is variable

c. silt charge in the channel is variable

d. channel flows in unlimited, incoherent alluvium of the same character as that transported material.

36. The level of the fioor of a syphon aqueduct can be obtained

a. by subtracting the depth of the culvert from the canal bed level

b. by subtracting the thickness of culvert plus the depth of the culvert from the canal bed level

c. both (a) and (b)

d. none of these.

37. An ideal vertical curve is

a. cubic spiral

b. cubic parabala

c. lamniscate

d. none of these.

38. The ratio of maximum deviation angle and maximum polar deflection angle of a Lemniscate curve, is

a. 3

b. 4

c. 5

d. 6

39. Maximum number of passenger cars that can pass a given point on a road during one hour under the most ideal road way and traffic conditions, is known as

a. traffic density

b. basic capacity of traffic lane

c. possible capacity of traffic lane

d. all the above.

40. Width of vehicles affects the width of

a. lanes

b. shoulders

c. parking spaces

d. all the above.

41. The weight of an item is measured correct to nearest

a. 0.50 kg

b. 0.75 kg

c. 1.00 kg

d. 5 kg

42. The concrete work for the following part of the building of specified thickness is measured in square metres

a. Floors

b. D.P.C.

c. Wall panels

d. All the above.

43. The unit of measurement is per quintal for the following:

a. Collapsible gates with rails

b. Rolling shutters

c. Expanded metal wire netting

d. M.S. reinforcement of R.C.C. works.

44. While preparing a detailed estimate

a. Dimension should be measured correct to 0.01 m

b. Area should be measured correct to 0.01 sqm

c. Volume should be measured correct to O.Olcum

d. All the above.

45. Works costing less than Rs. 20,000 are treated as

a. projects

b. major projects

c. minor projects

d. all the above.

46. Pick up the incorrect statement from the following:

a. The tail of the arrow indicates the start of the activity

b. The head of the arrow indicates the end of the activity

c. The arrows are drawn to scale from left to right

d. Each activity consumes a given time.

47. Pick up the correct statement from the following:

a. The difference of latest occurence time and earliest expected time, is called slack

b. The activities connecting the events having zero slack, lie on the critical path

c. The critical path consumes the maximum time

d. All the above.

48. In CPM analysis,

a. uncertainties are not allowed

b. activities are represented by arrows

c. beginning and end of an activity, are denoted by nodes

d. all the above.

49. The height of the pilot's eye above the runway surface is assumed

a. 1 m

b. 3 m

c. 4 m

d. 5 m

50. Pick up the correct statement from the following:

a. The speed of the aircraft relative to the ground, is called cruising speed

b. The speed of the aircraft relative to wind, is called air speed

c. When wind is blowing the direction of the flight, air speed is less than cruising speed

d. All the above.