***Sub Engineer***

1. The angle between two plane mirrors of optical square, is

a. 30°

b. 45°

c. 60°

d. 90°.

2. The tangent to the liquid surface in a level tube, is parallel to the axis of the level tube at

a. every point of the bubble

b. either end of the bubble

c. the mid-point of the bubble

d. no where.

3. Subtense tacheometry is generally preferred to if ground is

a. flat

b. undulating

c. mountaineous

d. deserts.

4. Pick up the correct statement from the following :

a. mistakes arise from inattention, inexperience or carelessness

b. systematic errors persist and have regular effects in the survey performances

c. accidental errors occur inspite of every precaution is taken

d. all the above.

5. The weight of a good quality brick when immersed in water for a period of 16 hours should not exceed the weight of dry brick

a. 20%

b. 15%

c. 10%

d. None of these.

6. The most important constituent of an oil paint, is

a. vehicle

b. pigment

c. base

d. all the above.

7. A bull nose brick is not used for

a. rounding off sharp corners

b. pillars

c. decoration purpose

d. arches.

8. Expanded metal is

a. manufactured from steel sheets

b. used for reinforced concrete in road pavements

c. measured in term of SWM (shortway mesh) and LWM (long way mesh)

d. all the above.

9. The shape factor of standard rolled beam section varies from

a. 1.10 to 1.20

b. 1.20 to 1.30

c. 1.30 to 1.40

d. 1.40 to 1.50

10. The ratio of crippling loads of a column having both the ends fixed to the column having both the ends hinged, is

a. 1

b. 2

c. 3

d. 4

11. If a rectangular beam measuring 10 x 18 x 400 cm carries a unformly distributed load such that the bending stress developed is 100 kg/cm2. The intensity of the load per metre length, is

a. 250 kg

b. 260 kg

c. 270 kg

d. 280 kg.

12. A simply supported wooden beam 150 cm long and having a cross section 16 cm x 24 cm carries a concentrated load, at the centre. If the permissible stress ft = 75 kg/cm2 and fs = 10 kg/cm2 the safe load is

a. 3025 kg

b. 3050 kg

c. 3075 kg

d. 3100 kg.

13. The flow in open channel is said to be subcritical if the Froude number is

a. less than 1.0

b. equal to 1.0

c. greater than 1.0

d. none.

14. To avoid the tendency of separation of liquid flow, the most suitable ratio of the diameters of the throat and the pipe, is

a. 1/4 to 1/8

b. 1/3 to 1/2

c. 1/2 to 3/4.

d. none of these.

15. A nozzle is fitted at the end of a pipe whose length is 320 m and diameter is 10 cm. If the value of f = 0.01, the diameter of the nozzle for the maximum transmission of power through the nozzle is

a. 2.4 cm

b. 2.5 cm

c. 2.6 cm

d. 2.7 cm.

16. The momentum correction factor (β) for the viscous flow through a circular pipe is

a. 1.33

b. 1.50

c. 1.66

d. 2.00

17. The intensity of active earth pressure at a depth of 10 metres in dry cohesionless sand with an angle of internal friction of 30° and with a weight of 1.8 t/m3, is

a. 5 t/m2

b. 6 t/m2

c. 7 t/m2

d. 8 t/m2

18. Pick up the correct statement from the following:

a. The freezing of water is accompained by a volume increase of 9%.

b. Below freezing point, higher soil suction develops.

c. The magnitude of frost heave decreases as the degree of saturation of soil decreases

d. All the above.

19. The consolidation time for soils

a. decreases with increasing permeability

b. increases rapidly with increasing size of soil mass

c. is independent of the magnitude of the stress change.

d. All the above.

20. In a liquid limit test, the moisture content at 10 blows was 70% and that at 100 blows was 20%. The liquid limit of the soil, is

a. 35%

b. 50%

c. 65%

d. none of these.

21. Pick up the correct statement from the following:

a. A pile is a slender member which transfers the load through its lower end on a strong strata

b. A pile is a slender member which transfers its load to the surrounding soil

c. A pile is a slender member which transfers its load by friction

d. A pile is a cylindrical body of concrete which transfers the load at a depth greater than its width.

22. The stem of a cantilever retaining wall which retains earth level with top is 6 m. If the angle of repose and weight of the soil per cubic metre are 30° and 2000 kg respectively, the effective width of the stem at the bottom, is

a. 52.5

b. 53.5

c. 54.5

d. 55.5

23. Dimensions of a beam need be changed if the shear stress is more than

a. 10 kg/cm2

b. 15 kg/cm2

c. 20 kg/cm2

d. 25 kg/cm2

24. If W is weight of a retaining wall and P is the horizontal earth pressure, the factor of safety against sliding, is

a. 1.25

b. 1.5

c. 2.0

d. 2.5

25. For different layers of cement concrete floor. Pick up the incorrect statement from the following :

a. The lowest layer consists of consolidated ground

b. A 10 cm thick clean sand is laid on consolidated ground

c. A 10 cm lime concrete (1 : 4 : 8) is laid on clean sand

d. A 10 cm thick cement concrete (1 : 2 : 4) is laid on top layer.

26. The members which support covering material of a sloping roof, are

a. rafters

b. purlins

c. battens

d. struts.

27. For the construction of cement con-certe dams, the maximum permissible size of the aggregates, is

a. 40 mm

b. 50 mm

c. 60 mm

d. 70 mm

28. Addition of pozzolana to cement causes

a. loss of heat of hydration

b. reduction in bleeding

c. increase in curing time

d. all the above.

29. For a city developed haphazardly, the layout of distribution pipes preferred to, is

a. ring system

b. radial system

c. grid iron system

d. dead end system.

30. Standard unit of turbidity of water is in one litre of distilled water, one milligram of finely divided

a. silica

b. mud

c. clay

d. organic matter.

31. Pick up the correct statement from the following :

a. Indore method of composting involves decomposition under aerobic conditions

b. Bangalore method of composting involves decomposing under anaerobic conditions

c. Fully stabilised refuse by the Bangalore method of compositing is a powdery mass called humus

d. all of these.

32. If D.O. concentration falls down to zero in any natural drainage, it indicates the zone of

a. active decomposition

b. recovery

c. cleaner water

d. none of these.

33. Lacy's regime condition is obtained if

a. silt grade in the channel is variable

b. discharge in the channel is variable

c. silt charge in the channel is variable

d. channel flows in unlimited, incoherent alluvium of the same character as that transported material.

34. Water-shed line is abondened for aligning an irrigation canal if

a. water shed forms a sharp loop

b. canal has to take off from a river

c. towns and villages are located on the water shed line

d. all the above.

35. If the optimum depth of kor watering for a crop is 15.12 cm, the outlet factor for the crop for four week period in hectares per cumec, is

a. 1200

b. 1400

c. 1600

d. 2000

36. If the straight sides of a triangular section of a lined canal with circular bottom of radius R, make an angle θ with horizontal, the perimeter of the canal is

a. 2R(θ + tan θ)

b. R(θ + cos θ)

c. 2R(θ + cos θ)

d. none of these.

37. Pavement is said to be flexible if it contains

a. stabilised soil base constructed of lime cement or tar

b. bitumen-bound stone layer of varying aggregates

c. lean concrete base

d. all the above.

38. The basic formula for determination of pavement thickness was first suggested by

a. Spanglar

b. Picket

c. Kelly

d. Goldbeck

39. Volume of traffic which would immediately use a new road or an improved one when opened to traffic, is known

a. development traffic

b. current traffic

c. general traffic

d. normal traffic growth.

40. The traffic carrying capacity of a single lane, depends on

a. level crossings

b. road intersections

c. surface texture

d. all the above.

41. Due to change in price level, a revised estimate is prepared if the sanctioned estimate exceeds

a. 2.0%

b. 2.5%

c. 4.0%

d. 5.0%

42. Pick up the correct statement from the following:

a. The bent up bars at a support resist the negative bending moment

b. The bent up bars at a support resist the sharing force

c. The bending of bars near supports is generally at 45°

d. All the above.

43. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

44. Size, capacity and materials need be specified for

a. Stop-cocks

b. Ferrules

c. Ball valves

d. All the above.

45. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

46. Mile Stone charts were invented in the year of

a. 1920

b. 1930

c. 1940

d. 1950

47. The Overall in-charge of an organisation at the site responsible for the execution of the works, is

a. Engineer

b. Junior Engineer

c. Sub overseer

d. Assistant Engineer.

48. Frederick W. Taylor introduced a system of working known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

49. The length of a runway under standard atmospheric conditions is 1800 m. If the actual reduced level of the site is 1200 m, the design length of the runway is

a. 2360 m

b. 2460 m

c. 2560 m

d. 2660 m

50. Airport elevation is the reduced level above M.S.L. of

a. control tower

b. highest point of the landing area

c. lowest point of the landing area

d. none of these.