***Sub Engineer***

1. The zero of the graduated circle of a prismatic compass is located at

a. north end

b. east end

c. south end

d. west end.

2. In case of a double line river, contours are

a. stopped at the banks of the river

b. stopped at the edge of the river

c. drawn across the water

d. drawn by parabolic curves having their vertex at the centre of the water.

3. Staff readings on pegs x and y from X station are 1.755 m and 2.850 m, and from station Y on staff head at Y and X are 0.655 m and 1.560 m. If reduced level of X is 105.5 m, the reduced level of Y is

a. 104.0 m

b. 104.5 m

c. 105.0 m

d. 105.5 m

4. If whole circle bearing of a line is 120°, its reduced bearing is

a. S 20° E

b. S 60° E

c. N 120° E

d. N 60° E.

5. Pick up the correct statement from the following:

a. Air bubbles in casting produce a dull sound by tapping their surfaces lightly with a hammer

b. Cupola furnace is used for the manufacture of cast iron

c. Red short iron is of no value for welding purpose

d. All the above.

6. Brass is an alloy of

a. copper and zinc

b. zinc and lead

c. tin and silver

d. zinc and nickel

7. A bull nose brick is not used for

a. rounding off sharp corners

b. pillars

c. decoration purpose

d. arches.

8. Pick up the incorrect statement from the following:

a. Plastics are chemical resistant

b. Plastics are durable

c. Plastics are ductile

d. Plastics are excellent electric insulators.

9. A body is said to be in equilibrium if

a. it moves horizontally

b. it moves vertically

c. it rotates about its C.G.

d. none of these.

10. Keeping the depth d constant, the width of a cantilever of length l of uniform strength loaded with a uniformly distributed load w varies from zero at the free end and

a.  at the fixed end

b.  at the fixed end

c.  at the fixed end

d.  at the fixed end

11. Failure of riverted joints is due to

a. Tearing of plates between rivets

b. Shearing of rivets

c. Crushing of rivets

d. All the above.

12. At either end of a plane frame, maximum number of possible transverse shear forces, are

a. one

b. two

c. three

d. four

13. Most economical section of a circular channel for maximum velocity, is if,

a. depth of water = 0.810 diameter

b. hydraulic mean depth = 0.304 diameter

c. wetted perimeter = 2.245 diameters

d. all the above.

14. The magnitude of water hammer in a pipe depends upon

a. length of the pipe line

b. elastic properties of the pipe material

c. elastic properties of the following liquid

d. all the above.

15. When a body is totally or partially immersed in a fluid, it is buoyed up by a force equal to

a. weight of the fluid displaced by the body

b. weight of the body and fluild displaced by the body

c. difference of weights of the fluid displaced and that of the body

d. none of these.

16. Weber number is the ratio of inertia force to

a. surface tension

b. gravitational force

c. elasticity

d. viscosity.

17. Pick up the incorrect statement from the following:

a. Below the water table, the pore water may be static.

b. The hydrostatic pressure depends on the depth below the water level.

c. The attractive forces between the particles, caused due to negative pressure of water held above the water table is called soil suction.

d. None of these.

18. A failure wedge develops if a retaining wall

a. moves away from the backfill

b. moves towards the backfill

c. sinks downwards

d. stresses equally by vertical and horizontal forces.

19. Through a point in a loaded soil, the principal stress is maximum on

a. minor principal plane

b. intermediate principal plane

c. major principal plane

d. none of these.

20. When the seepage pressure becomes equal to the pressure due to submerged weight of a soil, the effective pressure is reduced to zero and the soil particles have a tendency to move up in the direction of flow. This phenomenon is generally known

a. quick condition

b. boiling condition

c. quick sand

d. all the above.

21. According to I.S. : 456 specifications, the safe diagonal tensile stress for M 150 grade concrete, is

a. 5 kg/cm2

b. 10 kg/cm2

c. 15 kg/cm2

d. 20 kg/cm2

22. An R.C.C. beam of 25 cm width and 50 cm effective depth has a clear span of 6 metres and carries a U.D.L. of 3000 kg/m inclusive of its self weight. If the lever arm constant for the section is 0.865, the maximum intensity of shear stress, is

a. 8.3 kg/cm2

b. 7.6 kg/cm2

c. 21.5 kg/cm2

d. 11.4 kg/cm2

23. If the average bending stress is 6 kg/cm2 for M 150 grade concrete, the length of embedment of a bar of diameter d according to I.S. 456 specifications, is

a. 38 d

b. 48 d

c. 58 d

d. 95 d

24. The allowable tensile stress in mild steel stirrups, reinforced cement concrete, is

a. 1400 kg/cm2

b. 190 kg/cm2

c. 260 kg/cm2

d. 230 kg/cm2

25. For a rectangular foundation of width b, eccentricity of load should not exceed

a. b/3

b. b/4

c. b/5

d. b/6.

26. Pile foundations are suitable for

a. water logged soils

b. soft rocks

c. compact soils

d. multistoreyed buildings

27. Pick up the incorrect statement from the following:

a. The process of changing cement paste into hard mass, is known as setting of cement

b. The phenomenon by virtue of which cement does not allow transmission of sound, is known as soundness of cement

c. The heat generated during chemical reaction of cement with water, is known as heat of hydration

d. None of these.

28. Di-calcium silicate (C2S)

a. generates less heat of hydration

b. hardens rapidly

c. provides less ultimate strength to cement

d. has less resistance to sulphate attack.

29. Perched acquifers are generally found

a. on the surface of the ground

b. below the surface of the ground but above water table

c. below the water table

d. all the above.

30. The type of joint generally used in cast iron pipes, is

a. flanged joint

b. dresser coupling joint

c. flexible joint

d. all the above.

31. Pick up the correct statement from the following :

a. anaerobic bacteria flourish in the absence of oxygen

b. aerobic bacteria flourish in the presence of oxygen

c. facultative bacteria flourish with or without oxygen

d. all the above.

32. Water content of sewage is about

a. 90%

b. 95%

c. 99.9%

d. 9.9%.

33. Lane's weighted creep theory assumes

a. double weightage to horizontal creep and one weightage to vertical creep

b. triple weightage to horizontal creep and one weightage to vertical creep

c. triple weightage to vertical creep and one weightage to horizontal creep

d. double weightage to vertical creep and one weightage to horizontal creep.

34. Meandering of a river generally occurs, in

a. rocky stage

b. delta stage

c. boulder stage

d. trough stage.

35. An outlet is said to be proportional if its flexibility, is

a. zero

b. less than one

c. more than one

d. one

36. A river is said to be of

a. aggrading type if it builds up its bed to a certain slope

b. degrading type if it cuts its bed to a certain slope

c. meandering type if it flows in sinuous curve

d. all the above.

37. When a number of hair pin bends are introduced, a minimum intervening distance in between is kept

a. 20 m

b. 40 m

c. 60 m

d. 100 m

38. Formation width of a hill road is the distance between

a. outer end of side drain to outer end of parapet wall

b. outer end of side drain to inner end of parapet wall

c. inner end of side drain to outer end of parapet wall

d. inner end of side drain to inner end of parapet wall

39. The type of transition curves generally provided on hill roads, is

a. circular

b. cubic parabola

c. Lemniscate

d. spiral.

40. The type of curves generally provided on highways, is

a. critical curve

b. transition curve

c. vertical curve

d. all the above.

41. The rate of an item of work depends on

a. Specifications of materials

b. Proportion of mortar

c. Method of construction

d. All the above.

42. The expected out turn of 12 mm plastering with cement mortar is

a. 4.0 sq m

b. 6.0 sq m

c. 8.0 sq m

d. 10 sq m

43. The expected out turn of 2.5 cm cement concrete floor per manson per day

a. 2.5 sqm

b. 5.0 sqm

c. 7.5 sqm

d. 10 sqm

44. Pick up the incorrect statement from the following:

a. Dimensions are measured to the nearest 0.01 m

b. Areas are measured to the nearest 0.01 sq.m

c. Cubic contents are measured to the nearest 0.1 cm m

d. Weights are measured to the nearest 0.001 tonnes

45. The critical activity has

a. maximum float

b. minimum float

c. zero float

d. none of these.

46. Site order book is used for recording

a. instructions by the executive engineers

b. construction measurements

c. issue of store equipments

d. names of the casual labour.

47. The object of technical planning, is

a. preparation of estimates

b. initiating the procurement action of resources

c. taking remedial action for likely bottleneck in the execution

d. all the above.

48. Frederick W. Taylor introduced a system of working known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

49. Conical surface of the approach area rises outwards

a. 1 in 10

b. 1 in 15

c. 1 in 20

d. 1 in 25

50. Pick up the correct statement from the following:

a. Localizer indicates to the pilot his position with respect to the proposed alignment

b. The glide slope indicates the correct angle of descent

c. L.O.M. and L.M.M. help the pilot to judge his position for the runway

d. All the above.