***Sub Engineer***

1. The line of sight is kept as high above ground surface as possible to minimise the error in the observed angles due to

a. shimmering

b. horizontal refraction

c. vertical refraction

d. both shimmering and horizontal refraction.

2. The bearing of line AB is 152° 30' and angle ABC measured clockwise is 124° 28'. The bearing of BC is

a. 27° 52'

b. 96° 58'

c. 148° 08'

d. 186° 58'

3. The main principle of surveying is to work

a. from part to the whole

b. from whole to the part

c. from higher level to the lower level

d. from lower level to higher level.

4. The staff reading at a distance of 80 m from a level with the bubble at its centre is 1.31 m. When the bubble is moved by 5 divisions out of the centre, the reading is 1.39 m. The angular value of the one division of the bubble, is

a. 41.25 secs

b. 14.52 secs

c. 25.05

d. none of these

5. Seasoning of timber is done for

a. increasing moisture content

b. decreasing moisture content

c. increasing strength of timber

d. none to these.

6. Refractory bricks are used for

a. retaining walls

b. columns

c. piers

d. combustion chambers.

7. Softer variety of steel may be obtained by

a. Cementation process

b. crucible process

c. Bessemer process

d. Open hearth process.

8. The coefficient of hardness of stones used in road work should be greater than

a. 10

b. 12

c. 15

d. 17.

9. If a solid shaft (diameter 20 cm, length 400 cm, N = 0.8 x 105 N/mm2) when subjected to a twisting moment, produces maximum shear stress of 50 N/mm2, the angle of twist in radians, is

a. 0.002

b. 0.0025

c. 0.003

d. 0.005

10. Pick up the correct statement from the following:

a. Mcg = (M M2 + r2) where letters carry their usual meanings

b. Tcp = m2 + T2 where letters carry their usual meanings

c. The torque which when acting alone would produce maximum shear stress equal to the maximum shear stress caused by the combined bending and torsion, is called equivalent torque

d. All the above.

11. The stress at which extension of a material takes place more quickly as compared to the increase in load, is called

a. elastic point

b. plastic point

c. breaking point

d. yielding point.

12. Pick up the correct statement from the following :

a. In visco-elastic material, stress-strain relation is dependent on time

b. An instropic material has different properties is different directions

c. An orthotropic material has different properties in three mutually perpendicular directions

d. All the above.

13. A syphon is used

a. to connect water reservoirs at different levels intervened by a hill

b. to supply water to a town from higher level to lower level

c. to fill up a tank with water at higher level from a lower level

d. none of these.

14. Mach number is the ratio of inertia force to

a. viscosity

b. surface tension

c. gravitational force

d. elasticity.

15. Flow of water in pipes of diameter more than 3 metres, can be measured by

a. pitot tube

b. venturimeter

c. orifice plate

d. rotameter.

16. A triangular notch is preferred to a rectangular notch because

a. its formula is simple to remember

b. it gives more accurate results for low discharge

c. it measures a wide range of flows accurately

d. all the above.

17. Determination of water content of a soil sample suspected to contain gypsum is made by drying the sample for longer period at a temperature not more than

a. 60°C

b. 80°C

c. 100°C

d. 110°C

18. For a clay slope of height of 10 m, the stability number is 0.05, γ = 2.0 t/m3, C = 2.5 t/m2, the critical height of the slope of the soil, is

a. 4.0 m

b. 12.5 m

c. 25.0 m

d. 15.0 m

19. The fluid generally used for grouting is

a. clay suspension

b. sodium silicate

c. bitumen emulsion

d. all the above.

20. The equation τ = C + σ tan φ is given by

a. Rankine

b. Coulomb

c. Culaman

d. Mohr.

21. A foundation rests on

a. base of the foundation

b. subgrade

c. foundation soil

d. both (b) and (c)

22. For a circular slab carrying a uniformly distributed load, the ratio of the maximum negative to maximum positive radial moment, is

a. 2

b. 3

c. 4

d. 5

23. Piles are usually driven by

a. diesel operated hammer

b. drop hammer

c. single acting steam hammer

d. all the above.

24. The transverse reinforcements provided at right angles to the main reinforcement

a. distribute the load

b. resist the temperature stresses

c. resist the shrinkage stress

d. all the above.

25. The wedge shaped bricks forming an arch ring, are called

a. Soffits

b. voussoirs

c. haunchs

d. spandrils.

26. In flat roof of reinforced cement concrete, the recommended angle of slope, is

a. zero

b. a few degrees

c. 10°

d. 200°.

27. Pick up the correct statement from the following:

a. Water and aggregates are measured in litres

b. The finished concrete is measured in cubic metres

c. 20 bags of cement make one tonne

d. All the above.

28. Pick up the incorrect statement from the following:

a. In properly graded aggregates, bulk density is more

b. In single size aggregates, bulk density is least

c. In single size aggregates, bulk density is maximum

d. None of these.

29. By adding 1 ml of orthotolidine solution to 100 ml chlorinated water taken after contact period, the residual chlorine makes the colour of solution

a. yellowish

b. greenish

c. bluish

d. reddish

30. Mud balls may be removed by

a. breaking and washing

b. washing the filter with a solution of caustic soda

c. removing, cleaning and replacing the damaged sand

d. all the above.

31. Which one of the following part of human body withstands minimum radiation

a. thyroid

b. kidneys

c. eyes

d. ovaries/testis.

32. It is customary to design a sewer for D.W.F. on the basis of

a. average demand

b. twice the average demand

c. thrice the average demand

d. four times the average demand.

33. While deciding the alignment of a proposed canal taking off from a river at B, three alignments of approximately equal lengths are available. These cross a drainages at C1, C2 and C3 where drainage bed levels C1 > C2 > C3, you will provide at site C3

a. an aqueduct

b. a syphon aqueduct

c. a super passage

d. a syphon.

34. In a syphon aqueduct

a. drainage passes over the canal and F.S.L. of the canal is below the bottom of the drainage trough

b. drainage passes over the canal and F.S.L. of the canal is above the bottom of the drainage trough

c. canal passes over the drainage and H.F.L. of the drainage is above the bottom of the canal trough

d. canal passes over the drainage and H.F.L. of the drainage is below the bottom of the canal trough.

35. The top of the capillary zone

a. lies below the water table at every point

b. lies above the water table at every point

c. coincides the water table at every point

d. none of these.

36. Water-shed line is abondened for aligning an irrigation canal if

a. water shed forms a sharp loop

b. canal has to take off from a river

c. towns and villages are located on the water shed line

d. all the above.

37. Enoscope is used to determine

a. spot speed

b. average speed

c. travel time

d. none of these.

38. Retaining walls are generally constructed with dry rubble stones with 60 cm top width and

a. 1 : 2 front batter

b. 1 : 3 front batter

c. 1 : 4 front batter

d. 1 : 5 front batter

39. The distance travelled by a moving vehicle during perception and brake reaction times, is known as

a. sight distance

b. stopping distance

c. lag distance

d. none of these.

40. Width of a rotary round should be equal to

a. twice the width of narrowest radial road

b. width of the widest road

c. width of the widest road plus the width of one lane

d. width of the widest road plus 2 metres.

41. For 100 sq. m cement concrete (1 : 2: 4) 4 cm thick floor, the quantity of cement required, is

a. 0.90 m3

b. 0.94 m3

c. 0.98 m3

d. 1.00 m3

42. A cement concrete road is 1000 m long, 8 m wide and 15 cm thick over the sub-base of 10 cm thick gravel. The box cutting in road crust is

a. 500 m3

b. 1000 m3

c. 1500 m3

d. 2000 m3

43. Pick up the item whose weight is added to the weight of respective item, is

a. Brackets

b. Bolts

c. Distance separators

d. All the above.

44. Anti-siphonage pipe is connected to

a. Main soil pipe

b. Bottom of P trap W.C.

c. Top of P trap W.C.

d. Side of water closet.

45. Pick up the correct statement from the following:

a. The duration of time by which an activity can be delayed without affecting the succeeding activity, is called free float

b. The float which affects neither the processor nor the successor activities, is called independent float

c. The difference between total float and free float, is called interfering float

d. All the above.

46. Pick up the incorrect statement from the following:

a. The various functions under each activity, are shown by one bar on Bar Charts

b. Bar chart establishes the interdependency of one event on another

c. Only approximate percentage of the completed work is reported

d. None of these,

47. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

48. A CPM family includes

a. CPP (Critical Path Plotted)

b. MCE (Minimum Cost Expenditure)

c. CPS (Critical Path Scheduling)

d. All the above.

49. To cope up high temperature of 196°C, the taxi ways and aprons are constructed with

a. rubberised tar concrete

b. plain concrete

c. epoxy asphalt concrete

d. all the above.

50. For night landing, the thresholds are lighted

a. green

b. red

c. white

d. yellow