***Sub Engineer***

1. Pick up the correct statement from the following :

a. the principal axes of both the lenses coincide the optical axis of the telescope

b. the lens nearer the object to be viewed is convex and is called objective

c. the lens nearer the eye is called eyepiece

d. all the above.

2. Pick up the correct statement from the following :

a. the lines of sight while observing back sight and fore  sight lie in the same horizontal plane

b. the   staff readings are measurements made vertically downwards from a horizontal plane

c. the horizontal plane with reference to which staff readings are taken, coincides with the level surface through the telescope axis

d. all the above.

3. Bergchrund is a topograhical feature in

a. plains

b. water bodies

c. hills

d. glaciated region

4. Pick up the correct statement from the following :

a. the apparent error on reversal is twice the actual error

b. the correction may be made equal to half the observed discrepancy.

c. the good results may be obtained from a defective instrument by reversing and taking the mean of two erroneous results

d. all the above.

5. Pick up the compound responsible for early strength of cement from the following:

a. Tetra-calcium alumino-ferrite

b. Tricalcium silicate

c. Tricalcium aluminate

d. Dicalcium silicate.

6. The most commonly used synthetic abrasive is

a. aluminium carbide

b. boric acid

c. silicon

d. All of these.

7. Oil varnish generally consists of

a. synthetic resin and spirit

b. oil, wax and resin

c. resin, oil and turpentine

d. spirit, oil and wax.

8. For high grade instruments the steel preferred to, is

a. cast steel

b. bessemer steel

c. mild steel

d. whitworth compressed steel.

9. The yield moment of a cross section is defined as the moment that will just produce the yield stress in

a. the outer most fibre of the section

b. the inner most fibre of the section

c. the neutral fibre of the section

d. the fibre everywhere

10. If a three hinged parabolic arch, (span l, rise h) is carrying a uniformly distributed load w/unit length over the entire span,

a. horizontal thrust is wl2/8h

b. S.F. will be zero throughout

c. B.M. will be zero throughout

d. all the above.

11. A joint of a frame is subjected to three tensile force P, Q and R equally inclined to each other. If P is 10 tonnes, the other forces will be

a. R + 10 tonnes and Q = zero

b. Q + R = 10 tonnes

c. Q - R = zero

d. Q and R each is equal to 10 tonnes.

12. If two forces acting at a joint are not along the straight line, then for the equilibrium of the joint

a. one of the forces must be zero

b. each force must be zero

c. forces must be equal and of the same sign

d. forces must be equal in magnitude but opposite in sign.

13. Water belongs to

a. Newtonian fluids

b. non-Newtonian fluids

c. compressible fluid

d. none of these.

14. Which one of the following statements is true ?

a. The value of kinetic energy correction factor for turbulent flow lies between 1.03 to 1.06

b. The value of kinetic energy correction factor for laminar flow is 2

c. The practical value of kinetic energy correction factor for turbulent flow is unity

d. all the above.

15. The total pressure force on a plane area is equal to the area multiplied by the intensity of pressure at its centroid, if

a. area is horizontal

b. area is vertical

c. area is inclined

d. all the above.

16. For a most economical trapezoidal open channel, the half of the top width must be equal to

a. the bed width

b. one sloping side

c. the depth of flow

d. None of these.

17. The specific yield of soil depends upon

a. compaction of stratum

b. distribution of pores

c. shape and size of particles

d. all the above.

18. Darcy's law is applicable to seepage if a soil is

a. homogeneous

b. isotropic

c. incompressible

d. all the above.

19. The intensity of active earth pressure at a depth of 10 metres in dry cohesionless sand with an angle of internal friction of 30° and with a weight of 1.8 t/m3, is

a. 5 t/m2

b. 6 t/m2

c. 7 t/m2

d. 8 t/m2

20. Pick up the correct statement from the following:

a. A soil having pH value more than 7 is an acidic soil

b. A soil having pH value less than 7 is an acidic soil

c. A soil having pH value more than 7 is an alkaline soil

d. A soil containing chemicals for the manufacture of portland cement is preferred.

21. Design of R.C.C. cantilever beams, is based on the resultant force at

a. fixed end

b. free end

c. mid span

d. mid span and fixed support.

22. If P kg/m2 is the upward pressure on the slab of a plain concrete footing whose projection on either side of the wall is a cm, the depth of foundation D is given by

a. D = 0.00775 aP

b. D = 0.0775 aP

c. D = 0.07775 aP

d. D = 0.775 Pa

23. If the length of an intermediate span of a continuous slab is 5m, the length of the end span is kept

a. 4.5 m

b. 4.0 m

c. 3.5 m

d. 3.0 m

24. In a combined footing if shear stress exceeds 5 kg/cm2, the nominal stirrups provided are:

a. 8 legged

b. 10 legged

c. 12 legged

d. none of these.

25. The vertical faces of a door opening which support frame of the door, are

a. jambs

b. posts

c. reveals

d. styles.

26. The stepped structure provided for lateral support of a structure, is

a. retaining wall

b. breast wall

c. buttress

d. parapet wall.

27. Specified compressive strengh of concrete is obtained from cube tests at the end of

a. 7 days

b. 14 days

c. 21 days

d. 28 days.

28. According to the recommendations of IS : 456-1978, the expansion joints

a. are provided to ensure minimum resistance

b. are supported on separate columns

c. do not carry reinforcement across them

d. all the above.

29. The detention period for plain sedimentation water tanks, is usually

a. 4 to 8 hours

b. 8 to 16 hours

c. 16 to 24 hours

d. 24 to 36 hours.

30. Pick up the incorrect statement from the following :

a. An impervious layer a few metres below the water table in the sub-soil, is generally called Mota layer (6) Mota layer is very useful to give structural support to open deep wells

b. A bore hole is generally provided in the mota layer for deep wells

c. Shallow wells are always of less depth as compared to deep wells

d. None of these.

31. If the diameter of a sewer is 150 mm, the gradient required for generating self cleansing velocity, is

a. 1 in 60

b. 1 in 100

c. 1 in 120

d. none of these.

32. Bottom openings 15 cm x 15 cm in the standing baffle wall are provided

a. 15 cm c/c

b. 22.5 cm c/c

c. 30 cm c/c

d. 50 cm c/c.

33. According to G.W. Pickles the effect of confining the flood water of a river between levee, is to increase

a. water surface elevation during floods

b. maximum discharge

c. velocity and scouring action

d. surface slope of streams above the leveed portion.

34. A river is said to be of

a. aggrading type if it builds up its bed to a certain slope

b. degrading type if it cuts its bed to a certain slope

c. meandering type if it flows in sinuous curve

d. all the above.

35. A river training work is generally required when the river is

a. meandering

b. aggrading

c. degrading

d. all the above.

36. A fall in a canal bed is generally provided, if

a. ground slope exceeds the designed bed slope

b. designed bed slope exceeds the ground slope

c. ground slope is practically the same as the designed bed slope

d. none of these.

37. The width of the right of way, is influenced by

a. shoulders or berms

b. classification of road

c. availability of funds

d. all the above.

38. In water bound macadam roads, binding material, is

a. sand

b. stone dust

c. cement

d. brick dust.

39. Concrete pavement is provided if daily traffic per lane exceeds

a. 750 tonnes

b. 1000 tonnes

c. 1250 tonnes

d. 1300 tonnes

40. Round-abouts are not suitable if number of vehicles exceed

a. 3000

b. 4000

c. 5000

d. 6000

41. Pick up the item of work not included in the plinth area estimate

a. Room area

b. Verandah area

c. W.C. area

d. Courtyard area.

42. The most reliable estiamte is

a. Detailed estimate

b. Preliminary estimate

c. Plinth area estimate

d. Cube rate estimate

43. The unit of measurement is per quintal for the following:

a. Collapsible gates with rails

b. Rolling shutters

c. Expanded metal wire netting

d. M.S. reinforcement of R.C.C. works.

44. For 100 sq. m cement concrete (1 : 2: 4) 4 cm thick floor, the quantity of cement required, is

a. 0.90 m3

b. 0.94 m3

c. 0.98 m3

d. 1.00 m3

45. The artificial activity which indicates that an activity following it, cannot be started unless the preceding activity is complete, is known as

a. event

b. free float

c. dummy

d. constant

46. If TL is the latest allowable event occurrence time, total activity slack(s), is equal to

a. LST-EST

b. LFT-EFT

c. TL-EFT

d. all the above.

47. Final technical authority of a project lies with

a. Assistant Engineer

b. Executive Engineer

c. Superintending Engineer

d. Chief Engineer.

48. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

49. The bearing of the longest line of a wind rose is S 45° E, the bearing of the runway will be numbered

a. 315°

b. 13

c. 31

d. both (c) and (d)

50. For the proposed runway stated in if the aerodrome reference temperature is 17°.2, the net designed length of the runway is

a. 2716 m

b. 2816 m

c. 2916 m

d. 3016 m