***Sub Engineer***

1. The instrument which is used in plane tabling for obtaining horizontal and vertical distances directly without resorting to chaining, is known as

a. Plane alidade

b. telescopic alidade

c. clinometer

d. tacheometer.

2. During levelling if back sight is more than foresight

a. The forward staff is at lower point

b. The back staff is at lower point

c. The difference in level, cannot be ascertained.

d. none of these.

3. The operation of resection involves the following stepsrough orientation of the plane tablethe three lines form a triangle of errordrawing lines back through the three control pointsselect a point in the triangle of error such that each ray is equally rotated either clockwise or anti clockwisethe points obtained by three rays is the correct location. The correct sequence is

a. 1, 3, 2, 4, 5

b. 1, 2, 3, 4, 5

c. 1, 4, 3, 2, 5

d. 1, 4, 2, 3, 5

4. The angle of intersection of a contour and a ridge line, is

a. 30°

b. 45°

c. 60°

d. 90°.

5. Magnese steels

a. possess high electrical resistance

b. possess low coefficient of expansion

c. are used for the manufacture of rails

d. all the above.

6. Pick up the non-inflammable plastic from the following:

a. Cellulose acetate plastics

b. Polyvinyl chloride plastics

c. Phenol formaldehyde plastic

d. Urea formaldehyde plastic.

7. Duco is one of the patent forms of

a. plastic paints

b. bituminous paints

c. aluminium paints

d. cellulose paints.

8. The cracks caused by shrinkage of the exterior surface of the wood exposed to atmosphere, are called:

a. radial shakes

b. heart shakes

c. wind cracks

d. twisted fibres.

9. Keeping the depth d constant, the width of a cantilever of length l of uniform strength loaded with a uniformly distributed load w varies from zero at the free end and

a.  at the fixed end

b.  at the fixed end

c.  at the fixed end

d.  at the fixed end

10. The ratio of the area of cross-section of a circular section to the area of its core, is

a. 4

b. 8

c. 12

d. 16

11. The section modulus of a rectangular section is proportional to

a. area of the section

b. square of the area of the section

c. product of the area and depth

d. product of the area and width

12. The moment diagram for a cantilever carrying a concentrated load at its free end, will be

a. triangle

b. rectangle

c. parabola

d. cubic parabola.

13. In an open tube, free surface of mercury remains

a. horizontal

b. curved upwards

c. curved downwards

d. none of these.

14. The pressure rise due to water hammer depends upon

a. the length of pipe

b. time taken to close the valve

c. the elasticity of the pipe material

d. All of above.

15. The magnitude of water hammer in a pipe depends upon

a. length of the pipe line

b. elastic properties of the pipe material

c. elastic properties of the following liquid

d. all the above.

16. An independent mass of a fluid does not posses

a. elevation energy

b. kinetic energy

c. pressure energy

d. none of these.

17. If W1, W2, W3 and W4 are the sequential weights obtained during observations in pycnometer method for determining water content, the formula to be used, is

a. W =

b. W =

c. W =

d. W =

18. Through a point in a loaded soil, the principal stress is maximum on

a. minor principal plane

b. intermediate principal plane

c. major principal plane

d. none of these.

19. A pycnometer is used to determine

a. voids ratio

b. dry density

c. water content

d. density index.

20. Pile foundations are generally preferred to for

a. bridge foundations

b. sky scrapper buildings

c. residential buildings

d. runways.

21. For a number of columns constructed in a rcjw, the type of foundation provided, is

a. raft

b. strap

c. strip

d. a combination of the above.

22. Pick up the incorrect statement from the following: Tensile reinforcement bars of a rectangular beam

a. are curtailed if not required to resist the bending moment

b. are bent up at suitable places to serve as shear reinforcement

c. are bent down at suitable places to serve as shear reinforcement

d. are maintained at bottom to provide at least local bond stress.

23. If the permissible compressive and tensile stresses in a singly reinforced beam are 50 kg/cm2 and 1400 kg/cm2 respectively and the modular ratio is 18, the percentage area At of the steel required for an economic section, is

a. 0.596%

b. 0.696%

c. 0.796%

d. none of these.

24. If permissible working stresses in steel and concrete are respectively 1400 kg/cm2 and 80 kg/cm2 and modular ratio is 18, in a beam reinforced in tension side and of width 30 cm and having effective depth 46 cm, the lever arms of the section, is

a. 37 cm

b. 38 cm

c. 39 cm

d. 40 cm

25. Stud(s) of a common wooden partition

a. are vertical wooden members

b. is the upper horizontal wooden member

c. is the lower horizontal wooden member

d. are the intermediate horizontal wooden members.

26. In soft clay of low bearing capacity, the type of steel pile generally used, is

a. screw pile

b. disc pile

c. pipe pile

d. raking pile.

27. Slump test is done for

a. clay

b. sand

c. lime

d. concrete.

28. Under normal conditions using an ordinary cement, the period of removal of the form work, is :

a. 14 days for bottom slabs of spans 4.6 m and more

b. 21 days for bottom beams over 6 m spans

c. 2 days for vertical sides of columns

d. all the above.

29. Pick up the correct statement from the following :

a. Lime may be used for softening hard water

b. Excess lime when added to water, raises the pH value of water

c. the bacterias do not survive in water having pH value exceeding 9.5

d. All the above.

30. Velocity of flow of water in plain sedimentation water tank, is normally kept

a. 10 cm/minute

b. 20 cm/minute

c. 30 cm/minute

d. 40 cm/minute.

31. The width of a settling tank with 2 hour detention period for treating sewage 378 cu m per hour, is

a. 5.5 m

b. 6 m

c. 6.5 m

d. 7 m.

32. In areas where light rains are uniformly distributed throughout the year, the type of sewerage system to be adopted is

a. separate system

b. combined system

c. partially combined system

d. none of these.

33. For cereal crops the most commonly adopted method of irrigation, is

a. free flowing method

b. check method

c. furrow method

d. sprinkling method.

34. According to G.W. Pickles the effect of confining the flood water of a river between levee, is to increase

a. water surface elevation during floods

b. maximum discharge

c. velocity and scouring action

d. surface slope of streams above the leveed portion.

35. If the optimum depth of kor watering for a crop is 15.12 cm, the outlet factor for the crop for four week period in hectares per cumec, is

a. 1200

b. 1400

c. 1600

d. 2000

36. Pick up the incorrect statement from the following:

a. In free flooding irrigation, water is admitted at one corner of a field and is allowed to spread over the entire area

b. In check method of irrigation, the field is divided into smaller compartments and water is admitted to each in turn

c. In furrow irrigation water is admitted between the rows of plants in the field

d. None of these.

37. In case of a multi-lane road, overtaking is generally permitted

a. from right

b. from left

c. from both sides right and left

d. not at all.

38. Extra widening required at a horizontal curve on a single lane hill road of radius 80 m for a design speed of 50 km ph and for a vehicle with wheel base 6.0 m is

a. 0.225 m

b. 0.589 m

c. 1.250 m

d. none of these

39. Length of vehicles does not affect

a. minimum radius of turning

b. passing sight distance

c. width of shoulders

d. none of these.

40. As per recommendations of I.R.C., traffic volume study is carried out for rural roads for 7 days continuously during

a. harvesing

b. lean season

c. harvesting and lean season

d. none of these.

41. The measurement is not made in square metres in case of

a. D.P.C. (Damp proof course)

b. Form works

c. Concrete Jaffries

d. R.C. Chhajja.

42. Brick walls are measured in sq. m if the thickness of the wall is

a. 10 cm

b. 15 cm

c. 20 cm

d. None of these.

43. Pick up the item of work not included in the plinth area estimate

a. Room area

b. Verandah area

c. W.C. area

d. Courtyard area.

44. For 12 mm thick cement plastering 1 : 6 on 100 sq.m new brick work, the quantity of cement required, is

a. 0.200 m3

b. 0.247 m3

c. 0.274 m3

d. 0.295 m3

45. Which one of the following represents an event ?

a. concrete cured

b. fixing of door

c. plastering of walls

d. selecting sites

46. A dummy activity

a. is artificially introduced

b. is represented by a dotted line

c. does not consume time

d. all the above.

47. Pick up the correct statement from the following:

a. PERT analysis is event oriented

b. CPM does not make any allowance for the uncertainties in the duration of time

c. In CPM, the time is related to cost

d. All the above.

48. Pick up the correct statement from the following:

a. Programme Evaluation and Review Technique, is event oriented

b. Programme Evaluation and Review Technique is not event oriented

c. Critical Path Method is event oriented

d. Critical Path method is event oriented.

49. Pick up the correct statement from the following:

a. The basic length of a runway is increased at a rate of 7% per 300 m of elevation of M.S.L.

b. The standard temperature at the site is obtained by reducing the standard sea level temperature of 15°C at the rate of 6.5°C per 1000 m rise in elevation

c. The aerodrome reference temperature is the monthly mean of the mean daily temperature for the hottest month of the year

d. All the above.

50. Effective length of a runway is the distance between

a. ends of the runway

b. point of intersection of the obstruction clearance line and the extended plane of the runway surface, and the other end of the runway

c. point of intersection of the glide path and the extended plane of the runway surface and the other end of the runway

d. ends of the clear way on either side.