***Sub Engineer***

1. Metric chains are generally available in

a. 10 m and 20 m length

b. 15 m and 20 m length

c. 20 m and 30 m length

d. 25 m and 100 m length

2. If a tacheometer is fitted with an anal-latic lens

a. additive constant is 100, multiplying constant is zero

b. multiplying constant is 100, additive constant is zero

c. both multiplying and additive constants are 100

d. both multiplying and additive constants are 50.

3. A traverse deflection angle is

a. less than 90°

b. more than 90° but less than 180°

c. the difference between the included angle and 180°

d. the difference between 360° and the included angle.

4. The instrument which is used in plane tabling for obtaining horizontal and vertical distances directly without resorting to chaining, is known as

a. Plane alidade

b. telescopic alidade

c. clinometer

d. tacheometer.

5. In paints, the pigment is responsible for

a. colour

b. smoothness

c. glassy face

d. none of these.

6. Formula for quick lime, is

a. CaCO3

b. CaO

c. CO3CO2

d. none to these.

7. Initial setting of cement is caused due to

a. Tri-calcium silicate

b. Di-calcium silicate

c. Tri-calcium aluminate

d. Tetra calcium alumino ferrite.

8. Which one of the following is an air binding material ?

a. Gypsum

b. Acid-resistant cement

c. Quick lime

d. All of these.

9. A steel rod of sectional area 250 sq. mm connects two parallel walls 5 m apart. The nuts at the ends were tightened when the rod was heated to 100°C. If αsteel = 0.000012/C°, Esteel = 0.2 MN/mm2, the tensile force developed at a temperature of 50°C, is

a. 80 N/mm2

b. 100 N/mm2

c. 120 N/mm2

d. 150 N/mm2

10. A close coil helical spring when subjected to a moment M having its axis along the axis of the helix

a. it is subjected to pure bending

b. its mean diameter will decrease

c. its number of coils will increase

d. all the above.

11. If the beam is supported so that there are only three unknown reactive elements at the supports. These can be determined by using the following fundamental equation of statics

a. ∑V = 0

b. ∑H = 0; ∑H = 0

c. ∑H = 0; ∑V = 0; ∑M = 0

d. ∑M = 0; ∑H = 0

12. The width b and depth d of a beam cut from a wooden cylindrical log of 100 cm diameter for maximum strength are :

a. b = 57.73 cm d = 81.65 cm

b. b = 81.65 cm d = 57.73 cm

c. b = 50.00 cm d = 50.00 cm

d. b = 40.00 cm d = 80.00 cm

13. The line joining the points to which the liquid rises in vertical piezometer tubes fitted at different cross-sections of a conduit, is known as

a. piezometric line

b. pressure grade line

c. hydraulic grade line

d. all the above.

14. The velocity of the fluid particle at the centre of the pipe section, is

a. minimum

b. maximum

c. equal throughout

d. none of these.

15. A steady uniform flow is through

a. a long pipe at constant rate

b. an expanding tube at constant rate

c. an expanding tube at increasing rate

d. a long pipe at increasing rate.

16. Capillary rise of water is

a. directly proportional to surface tension

b. inversely proportional to water density

c. inversely proportional to diameter of the tube

d. All of these.

17. 260 g of wet soil was taken in a pycnometer jar of weight 400 g in order to find the moisture content in the soil, with specific gravity of soil particles 2.75. The weight of soil and remaining water filled in pycnometer without air bubbles was 1415 g and the weight of pycnometer filled with water alone was 1275 g. The moisture content in the soil is

a. 24.2%

b. 18.2%

c. 53.8%

d. none of these.

18. Failure of the stability of slopes, generally occurs along

a. slip plane

b. a horizontal surface

c. a curved surface

d. all the surfaces.

19. The neutral stress in a soil mass is

a. force per neutral area

b. force per effective area

c. stess taken up by the pore water

d. stress taken up by solid particles.

20. If the specific gravity of a soil particle of 0.05 cm diameter is 2.67, its terminal velocity while settling in distilled water of viscosity, 0.01 poise, is

a. 0.2225 cm/sec

b. 0.2250 cm/sec

c. 0.2275 cm/sec

d. 0.2300 cm/sec

21. The anchorage value of a hook is assumed sixteen times the diameter of the bar if the angle of the bend, is

a. 40°

b. 45°

c. 60°

d. all the above.

22. A flat slab is supported

a. on columns

b. on beams and columns

c. on columns monolithicaily built with slab

d. all the above

23. The effective span of a simply supported slab, is

a. distance between the centres of the bearings

b. clear distance between the inner faces of the walls plus twice the thickness of the wall

c. clear span plus effective depth of the slab

d. none of these.

24. If the loading on a prestressed rectangular beam, is uniformly distributed, the tendon to be provided should be .

a. parabolic with convexity downward

b. parabolic with convexity upward

c. straight above centroidal axis

d. none of these.

25. The stone blocks approximately triangular in shape, used as steps, are known

a. stone steps

b. built up steps

c. spandril steps

d. none of these.

26. In grillage foundations, distance between flanges of grillage beams, is kept

a. 40 cm

b. equal to flange width

c. twice the flange width

d. maximum of (a), (b) and (c).

27. The percentage of the aggregate of F.M. 2.6 to be combined with coarse aggregate of F.M. 6.8 for obtaining the aggregates of F.M. 5.4, is

a. 30%

b. 40%

c. 50%

d. 60%.

28. ISI has specified full strength of concrete after

a. 14 days

b. 21 days

c. 28 days

d. none of these.

29. Cast iron pipes

a. resist corrosion satisfactorily

b. may last for 100 years

c. are normally manufactured in lengths of about 3.5 m

d. all the above.

30. Distribution of wash water is provided in

a. sedimentation tank

b. slow sand filter

c. rapid gravity filter

d. all the above.

31. Hazin's formula  is used for the settlement velocity of the particles of diameter

a. less than 0.01 mm

b. less than 0.05 mm

c. less than 0.1 mm

d. greater than 0.1 mm.

32. A manhole is generally provided at each

a. junction

b. change of gradient

c. change of sewer diameter

d. all the above.

33. Bed bars in a canal are provided

a. to watch the general behaviour of canal

b. to measure the discharge

c. to raise the supply level

d. to control the silting

34. The width of a dowla is generally kept between 30 to 60 cm and its height above the road level should invariably be more than

a. 10 cm

b. 20 cm

c. 30 cm

d. 40 cm

35. Groynes are generally built

a. perpendicular to the bank

b. inclined up stream up to 30°

c. inclined down stream upto 30°

d. all the above.

36. The length and width of a meander and also the width of the river, vary roughly as

a. square root of the discharge

b. discharge

c. square of the discharge

d. cube of the discharge.

37. Volume of traffic which would immediately use a new road or an improved one when opened to traffic, is known

a. development traffic

b. current traffic

c. general traffic

d. normal traffic growth.

38. Pick up the incorrect statement from the following:

a. Highways are always constructed in straight line

b. Highways may be provided horizontal curves

c. Highways may be provided vertical curves

d. Highways may be provided both horizontal and vertical curves.

39. If cross slope of a country is 10% to 25%, the terrain is classified as

a. rolling

b. mountainous

c. steep

d. plain.

40. Road width 8.8 m of two lane National highways or State highways in mountainous terrain

a. excludes the width of parapet (0.6 m)

b. excludes the width of side drain (0.6 m)

c. excludes the width of parapet and side drain

d. includes the width of parapet and side drain

41. The cross-sections for a highway is taken at

a. 30 metres apart

b. intermediate points having abrupt change in gradient

c. the starting end points of the curves

d. All the above.

42. The total length of a cranked bar through a distance (d) at 45° in case of a beam of effective length L, is

a. L + 0.42 d

b. L + 2 x 0.42 d

c. L - 0.42 d

d. L - 2 x 0.4 d

43. In long and short wall method of estimation, the length of long wall is the centre to centre distance between the walls and

a. breadth of the wall

b. half breadth of wall on each side

c. one fourth breadth of wall on each side

d. None of these.

44. A cement concrete road is 1000 m long, 8 m wide and 15 cm thick over the sub-base of 10 cm thick gravel. The box cutting in road crust is

a. 500 m3

b. 1000 m3

c. 1500 m3

d. 2000 m3

45. Pick up the incorrect statement from the following:

a. The tail of the arrow indicates the start of the activity

b. The head of the arrow indicates the end of the activity

c. The arrows are drawn to scale from left to right

d. Each activity consumes a given time.

46. Pick up the incorrect statement from the following:

a. The difference between the maximum time available for the job and actual time it consumes, is called total float

b. The difference between the latest start time and earliest start time of an activity, is called total float

c. The difference between the earliest finish time of an activity and the earliest start time of its successor activity, is called free float of the activity

d. None of these.

47. PERT analysis is based on

a. optimistic time

b. pessimistic time

c. most likely time

d. all the above.

48. Pick up the correct network for the activities of pouring concrete, erection of form work, removal of form work and curing of concrete from the following:

a. 1. Pouring of Concrete  2. Erection of form work 3. Curing of concrete 4. Removal of form work

b. 1. Erection of form work 2. Pouring of concrete 3. Curing of concrete 4. Removal of form work

c. 1. Removal of form work  2. Erection of form work 3. Pouring of concrete 4. Curing of concrete

d. 1. Pouring of concrete 2. Curing of concrete 3. Erection of form work 4. Removal of form work

49. If the width of the approach area near the runway end is 150 m, the width of the approach area at a distance of 3 kilometres from runway end will be

a. 1500 m

b. 1200 m

c. 1000 m

d. 800 m

50. Pick up the incorrect statement from the following:

a. L.M.M. is installed at 1.0 km from the threshold

b. Localizer antenna is installed at 300 m from the other end of the runway

c. Glide slope antenna is installed at the centre of the runway about 150 m on one side

d. None of these.