***Sub Engineer***

1. Two contour lines, having the same elevation

a. cannot cross each other

b. can cross each other

c. cannot unite together

d. can unite together.

2. The curvature of the earth's surface, is taken into account only if the extent of survey is more than

a. 100 sq km

b. 160 sq km

c. 200 sq km

d. 260 sq km.

3. The included angles of a theodolite traverse, are generally measured

a. clockwise from the forward station

b. anti-clockwise from the back station

c. anti-clockwise from the forward station

d. clockwise from the back station.

4. Pick up the correct statement from the following :

a. Cross staff is used for setting out right angles

b. Gradiometer is used for setting out any required gradient

c. Line ranger is used for locating intermediate stations on a survey line

d. All the above.

5. With storage, strength of cement

a. increases

b. decreases

c. remains the same

d. none to these.

6. Smith's test of stones is performed to find out

a. the presence of soluble matter of stone

b. the compressive strength of the stone

c. the hardness of the stone

d. the toughness of the stone.

7. Pozzolana (or surkhi) is used in lime

a. to impart hydraulicity

b. to prevent shrinkage

c. to decrease the cost of construction

d. to decrease the setting time.

8. The standard size of masonry bricks, is

a. 19 cm x 9 cm x 9 cm

b. 20 cm x 10 cm x 10 cm

c. 21 cm x 11 cm x 11 cm

d. none of these.

9. A compound bar consists of two bars of equal length. Steel bar cross-section is 3500 mm2 and that of brass bar is 3000 mm2. These are subjected to a compressive load 100,000 N. If Eb = 0.2 MN/mm2 and Eb = 0.1 MN/mm2, the stresses developed are:

a. σb = 10 N/mm2, σs = 20 N/mm2

b. σb = 8 N/mm2, σs = 16 N/mm2

c. σb = 6 N/mm2, σs = 12 N/mm2

d. σb = 5 N/mm2, σs = 10 N/mm2

10. The ratio of the area of cross-section of a circular section to the area of its core, is

a. 4

b. 8

c. 12

d. 16

11. The maximum bending moment due to a moving load on a simply supported beam, occurs

a. at the supports

b. under the load

c. anywhere on the beam

d. none of these.

12. If a constant section beam is subjected to a uniform bending moment throughout, its length bends to

a. a circular arc

b. a parabolic arc

c. a catenary

d. none of these.

13. The discharge through a V-notch weir varies as

a. 1/H

b. H3/2

c. H7/2

d. H5/2.

14. If Cv, Cc, Cd and Cr are the hydraulic coefficients of an orifice, then

a. Cd = Cc . Cv

b. Cr = 1 + Cv2/Cd

c. Cv = Cc + Cd

d. Cc = Cv/Cd

15. The value of momentum correction factor (β) for a laminar flow through a circular pipe, is

a. 2/3

b. 3/4

c. 4/3

d. 3/2

16. In steady flow, which one of the following changes with time

a. velocity

b. pressure

c. density

d. none of these.

17. The water content of soil is defined as the ratio of

a. volume of water to volume of given soil

b. volume of water to volume of voids in soil

c. weight of water to weight of air in voids

d. weight of water to weight of solids of given mass of soil.

18. The ratio of volume of air voids to the volume of total voids, is known as

a. air content

b. percentage air voids

c. percentage voids

d. porosity.

19. The capillary rise of water

a. depends upon the force responsible

b. increases as the size of the soil particles increases

c. decreases as the size of the soil particles decreases

d. is less in wet soil than in dry soil.

20. In active state of plastic equilibrium in a non cohesive soil with horizontal ground surface

a. major principal stress is horizontal

b. minor principal stress is vertical

c. major principal stress is vertical

d. minor and major principal stresses are equally inclined to horizontal.

21. In a prestressed member it is advisable to use

a. high strength concrete only

b. low strength concrete but high tensile steel

c. high strength concrete and high tensile steel

d. high strength concrete but low tensile steel

22. The minimum head room over a stair must be

a. 205 cm

b. 210 cm

c. 200 cm

d. 230 cm

23. In a combined footing if shear stress exceeds 5 kg/cm2, the nominal stirrups provided are:

a. 8 legged

b. 10 legged

c. 12 legged

d. none of these.

24. A pre-cast pile generally used, is

a. circular

b. square

c. octagonal

d. square with corners chamfered.

25. Pick up the correct statement from the following:

a. Flush  door  is  generally  provided   in dinning room

b. Revolving door is generally provided in cinema halls

c. Sliding door is generally provided in show rooms

d. All the above.

26. A stair should not have pitch more than

a. 30°

b. 40°

c. 50°

d. 60°.

27. M10 grade of concrete approximates

a. 1 : 3 : 6 mix

b. 1 : 1 : 2 mix

c. 1 : 2 : 4 mix

d. 1 : 1.5 : 3 mix

28. Pick up the correct statement from the following:

a. The maximum size of the fine aggregate, is 4.75 mm and minimum 0.075 mm

b. The material having particles of size varying from 0.06 mm to 0.002 mm, is known as silt

c. The material having particles of size less than 0.002 mm, is known as clay

d. All the above.

29. The population of a city in 2000 is 50, 000. The average per decade of the previous records of population is 5000 and average percentage per decade is 20%. The population of the city based on geometrical increase method, in the year 2020 will be

a. 60, 000

b. 64, 000

c. 70, 000

d. 72, 000

30. Continuous flow of water can be expected from

a. gravity springs

b. surface springs

c. artesian springs

d. none of these.

31. D is the diameter of a circular sewer and α is the side of a square section sewer. If both are hydraulically equivalent, the relationship which holds good, is

a. πD8/3 = 4 b8/3

b. πD3/8 = 4 b3/8

c. πD2/3 = 4 b2/3

d. πD3/2 = 4 b3/2

32. Pathogens (or pathogentic bacterias) in water may cause

a. cholera

b. dysentery

c. infectious hepatitis

d. all the above.

33. The top soil of a water logged field becomes more alkaline and infertile if its pH value is

a. 8

b. 9

c. 10

d. 11

34. The difference in level between the top of a bank and supply level in a canal, is called

a. berm

b. free board

c. height of bank

d. none of these.

35. The main cause of silting up a channel,

a. inadequate slope

b. defective head regulator

c. defective outlets

d. all the above.

36. If average particle size of the silt in millimetres is m, the Lacey's silt factor f is proportional to

a. m3

b. m1/2

c. m1/3

d. m2/3

37. The top height of a route marker above crown level is

a. 1.50 m

b. 1.75 m

c. 2.00 m

d. 2.25 m

38. The normal road land width for a National or State highway, in open areas should be

a. 45 m

b. 30 m

c. 24 m

d. 20 m

39. Increase in traffic volume, due to increase in transport vehicles, is known as

a. normal traffic growth

b. generated traffic growth

c. current traffic

d. none of these.

40. Hill roads are

a. provided camber along spur curves

b. provided camber along valley curves

c. seldom provided a camber

d. None of these.

41. Pick up the incorrect statement from the following:

a. The built up covered area at the floor level of any storey of a building is called plinth area

b. The usable covered area of the rooms of any storey of a building is called carpet area

c. The carpet area of a building along with area of its kitchen, pantry, store, lavatory, bath room and glazed verandah, is called floor area

d. None of these.

42. The expected out turn of 2.5 cm cement concrete floor per manson per day

a. 2.5 sqm

b. 5.0 sqm

c. 7.5 sqm

d. 10 sqm

43. The area is measured correct to the nearest

a. 0.01 sqm

b. 0.02 sqm

c. 0.03 sqm

d. 0.04 sqm

44. The brick work is measured in sq metre, in case of

a. Honey comb brick work

b. Brick flat soling

c. Half brick walls or the partition

d. All the above.

45. The artificial activity which indicates that an activity following it, cannot be started unless the preceding activity is complete, is known as

a. event

b. free float

c. dummy

d. constant

46. Pick up the correct statement from the following:

a. Programme Evaluation and Review Technique, is event oriented

b. Programme Evaluation and Review Technique is not event oriented

c. Critical Path Method is event oriented

d. Critical Path method is event oriented.

47. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

48. Pick up the correct statement from the following:

a. Backward pass is used for calculating the latest occurence time

b. Maximum value of earliest expected time is used if there are more than one value of any event

c. Minimum value of latest occurrence time is used if there are more than one value of any event

d. All the above.

49. The height of the pilot's eye above the runway surface is assumed

a. 1 m

b. 3 m

c. 4 m

d. 5 m

50. Pick up the correct statement from the following:

a. Localizer indicates to the pilot his position with respect to the proposed alignment

b. The glide slope indicates the correct angle of descent

c. L.O.M. and L.M.M. help the pilot to judge his position for the runway

d. All the above.