***Sub Engineer***

1. The tangent to the liquid surface in a level tube, is parallel to the axis of the level tube at

a. every point of the bubble

b. either end of the bubble

c. the mid-point of the bubble

d. no where.

2. Contours of different elevations may cross each other only in the case of

a. an over hanging cliff

b. a vertical cliff

c. a saddle

d. an inclined plane.

3. The formula for the horizontal distances for inclined sights, on staff held normalis  s cos θ + (f + d) cos θ ± h sin θ

a. minus sign is used for angle of depression

b. plus sign is used for angle of depression

c. minus sign is used for angle of elevation

d. non of these.

4. In levelling operation

a. if first reading is more than second, it represents a rise

b. if  first reading is less than second, it represents a fall

c. if  second reading is less than first, it represents a fall

d. both (b) and (c).

5. Whitworth compressed steel is obtained when molten steel is subjected to a pressure of

a. 9 kg/mm2

b. 13 kg/mm2

c. 15 kg/mm2

d. 10 kg/mm2.

6. Upto a maximum of 72% of iron, is available in

a. Magnetite

b. Red haemetite

c. Limonite

d. Siderite

7. Quick lime (or caustic lime)

a. is obtained by the calcination of pure lime stone

b. has great affinity to moisture

c. is amorphous

d. All the above.

8. Pick up the rock which is not  a sedimentary rock from the following:

a. sand stone

b. gypsum

c. dolerite

d. lignite.

9. For determining the force in the member AB of the truss shown in the given figure by method of sections, the section is made to pass through AB, AD and ED and the moments are taken about

a. joint C

b. joint B

c. joint D

d. joint A

10. If a concrete column 200 x 200 mm in cross-section is reinforced with four steel bars of 1200 mm2 total cross-sectional area. Calculate the safe load for the column if permissible stress in concrete is 5 N/mm2 and Es is 15 Ec

a. 274 MN

b. 284 MN

c. 294 MN

d. None of these.

11. Pick up the correct statement from the following :

a. A ductile material has large plastic zone

b. A brittle material has no plastic zone

c. A rigid material has no plastic zone

d. All the above.

12. In a beam, the neutral plane

a. may be its centre

b. passes through the C.G. of the area of cross-section

c. does not change during deformation

d. none of these.

13. The height of water level in a tank above the centre of a circular hole 2.5 cm in diameter is 50 m. The velocity of water flowing through the hole, is

a. 31.1 m/sec

b. 31.2 m/sec

c. 31.3 m/sec

d. 31.4 m/sec.

14. To avoid vapourisation, pipe lines are laid over the ridge so that these are above the hydraulic gradient line, not more than

a. 2.4 m

b. 6.4 m

c. 10.0 m

d. 5.0 m.

15. When a liquid rotates at constant angular velocity about a vertical axis of a rigid body, the pressure

a. varies inversely as the altitude along any vertical line

b. varies as the square of the radial distance

c. decreases as the square of the radial distance

d. none of these.

16. Capillary rise of water is

a. directly proportional to surface tension

b. inversely proportional to water density

c. inversely proportional to diameter of the tube

d. All of these.

17. Pick up the correct statement from the following:

a. A soil classification system is meant to provide an accepted and systematic method of describing the various types of soils eliminating personal factors

b. The first category of soil classification is based on grain size of the soil

c. The second category of soil classification is based on fine as well as coarse grains

d. All the above.

18. The Mohr's straight theory is based on the following fact :

a. Material fails essentially by shear

b. Ultimate strength of the material is determined by the stress in the plane of slip

c. Failure criterion is independent of the intermediate principal stress

d. All the above.

19. The plasticity of fine soils may be assessed by means of

a. dry strength test

b. toughness test

c. dilatancy test

d. all of these.

20. The method of the slices is applicable to

a. homogenous soils

b. stratified soils

c. saturated soils

d. non-uniform slopes

21. In a combined footing for two columns carrying unequal loads, the maximum hogging bending moment occurs at

a. more loaded column

b. a point equidistant from either column

c. a point of the maximum shear force

d. a point of zero shear force.

22. If the width of the foundation for two equal columns is restricted, the shape of the footing generally adopted, is

a. square

b. rectangular

c. trapezoidal

d. triangular.

23. Pick up the incorrect statement from the following:

a. In the stem of a retaining wall, reinforcement is provided near the earth side

b. In the toe slab of a retaining wall, rein forcement is provided at the bottom of the slab

c. In the heel slab of a retaining wall, rein forcement is provided at the top of the slab

d. None of these.

24. If R and T are rise and tread of a stair spanning horizontally, the steps are supported by a wall on one side and by a stringer beam on the other side, the steps are designed as beams of width

a. R + T

b. T-R

c. R2 + T2

d. R-T

25. A pre-stressed concrete pile is

a. lighter in weight

b. extremely durable

c. suitable for heavy load

d. all the above.

26. Rotary drilling

a. is not suitable for deposits containing very coarse gravel

b. hinders the ground water observations and permeability test

c. is not economical for holes of less than 10 cm.

d. all the above.

27. Pick up the correct statement from the following:

a. Construction joints in columns are provided at the bottom haunching

b. Construction joints in beams and slabs are provided within middel third

c. Constructionjoints are generally provided in positions subjected to least shear force

d. All the above.

28. Permissible compressive strength of M 300 concrete grade is

a. 150 kg/cm2

b. 200 kg/cm2

c. 250 kg/cm2

d. 300 kg/cm2

29. Efficiency of removing bacterias from raw water by a slow sand filter, is

a. 80% to 81%

b. 85% to 86%

c. 90% to 97%

d. 98% to 99%.

30. For calculation of economical diameter D of a pipe in metres for a discharge Q to be pumped in cumecs, Lea suggested the empirical formula

a. D = 1.22 Q

b. D = 2.22 Q

c. D = 3.22 Q

d. D = 1.33 Q.

31. With self cleansing velocity in sewers

a. silting occurs at bottom

b. scouring occurs at bottom

c. both silting and scouring occur at bottom

d. neither silting nor scouring occurs at bottom.

32. If the diameter of sewer is 225 mm, the gradient required for generating self cleansing velocity, is

a. 1 in 60

b. 1 in 100

c. 1 in 120

d. none of these.

33. The crest level of a canal diversion head work, depends upon

a. discharge perimeters

b. pond level

c. all the above

d. none these.

34. If water table is comparatively high, the irrigation canal becomes useless, due to

a. water logging of the cultivated areas

b. uncertain water demand

c. spread of malaria

d. all the above.

35. The uplift pressure on the roof of an inverted syphon, is maximum when

a. drain in running dry

b. canal is running dry

c. canal is running with F.S.L.

d. drain is running with H.F.L.

36. A current meter measures the velocity of flow, if it is held

a. on the surface of water

b. at the bottom of channel

c. at any point within the cross-section

d. none of these.

37. The traffic manoeuvre means

a. diverging

b. merging

c. crossing

d. all the above.

38. Thickness of a pavement may be reduced considerably by

a. compaction of soil

b. stabilisation of soil

c. drainage of soil

d. combination of all the above.

39. The camber on pavements, is provided by

a. parabolic method

b. straight line and parabolic at crown

c. elliptical method

d. circular method.

40. The weaving length of a roadway is the distance

a. between the channelising islands

b. equal to half circumference

c. equal to total width of adjoining radial roads

d. equal to diameter of rotary.

41. The trap which is provided to disconnect the house drain from the street sewer is called

a. Intercepting trap

b. Interception manhole

c. Intercepterchamber

d. All the above.

42. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

43. The ground surface slopes 1 in 50 along a proposed railway embankment 150 m in length. The height of the embankment at zero chainage is 0.5 m, the width is 11 m and side slopes 2:1. If the falling gradient of the embankment is 1 in 150, the quantity of the earthwork calculated by prismoidal formula, is

a. 3250 m3

b. 3225 m3

c. 3275 m3

d. 3300 m3

44. In case of laying gullies, siphons, intercepting traps, the cost includes

a. Setting and laying

b. Bed concreting

c. Connection to drains

d. All of these.

45. While scheduling a project by C.P.M.

a. required time for each activity is established

b. sequence of various activities is made according to their importance

c. net work is drawn by connecting the activities and the events

d. All the above.

46. Final technical authority of a project lies with

a. Assistant Engineer

b. Executive Engineer

c. Superintending Engineer

d. Chief Engineer.

47. If D is the duration, ES and EF are the earliest start and finish, LS and LF are latest start and latest finish time, then the following relation holds good

a. LS = LF - D

b. LF = LS + D

c. D = EF - ES

d. all the above.

48. Site order book is used for recording

a. instructions by the executive engineers

b. construction measurements

c. issue of store equipments

d. names of the casual labour.

49. A gradient of + 0.08% is followed by a gradient of - 0.07%. If the permissible rate of change of grade is 0.003 per 30 metres, the length of the transition curve, is

a. 150 m

b. 140 m

c. 160 m

d. 175 m

50. According to the International Civil Aviation Organisation (I.C.A.O.) the strength of runway pavements, have been coded by

a. Last Seven English alphabets

b. First Seven English alphabets

c. First seven numbers

d. None of these.