***Sub Engineer***

1. While viewing through a level telescope and moving the eye slightly, a relative movement occurs between the image of the levelling staff and the cross hairs. The instrument is

a. correctly focussed

b. not correctly focussed

c. said to have parallax

d. free from parallax.

2. The formula for the horizontal distances for inclined sights, on staff held normalis  s cos θ + (f + d) cos θ ± h sin θ

a. minus sign is used for angle of depression

b. plus sign is used for angle of depression

c. minus sign is used for angle of elevation

d. non of these.

3. The radius of a simple circular curve is 300 m and length of its specified chord is 30 m. The degree of the curve is

a. 5.73°

b. 5.37°

c. 3.57°

d. 3.75°.

4. Hydrographic surveys deal with the mapping of

a. large water bodies

b. heavenly bodies

c. mountaineous region

d. canal system

5. The normal curing period for lime mortar, is:

a. 3 days

b. 7 days

c. 10 days

d. 14 days.

6. Pick up the metal refractory from the following:

a. Molyblendum

b. Tungsten

c. Zicronium

d. All of these.

7. Cast iron contains carbon approximately

a. 1.5% to 5.5%

b. 0.05% to 1.75%

c. 0.250%

d. none to these.

8. The commonly used thinner in oil paints, is

a. turpentine

b. both (a) and (b)

c. neither (a) nor (b)

d. none the these.

9. The load on a spring per unit deflection, is called

a. stiffness

b. proof resilience

c. proof stress

d. proof load.

10. For determining the support reactions at A and B of a three hinged arch, points B and C are joined and produced to intersect the load line at D and a line parallel to the load line through A at D'. Distances AD, DD' and AD' when measured were 4 cm, 3 cm and 5 cm respectively. The angle between the reactions at A and B is

a. 30°

b. 45°

c. 60°

d. 90°

11. The shape of the bending moment diagram over the length of a beam, having no external load, is always

a. linear

b. parabolic

c. cubical

d. circular.

12. A short masonry pillar is 60 cm x 60 cm in cross-section, the core of the pillar is a square whose side is

a. 14.14 cm

b. 20.00 cm

c. 22.36 cm

d. 25.22 cm.

13. The metacentric height of a body equals the distance between

a. the centre of gravity and centre of buoyancy

b. the metacentre and centre of gravity

c. the centre of buoyancy and metacentre

d. none of these.

14. The thickness of a sharp crested weir is kept less than

a. one-half of the height of water on the sill

b. one-fourth of the height of water on the sill

c. two-third of the height of water on the sill

d. none of these.

15. A fluid particle may possess the displacement of

a. translation

b. rotation

c. distortion

d. all the above.

16. With a clinging nappe of a weir, the excess discharge, is

a. 8% to 10%

b. 18% to 20%

c. 25% to 30%

d. 30% to 40%.

17. In a liquid limit test, the moisture content at 10 blows was 70% and that at 100 blows was 20%. The liquid limit of the soil, is

a. 35%

b. 50%

c. 65%

d. none of these.

18. The critical exist gradient of seepage water in soils, is

a. inversely proportional to the specific gravity

b. directly proportional to the specific gravity

c. inversely proportional to the voids ratio

d. none of these.

19. Pick up the correct statement from the following:

a. In hydrometer method, weight Wd per ml of suspension is found directly

b. In pipette analysis, weight Wd per ml of suspension is found indirectly

c. In pipette analysis, weight Wd per ml of suspension is found directly

d. None of these.

20. The reduction in volume of soil due to squeezing out of water from the voids, is termed

a. primary consolidation

b. primary compression

c. primary time effect

d. all the above.

21. The stem of a cantilever retaining wall which retains earth level with top is 6 m. If the angle of repose and weight of the soil per cubic metre are 30° and 2000 kg respectively, the effective width of the stem at the bottom, is

a. 52.5

b. 53.5

c. 54.5

d. 55.5

22. Enlarged head of a supporting column of a flat slabs is technically .known as

a. top of the column

b. capital

c. drop panel

d. none of these.

23. The width of the flange of a T-beam should be less than

a. one-third of the effective span of the T-beam

b. distance between the centres of T-beam

c. breadth of the rib plus twelve times the thickness of the slab

d. least of the above.

24. In a combined footing if shear stress does not exceed 5 kg/cm2, the nominal stirrups provided are

a. 8 legged

b. 10 legged

c. 12 legged

d. none of these.

25. Pile foundation is generally provided if soil is

a. compressible

b. water logged

c. made up

d. all the above.

26. Pick up the correct statement from the following :

a. D.P.C. should be continuous

b. D.P.C.   should  be  of good  impervious material

c. D.P.C. may be horizontal or vertical

d. All the above.

27. On a grading curve, the gap grading is represented by

a. a horizontal line

b. a vertical line

c. N.W. inclined line

d. N.E. inclined line

28. If fineness modulus of sand is 2.5, it is graded as

a. fine sand

b. medium sand

c. coarse sand

d. very coarse sand.

29. A city supply includes

a. industrial and commercial water demands

b. demand for public uses and fire

c. water losses

d. all the above.

30. Aqueducts are generally designed

a. circular

b. rectangular

c. horse shoe section

d. all the above.

31. The non-clog pump which permits solid matter to pass out with the liquid sewage, is

a. centrifugal pump

b. reciprocating pump

c. pneumatic ejector

d. none of these.

32. The ratio of the diameter of a circular section and the side of a square section hydraulically equivalent, is

a. 1.095

b. 1.085

c. 1.075

d. 1.065

33. Pick up the correct statement from the following

a. In a level crossing, a crest with its top at the canal F.S.L. is provided across the drainage at its up-stream junction with canal

b. In a level crossing a regulator is provided across the drainage at its down-stream

c. In a level crossing, a cross regulator is provided on the canal below the crossing

d. All the above.

34. If the irrigation efficiency is 80%, conveyance losses are 20% and the actual depth of watering is 16 cm, the depth of water required at the canal outlet, is

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm

35. The optimum depth of kor watering for a rice crop, is

a. 19.0 cm

b. 17.5 cm

c. 13.5 cm

d. 12.0 cm

36. In rigid module, the discharge

a. is independent of water levels in the distributary and water course

b. depends upon the water level in distributary

c. depends upon the water level in the water course

d. depends upon the water levels of both, i.e. distributary and water course

37. Extra widening required at a horizontal curve on a single lane hill road of radius 80 m for a design speed of 50 km ph and for a vehicle with wheel base 6.0 m is

a. 0.225 m

b. 0.589 m

c. 1.250 m

d. none of these

38. If the width of a pavement slab is 7.5 m, thickness 20 cm and working stress 1400 kg/cm2, spacing of 10 mm tie bars for the longitudinal joint, is

a. 20 cm

b. 30 cm

c. 40 cm

d. 50 cm

39. Minimum thickness of the base of a flexible pavement, is

a. 10 cm

b. 15 cm

c. 20 cm

d. 25 cm

40. Set-back distance is the distance between

a. road land boundary and building line

b. road land boundary and control line

c. building line and control line

d. road land boundary and control line.

41. Pick up the correct statement from the following:

a. Cement is paid per 50 kg bag

b. Lime is paid per quintal

c. Brick aggregates is paid per cum

d. All the above.

42. Pick up the correct statement from the following:

a. The bent up bars at a support resist the negative bending moment

b. The bent up bars at a support resist the sharing force

c. The bending of bars near supports is generally at 45°

d. All the above.

43. In the mid-section formula

a. The area of mid-sections is calculated by using mean depth

b. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two sections

c. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two original sections

d. (a), (b) and (c) of the above.

44. Pick up the excavation where measurements are made in square metres for payment.

a. surface dressing up to 15 cm depths

b. Surface excavation up to 30 cm depths

c. Both (b) and (c)

d. Both (a) and (b)

45. The main principle of an organisation, is

a. cohegency

b. effective control at all levels

c. delegation of authority

d. all the above.

46. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

47. Which one of the following represents an event ?

a. concrete cured

b. fixing of door

c. plastering of walls

d. selecting sites

48. While scheduling a project by C.P.M.

a. required time for each activity is established

b. sequence of various activities is made according to their importance

c. net work is drawn by connecting the activities and the events

d. All the above.

49. According to I.C.A.O. all markings on the runways are painted white and on taxiways

a. black

b. red

c. yellow

d. green

50. The thickness design of the pavement, is decided on the load carried by

a. main gears

b. nose wheel

c. tail wheel

d. all the above.