***Sub Engineer***

1. Surveys which are carried out to provide a national grid of control for preparation of accurate maps of large areas, are known

a. plane surveys

b. geodetic surveys

c. geographical surveys

d. topographical surveys.

2. The intercept of a staff

a. is maximum if the staff is held truly normal to the line of sight.

b. is minimum if the staff is held truly normal to the line of sight.

c. decreases if the staff is tilted away from normal

d. increases if the staff is tilted towards normal.

3. The main plate of a transit is divided into 1080 equal divisions. 60 divisions of the vernier coincide exactly with 59 divisions of the main plate. The transit can read angles accurate upto

a. 10"

b. 15"

c. 20"

d. 30"

4. The limiting length of an offset does not depend upon

a. accuracy of the work

b. method of setting out perpendiculars

c. scale of plotting

d. indefinite features to be surveyed.

5. Pick up the correct statement from the following:

a. The low voltage porcelain is prepared by wet process

b. The high voltage porcelain is prepared by dry process

c. The low voltage porcelain is prepared by dry process

d. None of the above.

6. PVC stands for

a. plastic very compact

b. polythene vinyl chloride

c. polythene vinyl carbon

d. polythene vanadium carbide.

7. Pick up the correct statement from the following:

a. Horn blende mineral is brittle

b. Muscovite is also known as white mica and potashmica

c. Biotite is also known as blackmica

d. All the above.

8. The colour of statuary marble used for sculptor's work, is

a. blue

b. white

c. green

d. yellow

9. A compound bar consists of two bars of equal length. Steel bar cross-section is 3500 mm2 and that of brass bar is 3000 mm2. These are subjected to a compressive load 100,000 N. If Eb = 0.2 MN/mm2 and Eb = 0.1 MN/mm2, the stresses developed are:

a. σb = 10 N/mm2, σs = 20 N/mm2

b. σb = 8 N/mm2, σs = 16 N/mm2

c. σb = 6 N/mm2, σs = 12 N/mm2

d. σb = 5 N/mm2, σs = 10 N/mm2

10. A material is said to be perfectly elastic if

a. it regains its original shape on removal of the load

b. It regains its original shape partially on removal of the load

c. it does not regain its original shape at all

d. none of these.

11. According to Unwin's formula, the diameter d of a rivet of plate of thickness t is :

a. d = 6.05 t

b. d = 1.5 t + 4

c. d = 5 t

d. d = t + 1.5

12. The ratio of the effective length of a column and minimum radius of gyration of its cross-sectional area, is known

a. buckling factor

b. slenderness ratio

c. crippling factor

d. none of these.

13. In order to avoid capillary correction, the minimum diameter of a manometer used for measuring pressure, should be

a. 4 mm

b. 6 mm

c. 8 mm

d. 10 mm.

14. To avoid the force of surface tension in an inclined manometer, the minimum angle of inclination is

a. 2°

b. 3°

c. 4°

d. 5°

15. Manning's formula is used for

a. flow in open channels

b. head loss due to friction in open channels

c. head loss due to friction in pipes flowing full

d. flow in pipes.

16. If u, v, w are the components of the velocity v of a moving particle, the equation  represents

a. one diamensional flow

b. two dimensional flow

c. three dimensional flow

d. none of these.

17. The minimum centre to centre distance of friction piles of 1 m diameter, is

a. 2 m

b. 2 m to 3 m

c. 3 m to 4 m

d. 5 m

18. Pick up the correct statement from the following:

a. A soil classification system is meant to provide an accepted and systematic method of describing the various types of soils eliminating personal factors

b. The first category of soil classification is based on grain size of the soil

c. The second category of soil classification is based on fine as well as coarse grains

d. All the above.

19. The consistency index of a soil is defined as the ratio of

a. liquid limit plus the natural water content to the plasticity index of the soil

b. liquid limit minus the natural water content to the plasticity index of the soil

c. natural water content of a soil minus plastic limit to the plasticity index of the soil

d. natural water content of a soil plus its plastic limit to the plasticity index of the soil.

20. Pick up the correct statement from the following:

a. The ratio of the liquid limit minus the natural water content to the plasticity index of soils, is called consistency index

b. The ratio of natural water content minus its plastic limit to its plasticity index is called liquidity index

c. The ratio between plasticity index and flow index (i.e. slope of flow curve in case of liquid limit), is called toughness index

d. All the above.

21. A foundation is called shallow if its depth, is

a. half of its width

b. three-fourth of its width

c. equal to its width

d. all the above.

22. An R.C.C. beam of 25 cm width and 50 cm effective depth has a clear span of 6 metres and carries a U.D.L. of 3000 kg/m inclusive of its self weight. If the lever arm constant for the section is 0.865, the maximum intensity of shear stress, is

a. 8.3 kg/cm2

b. 7.6 kg/cm2

c. 21.5 kg/cm2

d. 11.4 kg/cm2

23. Side face reinforcement shall be provided in the beam when depth of the web in a beam exceeds

a. 50 cm

b. 75 cm

c. 100 cm

d. 120 cm

24. In a cantilever retaining wall without a heel slab

a. thickness of the stem is kept same throughout

b. base slab is made 10 cm thicker than the stem

c. width of the base slab is kept 0.7 time the total height of the wall

d. all the above.

25. Open test pit is only suitable upto a depth of

a. 2 metres

b. 2.5 metres

c. 3 metres

d. none of the above.

26. In the method of tube boring of soil investigation, the following is essential :

a. a flap valve at the bottom of tube is provided to extract the soil sample

b. the tube is raised and lowered by 4 thick rope moving over a pulley suspended on a tripod stand

c. the tube is dropped to fall under gravity in side a metalic casing pipe which is driven as the depth of excavation proceeds.

d. all the above.

27. The aggregate containing moisture in pores and having its suface dry, is known as

a. very dry aggregates

b. dry aggregates

c. saturated surface dry aggregate

d. none of these.

28. For the construction of cement concrete floor, the maximum permissible size of aggregate, is

a. 6 mm

b. 8 mm

c. 10 mm

d. 12 mm

29. The least thickness of class B cast iron (spun) pipe, is

a. 7.2 mm

b. 7.9 mm

c. 8.6 mm

d. 10 mm.

30. Pick up the correct statement from the following :

a. Water with hardness more than 200 ppm is considered hard

b. Water needed for laundaries should contain hardness less than 75 ppm

c. To provide taste during drinking, water should possess hardness between 75 to 120

d. All the above.

31. Fresh sewage may become stale in

a. two to three hours

b. three to four hours

c. six hours.

d. ten hours.

32. A manhole is generally provided at each

a. junction

b. change of gradient

c. change of sewer diameter

d. all the above.

33. The width of a dowla is generally kept between 30 to 60 cm and its height above the road level should invariably be more than

a. 10 cm

b. 20 cm

c. 30 cm

d. 40 cm

34. Side slopes generally kept for canals in loam soil, are :

a. 1 : 1 in cutting and  : 1 in filling

b.  : 1 in cutting and 2 : 1 in filling

c. Neither (a) nor (b)

d. Both (a) and (b)

35. Garnett's diagrams are used for graphical solution of design equations of a canal by

a. Lacey's theory

b. Kennedy's theory

c. Gibb's theory

d. Lindlay theory.

36. For a given discharge in a channel, Blench curves give the relationship between the loss of head (HL) and

a. specific energy up-stream

b. specific energy down-stream

c. critical depth of water down-stream

d. depth of water down-stream

37. Reconnaissance is best done with the help of

a. aerial photographic survey

b. condastral surveys

c. topographical surveys

d. triangulation surveys

38. The wall constructed for the stability of a back filling portion of a road on the down hill side, is known as

a. retaining wall

b. breast wall

c. parapet wall

d. all the above.

39. Parapet walls along hill roads, are provided

a. to retain the back filling

b. to prevent the hill from sliding

c. to prevent the wheels of the vehicle from coming on the retaining wall

d. none of these.

40. Stability of hill slopes depends upon

a. angle of the slope

b. geological conditions

c. ground water conditions

d. all the above.

41. The item of steel work which is measured in sq.m, is

a. Rolling shutters

b. Steel doors

c. Ventilators and glazing.

d. All the above.

42. The area of a sloping surface of a protective embankment of mean height d, side slopes S : 1 and length L is

a. d x d x s

b. d2 x (ds)2

c. L.D 1 + s2

d. 2 LD 1 + s2

43. The measurement is not made in square metres in case of

a. D.P.C. (Damp proof course)

b. Form works

c. Concrete Jaffries

d. R.C. Chhajja.

44. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

45. Which one of the following represents an event ?

a. concrete cured

b. fixing of door

c. plastering of walls

d. selecting sites

46. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

47. In CPM analysis,

a. uncertainties are not allowed

b. activities are represented by arrows

c. beginning and end of an activity, are denoted by nodes

d. all the above.

48. The main principle of an organisation, is

a. cohegency

b. effective control at all levels

c. delegation of authority

d. all the above.

49. The coordinates of the ends of a runway are (5000, 5000) and (8000, 7000). The co-ordinates of an other runway are (4600, 5100) and (7000, 5300). The co-ordinates of the A.R.P. are

a. (6500, 6000)

b. (5800, 5200)

c. (61500, 5600)

d. (8000, 7000)

50. For the proposed air port, the survey project provides

a. topograhic plan

b. grading plan

c. drainage plan

d. all the above.