***Sub Engineer***

1. The bearings of two traverse legs AB and BC are N52° 45' E and N34° 30' E respectively. The deflection angle is

a. 18° 15' N

b. 18° 15' W

c. 18° 15' R

d. 18° 15' L

2. The properties of autogenous curve for automobiles are given by

a. true spiral

b. cubic parabola

c. Bernoulli's Lemniscate

d. clothoid spiral.

3. While measuring the distance between two points along upgrade with the help of a 20 m chain, the forward end of the chain is shifted forward through a distance

a. 20 (sin θ - 1)

b. 20 (cos θ - 1)

c. 20 (sec θ - 1)

d. 20 (cosec θ - 1).

4. Pick up the correct statement from the following

a. the contour lines having the same elevation cannot unite and continue as one line

b. a contour can not end abruptly, but must ultimately close itself not necessarily within the limits of map.

c. the direction of steepest slope at a point on a contour is at right angles to the contour

d. all the above.

5. The size of mould for bricks, is generally kept

a. a little large to specified size

b. a little small to specified size

c. equal to specified size

d. 10% larger than specified size

6. Pig iron made from heamatite ores free from sulphur, phosphorus and copper, is known as

a. Bessemer pig

b. Grey or foundry pig

c. White or forge pig

d. Mottled pig

7. For light and ornamental casting, the most unsuitable pig iron, is

a. Bessemer pig

b. Grey or foundry pig

c. White or forge pig

d. Mottled pig.

8. Quick setting cement is produced by adding

a. more amount of gypsum in very fine powdered form

b. aluminium sulphate in very fine powdered form

c. pozzolana in very fine powdered form

d. none of these.

9. The ratio of tangential and normal components of a stress on an inclined plane through θ° to the direction of the force, is :

a. cos θ

b. tan θ

c. cos θ

d. sec θ

10. For beams of uniform strength, if depth is constant,

a. width b ∝ M

b. width b ∝ M

c. width b ∝ 3 M

d. width b ∝

11. For keeping the stress wholly compressive the load may be applied on a circular column anywhere within a concentric circle of diameter

a. d/3

b. d/4

c. d/8

d. d/10

12. According to Unwin's formula, the diameter d of a rivet of plate of thickness t is :

a. d = 6.05 t

b. d = 1.5 t + 4

c. d = 5 t

d. d = t + 1.5

13. Pick up the correct statement from the following :

a. For tranquil flow h > 2g or v <

b. For critical flow

c. For torrential flow h >  or v < gh

d. None of these.

14. Total energy line is

a. pressure head

b. datum head

c. kinetic head

d. All the above.

15. In an open tube, free surface of mercury remains

a. horizontal

b. curved upwards

c. curved downwards

d. none of these.

16. If a pitot tube is placed with its nose facing down stream, the liquid

a. does not rise in the tube

b. rises in the tube to a height

c. falls in the tube to a depth

d. none of these.

17. During seepage through a soil, direction of seepage is always

a. parallel to equipotential lines

b. perpendicular to stream lines

c. perpendicular to equipotential lines

d. none of these.

18. A soil mass coated with a thin layer of paraffin weighs 460 g. When immersed, it displaces 299 cc of water. The weight of paraffin is 10 g. If specific gravity of solids is 2.5 and that of paraffin 0.9, the void ratio of soil, is

a. 0.55

b. 0.60

c. 0.65

d. 0.70

19. Over-consolidation of soils is caused due to

a. erosion of over burden

b. melting of ice sheets after glaciation

c. permanent rise of water table

d. all the above.

20. In non-cohesive soil in passive state of plastic equilibrium

a. major principal stress is horizontal

b. minor principal stress is vertical

c. major principal stress is vertical

d. minor and major principal stresses are equally inclined to the horizontal.

21. If H is the overall height of a retaining wall retaining a surcharge, the width of the base slab usually provided, is

a. 0.4 H

b. 0.5 H

c. 0.6 H

d. 0.7 H

22. If permissible working stresses in steel and concrete are respectively 1400 kg/cm2 and 80 kg/cm2 and modular ratio is 18, in a beam reinforced in tension side and of width 30 cm and having effective depth 46 cm, the lever arms of the section, is

a. 37 cm

b. 38 cm

c. 39 cm

d. 40 cm

23. For initial estimate for a beam design, the width is assumed

a. 1/20th of span

b. 1/25th of span

c. 1/30th of span

d. 1/40th of span.

24. The minimum thickness of the cover at the end of a reinforcing bar should not be less than twice the diameter of the bar subject to a minimum of

a. 15 mm

b. 20 mm

c. 25 mm

d. 30 mm

25. The depth of the ground water table may be ascertained by

a. looking through the well in the vicinity

b. standing on the well in the vicinity

c. measuring the depth of water in the well

d. none of the above.

26. In horizontal D.P.C, thickness of cement concrete (1 : 2 : 4) is

a. 4 cm

b. 6 cm

c. 8 cm

d. 10 cm.

27. For quality control of Portland cement, the test essentially done is

a. soundness

b. tensile strength

c. consistency

d. all the above.

28. Separation of water or water sand cement from a freshly concrete, is known

a. bleeding

b. creeping

c. segregation

d. flooding

29. The type of joint generally used in cast iron pipes, is

a. flanged joint

b. dresser coupling joint

c. flexible joint

d. all the above.

30. Pick up the correct statement from the following :

a. The difference between water table level and the water level in a well after pumping, is called depression head

b. The surface of water table surrounding a well during pumping, forms a cone of depression

c. The distance from the centre of the well up to the place where level of water table is not affected, is called radius of influence

d. All the above.

31. In the activated sludge process

a. aeration is continued till stability

b. aeration is done with an admixture of previously aerated sludge

c. sludge is activated by constant stirring

d. water is removed by centrifugal action.

32. In areas where rainy season is limited to a few months, the type of sewerage system recommended is

a. combined system

b. partially separate system

c. separate system

d. none of these.

33. Pick up the correct statement from the following:

a. Gravity water is harmful to crops

b. Hygroscopic water remains attached to soil molecules by chemical bond

c. Capillary moisture held in the soil pores against gravity by surface tension, is utilised by plants

d. All the above.

34. The useful moisture of soil, is equal to its

a. field capacity

b. saturation capacity

c. moisture content at permanent wilting point

d. difference between filed capacity and permanent wilting point within the root zone of plants.

35. When a canal is carried over a natural drainage, the structure provided, is known as

a. syphon

b. aqueduct

c. super passage

d. syphon-aqueduct.

36. A current meter measures the velocity of flow, if it is held

a. on the surface of water

b. at the bottom of channel

c. at any point within the cross-section

d. none of these.

37. While designing hair pin bends on higways, the minimum

a. gradient is 1 in 40

b. gradient is 1 in 200

c. superelevations is 1 in 10

d. All the above.

38. If cross slope of a country is upto 10% the terain is classified as

a. plain

b. rolling

c. mountainous

d. steep.

39. If L is the length of the transition curves provided on either side of a circular curve of radius R, the maximum angle of deflection with tangent for the junctions of the transition curve and circular curve, is

a. L/2R

b. L/3R

c. L/4R

d. none of these.

40. The convexity provided to the carriageway between the crown and edge of the pavement, is known as

a. super-elevation

b. camber

c. height of the pavement

d. None of these.

41. Anti-siphonage pipe is connected to

a. Main soil pipe

b. Bottom of P trap W.C.

c. Top of P trap W.C.

d. Side of water closet.

42. Pick up the correct statement from the following:

a. In order to check up the average depth of excavation, 'Dead mans' are left at the mid-widths of borrow pits

b. The earthwork calculation in excavation is made from the difference in levels obtained with a level

c. The earth work in excavation to form the road embankment includes the formation of correct profile and depositing the soil in layers

d. All the above.

43. Pick up the incorrect statement regarding a master trap from the following :

a. It is provided a cleaning eye at the top of the trap

b. The height of fresh air inlet pipe fixed vertically with wall is 3 m

c. The mica flap valve which opens inwards only, is fitted at the top of the inlet pipe

d. The water seal is less than that of ordinary traps.

44. The 'centre line method' is specially adopted for estimating

a. Hexagonal buildings

b. Octagonal buildings

c. Other geometrical shaped buildings

d. All the above.

45. The most popular type of organisation used for Civil Engineering Constructions, is

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

46. If D is the duration, ES and EF are the earliest start and finish, LS and LF are latest start and latest finish time, then the following relation holds good

a. LS = LF - D

b. LF = LS + D

c. D = EF - ES

d. all the above.

47. Pick up the incorrect statement from the following:

a. The tail of the arrow indicates the start of the activity

b. The head of the arrow indicates the end of the activity

c. The arrows are drawn to scale from left to right

d. Each activity consumes a given time.

48. The estimated time required to perform an activity, is known as

a. event

b. dummy

c. duration

d. float.

49. If lift off distance of an craft is 2000 m, the clear way at the end of the runway should not be less than

a. 145 m

b. 152.5 m

c. 162.5 m

d. 172.5 m

50. Airport elevation is the reduced level above M.S.L. of

a. control tower

b. highest point of the landing area

c. lowest point of the landing area

d. none of these.