***Sub Engineer***

1. The desired sensitivity of a bubble tube with 2 mm divisions is 30". The radius of the bubble tube should be

a. 13.75 m

b. 3.44 m

c. 1375 m

d. none of these.

2. In levelling operation

a. if first reading is more than second, it represents a rise

b. if  first reading is less than second, it represents a fall

c. if  second reading is less than first, it represents a fall

d. both (b) and (c).

3. In reciprocal levelling, the error which is not completely eliminated, is due to

a. earth's curvature

b. non-adjustment of line of collimation

c. refraction

d. non-adjustment of the bubble tube.

4. The ratio of the linear displacement at the end of a line, subtended by an arc of one second to the length of the line, is

a. 1:206 300

b. 1:3440

c. 1:57

d. 1:100.

5. The curved swellings from the growth of layers or wounds left after branches are cut off in an irregular manner, are known as

a. knots

b. rindgalls

c. burls

d. none of these.

6. Dextrin is

a. starch glue

b. albumin glue

c. rubber based adhesive

d. none to these.

7. The portion of the brick cut across its width and having its length equal to that of a full brick, is known as

a. queen closer

b. king closer

c. prince closer

d. none of these.

8. The initial setting time of hydraulic lime, is

a. 60 minutes

b. 90 minutes

c. 120 minutes

d. 150 minutes.

9. For determining the support reactions at A and B of a three hinged arch, points B and C are joined and produced to intersect the load line at D and a line parallel to the load line through A at D'. Distances AD, DD' and AD' when measured were 4 cm, 3 cm and 5 cm respectively. The angle between the reactions at A and B is

a. 30°

b. 45°

c. 60°

d. 90°

10. If a solid shaft (diameter 20 cm, length 400 cm, N = 0.8 x 105 N/mm2) when subjected to a twisting moment, produces maximum shear stress of 50 N/mm2, the angle of twist in radians, is

a. 0.002

b. 0.0025

c. 0.003

d. 0.005

11. If the stress in each cross-section of a pillar is equal to its working stress, it is called

a. body of equal

b. body of equal section

c. body of equal strength

d. none of these.

12. The section modulus of a rectangular light beam 25 metres long is 12.500 cm3. The beam is simply supported at its ends and carries a longitudinal axial tensile load of 10 tonnes in addition to a point load of 4 tonnes at the centre. The maximum stress in the bottom most fibre at the mid span section, is

a. 13.33 kg/cm2 compressive

b. 26.67 kg/cm2 tensile

c. 26.67 kg/cm2 compressive

d. none of these.

13. Barometres are used to measure

a. difference in pressure at two points

b. atmospheric pressure

c. very low pressure

d. very high pressure.

14. Orifice-meter is used to measure

a. pressure at the point

b. discharge

c. average speed

d. velocity.

15. The discharge through a 100 mm diameter external mouth piece fitted to the side of a large vessel is 0.05948 m3/s. The head over the mouth piece is

a. 2 m

b. 2.5 m

c. 3.0 m

d. 4.0 m.

16. The pressure less than atmospheric , pressure, is known

a. suction pressure

b. vacuum pressure

c. negative gauge pressure

d. all the above.

17. A suspended particle falls through a height H cm in water in t minutes. If the viscosity of water is η and specific gravity of the particle is G, the diameter of the particle is (where M is a constant factor)

a. 103M

b. 104M

c. 105M

d. 102M

18. A sample of saturated soil has 30% water content and the specific gravity of soil grains is 2.6. The dry density of the soil mass in g/cm3, is

a. 1.47

b. 1.82

c. 1.91

d. none of these.

19. Pick up the correct statement from the following:

a. A soil classification system is meant to provide an accepted and systematic method of describing the various types of soils eliminating personal factors

b. The first category of soil classification is based on grain size of the soil

c. The second category of soil classification is based on fine as well as coarse grains

d. All the above.

20. Factor of safety against sliding of a slope, is the ratio of

a. actual cohesion to that required to maintain stability of slope

b. shear strength to shear stress along the surface

c. neither (a) nor (b)

d. both (a) and (b).

21. The weight of reinforced concrete, is generally taken as

a. 2300 kg/m3

b. 2400 kg/m3

c. 2500 kg/m3

d. 2800 kg/m3

22. For M 150 grade concrete (1:2:4) the moment of resistance factor is

a. 8.50

b. 7.50

c. 5.80

d. none of these.

23. The system in which high tensile alloy steel bars (silica manganese steel) are used as prestressing tendons, is known as

a. Freyssinet system

b. Magnel-Blaton system

c. C.C.L. standard system

d. Lee-McCall system.

24. The effective span of a simply supported slab, is

a. distance between the centres of the bearings

b. clear distance between the inner faces of the walls plus twice the thickness of the wall

c. clear span plus effective depth of the slab

d. none of these.

25. Stability of an existing structure may be disturbed by

a. vibrations caused by traffic movements

b. mining in the neighbourhood

c. excavation in the neighbourhood

d. all the above.

26. For different layers of cement concrete floor. Pick up the incorrect statement from the following :

a. The lowest layer consists of consolidated ground

b. A 10 cm thick clean sand is laid on consolidated ground

c. A 10 cm lime concrete (1 : 4 : 8) is laid on clean sand

d. A 10 cm thick cement concrete (1 : 2 : 4) is laid on top layer.

27. If the engineer-in-charge approves, the 10 cm cubes may be used for the work test of concrete provided maximum nominal size of aggregate, does not exceed

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm

28. The bulk density of aggregates does not depend upon :

a. specific gravity of aggregates

b. grading of aggregates

c. size and shape of the container

d. none of these.

29. If average volume of sediment deposits is one-tenth million cubic metres per year in a reservoir of total capacity 10 million cubic metres, the dead storage will be filled up, in

a. 15 years

b. 20 years

c. 25 years

d. 50 years.

30. The pH value of water fit for drinking, is

a. 11

b. 9

c. 7

d. 5

31. Pick up the in-correct statement from the following :

a. Manholes are provided in sewer pipes at suitable intervals

b. Catch basins are generally provided in sewers for carrying drainage discharge

c. Inlets are generally provided in all sewers

d. None of these.

32. Pick up the correct statement from the following :

a. Inlets are generally provided at an interval of 30 m to 60 m along straight roads

b. Inlets having vertical openings, are called curb inlets

c. Inlets having horizontal openings, are called horizontal inlets

d. All the above.

33. The optimum depth of kor watering for a rice crop, is

a. 19.0 cm

b. 17.5 cm

c. 13.5 cm

d. 12.0 cm

34. A deficit of sediments in flowing water may cause a river

a. meandering type

b. aggrading type

c. degrading type

d. sub-critical type.

35. For a given discharge in a channel, Blench curves give the relationship between the loss of head (HL) and

a. specific energy up-stream

b. specific energy down-stream

c. critical depth of water down-stream

d. depth of water down-stream

36. If d1 is the depth of cutting, d2 is the height of the bank from bed level r2 : 1 and r1 : 1 are the slopes in filling and cutting respectively, the horizontal distance n between the bed and bank, is

a. x = r1 d1

b. x = r2 d2

c. x = d1 / r1

d. x = d2 / r2

37. The steepest gradient permitted on roads which, in ordinary conditions, does not exceed, is known

a. maximum gradient

b. exceptional gradient

c. floating gradient

d. all the above.

38. Design of flexible pavements is based on

a. mathematical analysis

b. empirical formulae

c. a compromise of pure theory and pure empirical formula

d. none of these.

39. On concrete roads, the camber generally provided, is

a. 1 in 30 to 1 in 48

b. 1 in 36 to 1 in 48

c. 1 in 48 to 1 in 60

d. 1 in 60 to 1 in 72

40. If the designed speed on a circular curve of radius 1400 m is 80 km/hour, no superelevation is provided, if the camber, is

a. 4%

b. 3%

c. 2%

d. 1.7%

41. Pick up the correct statement from the following:

a. The bent up bars at a support resist the negative bending moment

b. The bent up bars at a support resist the sharing force

c. The bending of bars near supports is generally at 45°

d. All the above.

42. The trap which is provided to disconnect the house drain from the street sewer is called

a. Intercepting trap

b. Interception manhole

c. Intercepterchamber

d. All the above.

43. The expected out turn of brick work in cement mortar in foundation and plinth per mason per day, is

a. 1.00 m3

b. 1.25 m3

c. 1.50 m3

d. 1.75 m3

44. Berms are provided in canals if these are

a. fully in excavation

b. partly in excavtion and partly in embankment

c. fully in embankment

d. All the above.

45. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

46. The salient feature of functional organisation is

a. separation of planning and design part

b. each individual maintains functional efficiency

c. work is properly planned and distributed

d. all the above.

47. A construction schedule is prepared after collecting

a. output of labour

b. output of machinery

c. quantity of various items

d. all the above.

48. Bar charts are suitable for

a. minor works

b. major works

c. large projects

d. all the Above.

49. Pick up the correct statement from the following:

a. Approach areas are measured in horizontal surfaces

b. The imaginary inclined plane which is directly above the approach area is called approach surface

c. Obstruction clearance surface and approach surface are same

d. All the above.

50. The coordinates of the ends of a runway are (5000, 5000) and (8000, 7000). The co-ordinates of an other runway are (4600, 5100) and (7000, 5300). The co-ordinates of the A.R.P. are

a. (6500, 6000)

b. (5800, 5200)

c. (61500, 5600)

d. (8000, 7000)