***Sub Engineer***

1. The main principle of surveying is to work

a. from part to the whole

b. from whole to the part

c. from higher level to the lower level

d. from lower level to higher level.

2. The 'fix' of a plane table station with three known points, is bad if the plane table station lies

a. in the great triangle

b. outside the great triangle

c. on the circumference of the circumscribing circle

d. none of these.

3. In precision theodolite traverse if included angles are read twice and the mean reading accepted using both verniers having a least count of 30". Assuming the instrument to be in perfect adjustment, linear measurements correct to 6 mm per 30 metre tape duly corrected for temperature, slope and sag, the angular error of closure not to exceed (where n is the number of traverse legs)

a. 50" n

b. 30" n

c. 60" n.

4. Which one of the following mistakes/errors may be cumulative + or - :

a. bad ranging

b. bad straightening

c. erroneous length of chain

d. sag.

5. To give a brilliant finish, the type of varnish used, is

a. spirit varnish

b. turpentine varnish

c. oil varnish

d. none of these.

6. Pick up the correct statement from the following:

a. The articles prepared from clay which is burnt at low temperature and cooled down slowly, are called earthen-ware

b. The articles prepared from refractory clays which as mixed with stone and crushed pottery, are called stone ware

c. The articles prepared from fine earthen ware which is white, thin and semi-transparent, are called procelain

d. All the above.

7. Basalt is

a. sedimentary rock

b. metamorphic rock

c. extrusive igneous rock

d. intrusive igneous rock.

8. Pick up the correct statement from the following:

a. The commercial plywoods are available upto 150 cm wide and upto 300 cm long

b. The plywoods possess uniform tensile strength in all directions

c. The plywoods are light in weight

d. All the above.

9. For a strongest rectangular beam cut from a circular log, the ratio of the width and depth, is

a. 0.404

b. 0.505

c. 0.606

d. 0.707

10.  is the equation of Euler's crippling load, if

a. both the ends are fixed

b. both the ends are hinged

c. one end is fixed and other end is free

d. one end is fixed and other end is hinged.

11. The maximum compressive stress at the top of a beam is 1600 kg/cm2 and the corresponding tensile stress at its bottom is 400 kg/cm2 . If the depth of the beam is 10 cm, the neutral axis from the top, is

a. 4 cm

b. 6 cm

c. 8 cm

d. 10 cm.

12. A shaft turning 150 r.p.m. is subjected to a torque of 150 kgm. Horse power transmitted by the shaft is

a. π

b. 10 π

c. π2

d. 1/π

13. If velocities of fluid particles vary from point to point in magnitude and direction, as well as from instant to instant, the flow is said to be

a. laminar

b. turbulent flow

c. uniform flow

d. non-uniform flow.

14. If the atmospheric pressure on the surface of an oil tank (sp. gr. 0.8) is 0.1 kg/cm2, the pressure at a depth of 2.5 m, is

a. 2 metres of water

b. 3 metres of water

c. 3.5 metres of water

d. 4.0 metres of water.

15. A piezometer opening in pipes measures

a. velocity head

b. static pressure

c. total pressure

d. negative static pressure.

16. For critical flow, the Froude number is :

a. 1.0

b. less than 1.0

c. more than 1.0

d. 2

17. A partially saturated sample of soil has a unit weight of 2.0 g/cm3 and specific gravity of soil particles is 2.6. If the moisture content in the soil is 20%, the degree of saturation is

a. 20%

b. 77%

c. 92%

d. none of these.

18. The quantity of seepage of water through soils is proportional to

a. coefficient of permeability of soil

b. total head loss through the soil

c. neither (a) nor (b)

d. both (a) and (b).

19. According to Coulomb's wedge theory, the active earth pressure slides the wedge

a. down and outwards on a slip surface

b. up and inwards on a slip surface

c. horizontal upward and parallel to base

d. horizontal inward and parallel to base.

20. The slip at critical angle, is generally known

a. δ 1-line

b. rupture plane

c. slip plane

d. all the above.

21. The live load to be considered for an inaccessible roof, is

a. Nil

b. 75 kg/m2

c. 150 kg/cm2

d. 200 kg/m2

22. The toe projection of foundation slabs is taken

a. as one third of the base

b. as one sixth of overall height of the wall

c. equal to heel slab

d. below ground surface.

23. If the bearing capacity of soil is 10 tonnes/cm2 and the projection of plain concrete footing from walls, is a cm, the depth D of footing is

a. D = 0.0775 a

b. D = 0.775 a

c. D = 0.775a

d. D = 0.775 a2

24. The live load to be considered for an accessible roof, is

a. Nil

b. 75 kg/m3

c. 150 kg/m2

d. 200 kg/cm2

25. Bearing capacity of soils cannot be improved by

a. ramming crushed stone in soil

b. driving sand piles

c. watering surface of soil

d. none of these.

26. Stud(s) of a common wooden partition

a. are vertical wooden members

b. is the upper horizontal wooden member

c. is the lower horizontal wooden member

d. are the intermediate horizontal wooden members.

27. Pick up the correct statement from the following:

a. Continuous grading is not necessary for obtaining a minimum of air voids

b. The omission of a certain size of aggregate is shown by a straight horizontal line on the grading curve

c. The omission of a certain size of aggregate in concrete increases the workability but also increases the liability to segregation

d. All the above.

28. An aggregate is said to be flaky, if its least dimension is less than

a. 3/4 mean dimension

b. 3/5 mean dimension

c. 5/8 mean dimension

d. none of these.

29. In a well planned city, the layout of distribution pipes generally adopted, is

a. grid-iron system

b. interlaced system

c. reticulation system

d. all the above.

30. A centrifugal pump is required to be primed before starting if it is located

a. at higher level than water level of reservoir

b. at lower level than water level of reservoir

c. both (a) and (b)

d. neither (a) nor (b).

31. In a trickling filter

a. filtration process is used

b. biological action is used

c. neither (a) nor (b)

d. both (a) and (b).

32. In septic tanks, decomposition of organic bacteria, is done by

a. anaerobic bacteria

b. aerobic bacteria

c. both types of bacterias

d. none of these.

33. Disposal of extra excavated earth of canals, is utilised to provide a spoil bank on

a. left side

b. right side

c. both sides

d. all the above.

34. Lane's weighted creep theory assumes

a. double weightage to horizontal creep and one weightage to vertical creep

b. triple weightage to horizontal creep and one weightage to vertical creep

c. triple weightage to vertical creep and one weightage to horizontal creep

d. double weightage to vertical creep and one weightage to horizontal creep.

35. The uplift pressure on the roof of an inverted syphon, is maximum when

a. drain in running dry

b. canal is running dry

c. canal is running with F.S.L.

d. drain is running with H.F.L.

36. Garnett's diagrams are used for graphical solution of design equations of a canal by

a. Lacey's theory

b. Kennedy's theory

c. Gibb's theory

d. Lindlay theory.

37. On earth roads, the camber should preferably be

a. 1 in 20 to 1 in 24

b. 1 in 30 to 1 in 48

c. 1 in 15 to 1 in 20

d. 1 in 10 to 1 in 15

38. If cross slope of a country is greater than 60%, the terrain is classified as

a. rolling

b. mountainous

c. steep

d. plain.

39. Maximum number of vehicles that can pass a given point on a lane during one hour without creating unreasonable delay, is known as

a. traffic density of lane

b. basic capacity of lane

c. probable capacity of lane

d. practical capacity of lane.

40. To ensure that bullock carts may not overturn on curves, the maximum value of super-elevation, recommended by I.R.C., is

a. 1 in 12

b. 1 in 15

c. 1 in 20

d. 1 in 25

41. While estimating a reinforced cement structure, the omitted cover of concrete is assumed

a. at the end of reminforcing bar, not less than 25 mm or twice the diameter of the bar

b. in thin slabs, 12 mm minimum or diameter of the bar whichever is more

c. for reinforcing longitudinal bar in a beam 25 mm minimum or diameter of the largest bar which is more

d. All the above.

42. The area of a sloping surface of a protective embankment of mean height d, side slopes S : 1 and length L is

a. d x d x s

b. d2 x (ds)2

c. L.D 1 + s2

d. 2 LD 1 + s2

43. The measurement is made for stone work in square metre in case of

a. Columns, lintels, copings

b. Building work

c. Dressed stones in Chajjas

d. (a) and (d) of the above.

44. Brick walls are measured in sq. m if the thickness of the wall is

a. 10 cm

b. 15 cm

c. 20 cm

d. None of these.

45. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

46. An Executive Engineer may have powers upto

a. Rs. 25,000

b. Rs. 50,000

c. Rs. 100,000

d. Rs. 200,000

47. Final technical authority of a project lies with

a. Assistant Engineer

b. Executive Engineer

c. Superintending Engineer

d. Chief Engineer.

48. A CPM family includes

a. CPP (Critical Path Plotted)

b. MCE (Minimum Cost Expenditure)

c. CPS (Critical Path Scheduling)

d. All the above.

49. The length of a runway under standard atmospheric conditions is 1800 m. If the actual reduced level of the site is 1200 m, the design length of the runway is

a. 2360 m

b. 2460 m

c. 2560 m

d. 2660 m

50. The thickness design of the pavement, is decided on the load carried by

a. main gears

b. nose wheel

c. tail wheel

d. all the above.