***Sub Engineer***

1. The line of collimation method of reduction of levels, does not provide a check on

a. intermediate sights

b. fore sights

c. back sights

d. reduced levels.

2. In a telescope the object glass of focal length 14 cm, is located at 20 cm from the diaphragm. The focussing lens is midway between them when a staff 16.50 m away is focussed. The focal length of the focussing lens, is

a. 5.24 cm

b. 6.24 cm

c. 7.24 cm

d. 8.24 cm

3. To orient a plane table at a point with two inaccessible points, the method generally adopted, is

a. intersection

b. resection

c. radiation

d. two point problem.

4.  is a conventional sign of

a. mosque

b. temple

c. church

d. idgah.

5. Commonly used thinner in

a. cellulose paints is ethyle acetate

b. oil paints, is naptha

c. distemper, is water

d. all the above.

6. Forge pig may be converted to wrought iron by

a. rolling

b. pudding

c. shingling

d. refining.

7. For preparing porcelains, the clay should be

a. sufficiently pure

b. of high degree of tanacity

c. of good plasticity

d. All the above.

8. The method of addition polymerization is used for obtaining:

a. polypropylene

b. polyvinylchloride

c. polystyrene

d. All of these.

9. A spring of mean radius 40 mm contains 8 action coils of steel (N = 80000 N/mm2), 4 mm in diameter. The clearance between the coils being 1 mm when unloaded, the minimum compressive load to remove the clearance, is

a. 25 N

b. 30 N

c. 35 N

d. 40 N

10. Shear strain energy theory for the failure of a material at elastic limit, is due to

a. Guest or Trecas

b. St. Venant

c. Haig

d. Von Mises.

11. If the shear force along a section of a beam is zero, the bending moment at the section is

a. maximum

b. minimum

c. average of maximum-minimum

d. none of these.

12. The property by which a body returns to its original shape after removal of the force, is called

a. plasticity

b. elasticity

c. ductility

d. malleability.

13. Practical fluids possess

a. viscosity

b. surface tension

c. compressibility

d. all the above.

14. The discharg formula Q = Cd 2gH x A is used for rectangular

a. small orifices only

b. large orifices only

c. small and large orifices only

d. for all types of orifices.

15. Total head of a liquid particle in motion is the sum of

a. potential head and kinetic head

b. kinetic head and pressure head

c. potential head and pressure head

d. potential head, kinetic head and pressure head.

16. Total energy line is

a. pressure head

b. datum head

c. kinetic head

d. All the above.

17. Pick up the correct statement from the following:

a. The ratio of the liquid limit minus the natural water content to the plasticity index of soils, is called consistency index

b. The ratio of natural water content minus its plastic limit to its plasticity index is called liquidity index

c. The ratio between plasticity index and flow index (i.e. slope of flow curve in case of liquid limit), is called toughness index

d. All the above.

18. You are given a sample of soil containing coarse grains to determine its water content, you will use

a. pycnometer

b. oven-drying method

c. calcium carbide method

d. alcohol method.

19. Pick up the correct statement from the following:

a. The freezing of water is accompained by a volume increase of 9%.

b. Below freezing point, higher soil suction develops.

c. The magnitude of frost heave decreases as the degree of saturation of soil decreases

d. All the above.

20. The maximum net pressure intensity causing shear failure of soil, is known

a. safe bearing capacity

b. net safe bearing capacity

c. net ultimate bearing capacity

d. ultimate bearing capacity.

21. If the depth of actual neutral axis of a doubly reinforced beam

a. is greater than the depth of critical neutral axis, the concrete attains its maximum stress earlier

b. is less than the depth of critical neutral axis, the steel in the tensile zone attains its maximum stress earlier

c. is equal to the depth of critical neutral axis, the concrete and steel attain their maximum stresses simultanesouly

d. all the above.

22. The live load to be considered for an inaccessible roof, is

a. Nil

b. 75 kg/m2

c. 150 kg/cm2

d. 200 kg/m2

23. Total pressure on the vertical face of a retaining wall of height h acts parallel to free surface and from the base at a distance of

a. h/4

b. h/3

c. h/2

d. 2h/3

24. The width of the flange of a T-beam, which may be considered to act effectively with the rib depends upon

a. overall thickness of the rib

b. centre to centre distance between T-beams

c. span of the T-beam

d. all the above.

25. The foundation in which a cantilever beam is provided to join two footings, is known as

a. strap footing

b. combined footing

c. raft footing

d. none of these.

26. The minimum distance between the centres of bulb of diameter du, of a multi under reamed piles, is

a. 1.25 du

b. 1.5 du

c. 1.75 du

d. 2 du.

27. For given workability the grading requiring the least amount of water is one that gives

a. greatest surface area for the given cement and aggregates

b. least surface area for the given cement and aggregates

c. least weight for the given cement and aggregates

d. greatest weight for the given cement and aggregates

28. Concrete is unsuitable for compaction by a vibrator if it is

a. earth moist

b. semi-plastic

c. plastic

d. none of these.

29. If P is population of a city in thousands and Q is fire demand in litres per minute, for proper estimate of water, the Empirical formula  is suggested by

a. National Board of fire under-writers

b. Freeman

c. Kuichling

d. None of these.

30. The formula  for the head loss in conduits is generally known as

a. Hazen-William's formula

b. Manning's formula

c. Darcy-Weisbach formula

d. Nikuradse formula.

31. In septic tanks,

a. the baffles or tees are extended up to top level of scum

b. the clear space between the baffle top and covering slab is about 7.5 cm

c. the inlet is kept 30 cm below the sewage line and outlet is kept at a level 40% depth of the sewage

d. all the above.

32. In 1000 kg of sewage, the total solids approximate

a. 0.5 to 1.0 kg

b. 1 kg to 2.0 kg

c. 5 kg to 7.5 kg

d. 7.5 kg to 10 kg.

33. To control the silt entry into a distributary at head regulator, King's vanes are provided which are the walls

a. of height l/3rd to l/4th the depth of water in the main canal

b. spaced at interval of  times their heights

c. extending about one metre beyond a line drawn at an inclination of 2 : 1 to the axis of main canal

d. all the above.

34. According to Lacey, in regime conditions

a. silt is kept in suspension by vertical components of eddies

b. entire cross-section of the channel is generated at all points by the forces normal to the wetted perimeter

c. both (a) and (b)

d. neither (a) nor (b).

35. Water-shed line is abondened for aligning an irrigation canal if

a. water shed forms a sharp loop

b. canal has to take off from a river

c. towns and villages are located on the water shed line

d. all the above.

36. Disposal of extra excavated earth of canals, is utilised to provide a spoil bank on

a. left side

b. right side

c. both sides

d. all the above.

37. To prevent a head-on-collision of vehicles travelling in opposite directions along four-lane roads

a. physical dividers are provided

b. area dividers are provided

c. medians of wide area are provided

d. none of these.

38. The distance travelled by a moving vehicle during perception and brake reaction times, is known as

a. sight distance

b. stopping distance

c. lag distance

d. none of these.

39. The length of the side of warning sign boards of roads is

a. 30 cm

b. 40 cm

c. 45 cm

d. 50 cm

40. A gradient along which the vehicle does not require any tractive effort to maintain a specified speed, is known as

a. ruling gradient

b. pushing gradient

c. floating gradient

d. minimum gradient

41. The correct prismoidal formula for volume is

a. D [first area + last area + ∑ Even area + 2 ∑ odd areas]

b.  [first area + last area + 4 ∑ Even area + 2 ∑ odd areas]

c.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas]

d.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas].

42. Pick up the incorrect statement from the following:

a. Lead is the average horizontal straight distance between the borrow pit and the place of spreading soil

b. The lead is calculated for each block of the excavated area

c. The unit of lead is 50 m for a distance upto 500 m

d. The unit of lead is 1 km where the lead exceeds 2 km.

43. If tensile stress of a steel rod of diameter D is 1400 kg/cm2 and bond stress is 6 kg/cm2, the required bond length of the rod is

a. 40 D

b. 50 D

c. 53 D

d. 59 D

44. Pick up the excavation where measurements are made in square metres for payment.

a. surface dressing up to 15 cm depths

b. Surface excavation up to 30 cm depths

c. Both (b) and (c)

d. Both (a) and (b)

45. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

46. Critical path lies along the activities having total float

a. positive

b. negative

c. zero

d. same.

47. Bar charts are suitable for

a. minor works

b. major works

c. large projects

d. all the Above.

48. The most popular type of organisation used for Civil Engineering Constructions, is

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

49. Pick up the correct statement from the following:

a. The distance between the points of intersection of the extreme tangents to the transition curve is kept greater than 7500 m x sum of grade changes at the point of intersection

b. The rate of change of grade is limited to 0.3% per 30 m length of the curve

c. According to I.C.A.O. the maximum longitudinal gradient along a runway is limited to 1.5%

d. All the above.

50. For the taxiways, the following statement is true

a. The maximum longitudinal grade is 3%

b. The permissible rate of change of grade is 1%

c. The permissible transverse grade is 1.5%

d. All the above.