***Sub Engineer***

1. Cross hairs in surveying telescopes, are fitted

a. in the objective glass

b. at the centre of the telescope

c. at the optical centre of the eye piece

d. in front of the eye piece.

2. Prolongation of chain line across an obstruction in chain surveying, is done by

a. making angular measurements

b. drawing perpendiculars with a chain

c. solution of triangles

d. all the above.

3. The formula for the horizontal distances for inclined sights, on staff held normalis  fi s cos θ + (f + d) cos θ ± h sin θ

a. minus sign is used for angle of depression

b. plus sign is used for angle of depression

c. minus sign is used for angle of elevation

d. none of these.

4. An angle of deflection right, may be directly obtained by setting the instrument to read

a. zero on back station

b. 180° on back station

c. 90°

d. 270° on back station.

5. The frog of a brick is normally made on its

a. top face

b. bottom face

c. longer face

d. shorter side.

6. Refractory bricks resist

a. high temperature

b. chemical action

c. dampness

d. all the above.

7. For filling cracks in masonry structures, the type of bitumen used, is

a. cut-back bitumen

b. bitumen-emulsion

c. blown bitumen

d. plastic bitumen.

8. Kaolin is chemically classified as

a. metamorphic rock

b. argillaceous rock

c. calcareous rock

d. silicious rock.

9. For beams breadth is constant,

a. depth d ∝ M

b. depth d ∝ M

c. depth d ∝ 3M

d. depth d ∝ 1M

10. Pick up the incorrect statement from the following : The torsional resistance of a shaft is directly proportional to

a. modulus of rigidity

b. angle of twist

c. reciprocal of the length of the shaft

d. moment of inertia of the shaft section.

11. If the stress in each cross-section of a pillar is equal to its working stress, it is called

a. body of equal

b. body of equal section

c. body of equal strength

d. none of these.

12. The shape of the bending moment diagram over the length of a beam, carrying a uniformly increasing load, is always

a. linear

b. parabolic

c. cubical

d. circular.

13. A body of dimensions 1.5 m x 1.0 m x 2 m weighs 3000 kg in water. Its specific gravity is

a. 0.8

b. 0.9

c. 1.0

d. 1.1

14. A pipe of 0.1 m2 cross sectional area suddenly enlarges to 0.3 m2 cross-sectional area. If the discharge of the pipe is 0.3 m3 /sec, the head loss is

a. 2/g m of water

b. g/2 m of water

c. 1g m of water

d. g m of water.

15. For uniform flow in canals

a. there is a balance between the frictional loss and drop in elevation of the channel

b. bed and free water surfaces of a channel are parallel to each other

c. bed of channel represents the hydraulic gradient

d. all the above.

16. Energy equation is usually applicable to

a. non-uniform flow

b. turbulent flow

c. laminar flow

d. steady flow.

17. For determining the ultimate bearing capacity of soil, the recommended size of a square bearing plate to be used in load plate test should be 30 to 75 cm square with a minimum thickness of

a. 10 mm

b. 15 mm

c. 20 mm

d. 25 mm

18. Pick up the correct statement from the following:

a. A soil having pH value more than 7 is an acidic soil

b. A soil having pH value less than 7 is an acidic soil

c. A soil having pH value more than 7 is an alkaline soil

d. A soil containing chemicals for the manufacture of portland cement is preferred.

19. A sample of saturated soil has 30% water content and the specific gravity of soil grains is 2.6. The dry density of the soil mass in g/cm3, is

a. 1.47

b. 1.82

c. 1.91

d. none of these.

20. Tergazhi's theory of one dimensional consolidation assumes

a. coefficient of permeability is constant

b. excess pore water drains out only in the vertical direction

c. time lag in consolidation is due entirely to permeability

d. all the above.

21. An R.C.C. column is treated as long if its slenderness ratio is greater than

a. 35

b. 40

c. 50

d. 60

22. Steel beam theory is used for

a. design of simple steel beams

b. steel beams encased in concrete

c. doubly reinforced beams ignoring compressive stress in concrete

d. beams if shear exceeds 4 times allowable shear stress.

23. In a simply supported slab the minimum spacing of distribution reinforcement, should be four times the effective thickness of the slab or

a. 30 cm

b. 40 cm

c. 50 cm

d. 60 cm

24. An R.C.C. lintel is spanning an opening of 2 m span in a brick wall. The height of the roof is 2.9 m above the floor level and that of the opening is 2.1 m above the floor level. The lintel is to be designed for self weight plus

a. triangular load of the wall

b. UDL of wall

c. UDL of wall + load from the roof

d. triangular load + load from the roof.

25. Which one of the following piles has a cast iron shoe even after removal of the hollow cylindrical steel casing

a. pedastal pile

b. Franki pile

c. vibro pile

d. both (a) and (d) of the above.

26. If the depth of an excavation is 20 metres, number of single stage well points to be installed at various levels, is

a. 4

b. 3

c. 2

d. 6.

27. The ratio of various ingredients (cement, sand, aggregates) in concrete of grade M 200, is

a. 1 : 3 : 6

b. 1 : 1 ½  : 3

c. 1 : 1 : 2

d. 4 : 2 : 1

28. Pick up the incorrect statement from the following:

a. Cement bags should preferably be piled on wooden planks

b. Cement bags should be placed such that bags of one layer does not touch the bags of the adjacent layer

c. Width and height of the pile should not exceed 3 m and 2.70 m respectively

d. None of these.

29. A circular gravity aqueduct is not generally preferred to because of

a. its maximum hydraulic mean depth

b. maximum area per unit of wetted perimeter

c. minimum cost of construction

d. its proper support on the ground.

30. Maximum permissible colour for domestic water supplies, based on Cobalt scale, is

a. 10 P.P.M.

b. 15 P.P.M.

c. 20 P.P.M.

d. 25 P.P.M.

31. The pH value of fresh sewage is usually

a. equal to 7

b. more than 7

c. less than 7

d. equal to zero.

32. Pick up the incorrect statement from the following :

a. The treated sewage effluents, are generally used for irrigating the crops

b. The process of collecting, treating and disposing off the sewage, is called sewerage

c. The old convergency system was definitely better than water carried sewerage system

d. None of these.

33. In rigid module, the discharge

a. is independent of water levels in the distributary and water course

b. depends upon the water level in distributary

c. depends upon the water level in the water course

d. depends upon the water levels of both, i.e. distributary and water course

34. In a barrage, the crest level is kept

a. low with large gates

b. high with large gates

c. high with no gates

d. low with no gates.

35. The consumptive use of water for a crop

a. is measured as the volume of water per unit area

b. is measured as depth of water on irrigated area

c. may be supplied partly by precipitation and partly by irrigation

d. all the above.

36. Pick up the correct statement from the following:

a. Gravity water is harmful to crops

b. Hygroscopic water remains attached to soil molecules by chemical bond

c. Capillary moisture held in the soil pores against gravity by surface tension, is utilised by plants

d. All the above.

37. In complex situations, total time required for a driver to form a judgement and to act, may be taken as

a. 1.5 sec

b. 2.0 sec

c. 2.5 sec

d. 3.0 sec

38. The gradients of trace cuts for hilly roads, are kept

a. equal to ruling gradient

b. 10 tO 20 per cent easier than ruling gradients

c. 10 to 20 per cent steeper than ruling gradients

d. none of these.

39. According to Highway Research Board of U.S.A. practical land width, is

a. 3.0 m

b. 3.3 m

c. 3.6 m

d. 4.5 m

40. If cross slope of a country is upto 10% the terain is classified as

a. plain

b. rolling

c. mountainous

d. steep.

41. The height of the sink of wash basin above floor level is kept

a. 60 cm

b. 70 cm

c. 75 cm to 80 cm

d. 80 cm

42. Pick up the incorrect statement from the following:

a. Dimensions are measured to the nearest 0.01 m

b. Areas are measured to the nearest 0.01 sq.m

c. Cubic contents are measured to the nearest 0.1 cm m

d. Weights are measured to the nearest 0.001 tonnes

43. The correct prismoidal formula for volume is

a. D [first area + last area + ∑ Even area + 2 ∑ odd areas]

b.  [first area + last area + 4 ∑ Even area + 2 ∑ odd areas]

c.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas]

d.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas].

44. A cement concrete road is 1000 m long, 8 m wide and 15 cm thick over the sub-base of 10 cm thick gravel. The box cutting in road crust is

a. 500 m3

b. 1000 m3

c. 1500 m3

d. 2000 m3

45. PERT analysis is based on

a. optimistic time

b. pessimistic time

c. most likely time

d. all the above.

46. Pick up the incorrect statement from the following:

a. The various functions under each activity, are shown by one bar on Bar Charts

b. Bar chart establishes the interdependency of one event on another

c. Only approximate percentage of the completed work is reported

d. None of these,

47. A goiden rule for the procurement of construction stones, suggests

a. 67% at the site and 33% under procurement

b. 50% at the site and 50% under procurement

c. 33% at the site and 67% under procurement

d. none of these.

48. While scheduling a project by C.P.M.

a. required time for each activity is established

b. sequence of various activities is made according to their importance

c. net work is drawn by connecting the activities and the events

d. All the above.

49. Pick up the correct statement from the following:

a. The propellers which are driven by turbine engines, are technically called turboprops

b. The aircrafts which obtain the thrust directly from turbine engine, are called turbo-jets

c. The aircraft which is filled with a fan infront of turbo-jet engine, is called turbofan propelled aeroplane

d. All the above.

50. The air is blowing at a speed of 75 knots in the direction of the aircraft flying at a ground speed of 775 knots. The air speed of the aircraft, is

a. 775 knots

b. 75 knots

c. 850 knots

d. 675 knots