***Sub Engineer***

1. The staff intercept will be

a. greater farther off the staff is held

b. smaller, farther off the staff is held

c. smaller, nearer the staff is held

d. same, wherever the staff is held.

2. Diopter is the power of a lens having a focal length of

a. 50 cm

b. 75 cm

c. 100 cm

d. 125 cm

3. Orientation of a plane table by solving two point problem is only adopted when

a. saving of time is a main factor

b. better accuracy is a main factor

c. given points are inaccessible

d. none of these.

4. If the long chord and tangent length of a circular curve of radius R are equal the angle of deflection, is

a. 60°

b. 90°

c. 120°

d. 150°.

5. Pick up the correct statement from the following:

a. Horn blende mineral is brittle

b. Muscovite is also known as white mica and potashmica

c. Biotite is also known as blackmica

d. All the above.

6. The property by virtue of which lime sets under water, is known as

a. slacking

b. setting

c. hydraulicity

d. calcining.

7. For melting one tonne of cast iron

a. 700 m3 air is required

b. 20 kg limestone is required

c. one quintal coke is required

d. all the above.

8. For slaking of 10 kg of CaO, the theoretical amount of water is

a. 2.2 kg

b. 1.5 kg

c. 3.2 kg

d. None of these.

9. For calculating the allowable stress of long columns. The empirical formula , is known as

a. Straight line formula

b. Parabolic formula

c. Perry's formula

d. Rankine's formula.

10. The ratio of shear stress and shear strain of an elastic material, is

a. Shear Modulus

b. Young's Modulus

c. Modulus of Elasticity

d. both (a) and (b).

11. Every material obeys the Hooke's law within its

a. elastic limit

b. plastic point

c. limit of proportionality

d. none of these.

12. A rectangular beam 20 cm wide is subjected to a maximum shearing force of 10, 000 kg, the corresponding maximum shearing stress being 30 kg/cm2. The depth of the beam is

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm.

13. The flow in open channel is said to be subcritical if the Froude number is

a. less than 1.0

b. equal to 1.0

c. greater than 1.0

d. none.

14. For critical flow, the Froude number is :

a. 1.0

b. less than 1.0

c. more than 1.0

d. 2

15. Unit of kinematic viscosity is

a. m2/sec

b. Newton sec/m2

c. Newton sec/m3

d. Kg sec/m2.

16. Pick up the incorrect statement from the following :

a. In radial flow, fluid flows such that pressure and velocity at any point change with respect to its distance from the central axis

b. In radial flow, velocity of flow is in a radial direction

c. In radial flow, flow may take place radially inward to or outward from the centre

d. In radial flow, flow is one dimensional with stream lines parallel.

17. Minimum size of the particles of silt soil, is

a. 0.002 mm

b. 0.04 mm

c. 0.06 mm.

d. 0.08 mm

18. The property of a soil which permits water to percolate through it, is called

a. moisture content

b. permeability

c. capillarity

d. none of these.

19. Si particles

a. show dilatancy

b. swell when moist

c. possess high strength when dry

d. disintegrate easily.

20. The passive earth pressure of a soil, is proportional to (where φ is the angle of internal friction of the soil.)

a. tan (45° - φ)

b. tan (45° + φ)

c. tan2 (45° - φ

d. tan2 (45° + φ)

21. For initial estimate for a beam design, the width is assumed

a. 1/20th of span

b. 1/25th of span

c. 1/30th of span

d. 1/40th of span.

22. Pick up the incorrect statement from the following: Tensile reinforcement bars of a rectangular beam

a. are curtailed if not required to resist the bending moment

b. are bent up at suitable places to serve as shear reinforcement

c. are bent down at suitable places to serve as shear reinforcement

d. are maintained at bottom to provide at least local bond stress.

23. The stresses developed in concrete and steel in reinforced concrete beam 25 cm width and 70 cm effective depth, are 62.5 kg/cm2 and 250 kg/cm2 respectively. If m = 15, the depth of its neutral axis is

a. 25 cm

b. 30 cm

c. 35 cm

d. 40 cm

24. A per IS : 1343, total shrinkage for a pretensioned beam, is

a. 3.0 x 10-3

b. 3.0 x 10-4

c. 3.0 x 10-5

d. 3.5 x 10-5

25. The stone masonry of finely dressed stones laid in cement or lime, is

a. random rubble masonry

b. coursed rubble masonry

c. dry rubble masonry

d. ashlar masonry.

26. A wall constructed to resist the pressure of an earth filling, is called

a. retaining wall

b. breast wall

c. buttress

d. parapet wall.

27. If 1500 g of water is required to have a cement paste 1875 g of normal consistency, the percentage of water is,

a. 25%

b. 30%

c. 35%

d. 40%

28. Batching error means inaccuracy in the quantity of

a. aggregates

b. cement

c. water

d. all the above.

29. If the specific capacity of a well is 0.3183 x 10-3 per sec, the discharge from a well of 4 m diameter under a depression head of 4 m, is

a. 10 litres/sec

b. 12 litres/sec

c. 14 litres/sec

d. 16 litres/sec.

30. Water may not contain much impurities if its source is

a. stream flowing in plains

b. lakes in lower regions

c. spring along hill slopes

d. none of these.

31. In areas where light rains are uniformly distributed throughout the year, the type of sewerage system to be adopted is

a. separate system

b. combined system

c. partially combined system

d. none of these.

32. For the survival of fish in a river stream, the minimum dissolved oxygen is prescribed

a. 3 PPm

b. 4 PPm

c. 5 PPm

d. 10 ppm.

33. Pick up the correct statement from the following:

a. The full supply level of a canal should be above ground level

b. According to Lacey, regime conditions require a particular slope for a given discharge and silt factor

c. In case the ground slope is less than the required bed slope, the silt factor must be reduced by permitting the entry of coarse silt

d. All the above.

34. Process of meandering is due to

a. sediment load of streams

b. discharge and hydraulic properties of streams

c. relative erodibility of the bed and banks

d. all the above.

35. The most suitable section of a lined canal, is

a. trapezoidal section with rounded corners for large canals

b. rectangular section with rounded corners for large canals

c. both (a) and (b)

d. both (a) and (c)

36. The intensity of irrigation means

a. percentage of culturable commanded area to be irrigated annually

b. percentage of gross commanded area to be irrigated annually

c. percentage of the mean of culturable commanded area and the gross commanded area to be irrigated annually

d. total depth of water supplied by the number of waterings.

37. The desirable camber for straight roads with thin bituminous surfacing, is

a. 1 in 40 to 1 in 33

b. 1 in 150 to 1 in 140

c. 1 in 160 to 1 in 140

d. none of these.

38. If the coefficient of friction on the road surface is 0.15 and a maximum super-elevation 1 in 15 is provided, the maximum speed of the vehicles on a curve of 100 metre radius, is

a. 42.44 kg/hour

b. 52.44 km/hour

c. 62.44 km/hour

d. 72.44 km/hour

39. Customers prefer parking of their vehicles at

a. 85° to aisles

b. 80° to aisles

c. 75° to aisles

d. 60° to aisles

40. The normal road width of National and State highways

a. is kept 45 m

b. in plain and rolling terrain built-up area, is 30 m

c. in mountainous built-up area is 20 m

d. All the above.

41. If the formation level of a highway has a uniform gradient for a particular length, and the ground is also having a longitudinal slope, the earthwork may be calculated by

a. Mid-section formula

b. Trapezoidal formula

c. Prismoidal formula

d. All the above.

42. The brick work is measured in sq metre, in case of

a. Honey comb brick work

b. Brick flat soling

c. Half brick walls or the partition

d. All the above.

43. The following item of earth work is not measured separately.

a. Site clearence

b. dead men

c. Steps in deep exavaction

d. All the above.

44. Brick walls are measured in sq. m if the thickness of the wall is

a. 10 cm

b. 15 cm

c. 20 cm

d. None of these.

45. Pick up the correct statement from the following:

a. Backward pass is used for calculating the latest occurence time

b. Maximum value of earliest expected time is used if there are more than one value of any event

c. Minimum value of latest occurrence time is used if there are more than one value of any event

d. All the above.

46. Pick up the correct statement from the following:

a. Programme Evaluation and Review Technique, is event oriented

b. Programme Evaluation and Review Technique is not event oriented

c. Critical Path Method is event oriented

d. Critical Path method is event oriented.

47. While scheduling a project by C.P.M.

a. required time for each activity is established

b. sequence of various activities is made according to their importance

c. net work is drawn by connecting the activities and the events

d. All the above.

48. Critical path lies along the activities having total float

a. positive

b. negative

c. zero

d. same.

49. The length of runway is increased per 300 m rise above M.S.L.

a. 4%

b. 5%

c. 6%

d. 7%

50. The maximum value of the angle of turning of the nose gear large jet aircrafts, is limited to

a. 20°

b. 30°

c. 45°

d. 60°