***Sub Engineer***

1. The branch of surveying in which both horizontal and vertical positions of a point, are determined by making instrumental observations, is known

a. tacheometry

b. tachemetry

c. telemetry

d. all the above.

2. The co-ordinate of a point measured perpendicular to the parallel, is called

a. total latitude

b. meridian distance

c. total departure

d. consecutive co-ordinate.

3. In levelling operation

a. when the instrument is being shifted, the staff must not be moved

b. when the staff is being carried forward, the instrument must remain stationary

c. both (a) and (b)

d. neither (a) nor (b).

4. If Δ is the angle of deflection of a simple curve of radius R, the length of its long chord, is

a. R cos

b. 2R cos

c. R sin

d. 2R sin .

5. Pick up the correct statement from the following:

a. The edges of a finished wrought iron, are rough due to red shortage

b. Pig iron (charcoal) is manufactured from magnetic ore (Fe3O4)

c. For the manufacture of wrought iron, non-sulphurous fuel is not necessary

d. All the above.

6. Pick up the correct statement from the following:

a. The low voltage porcelain is prepared by wet process

b. The high voltage porcelain is prepared by dry process

c. The low voltage porcelain is prepared by dry process

d. None of the above.

7. Pick up the correct statement from the following:

a. Formaldehyde is prepared synthetically from methane

b. The melamine when reacted with formaldehyde forms the melamine-formaldehyde resin

c. The plastics made from melamine formaldehyde resin, are used for electrical insulators

d. All the above.

8. Steel contains carbon approximately

a. 1.50% to 5.6%

b. 0.05% to 1.75%

c. 0.25%

d. none to these.

9. Maximum strain theory for the failure of a material at the elastic limit, is known as

a. St. Venant's theory

b. Rankine's theory

c. Haig's theory

d. Von Mises's theory.

10. Stress may be expressed in Newtons

a. per millimetre square (N/mm2)

b. per centimetre square (N/cm2)

c. per metre square (N/m2)

d. All of the above.

11. The tensile force required to cause an elongation of 0.045 mm in a steel rod of 1000 mm length and 12 mm diameter, is (where E = 2 x 106 kg/cm2)

a. 166 kg

b. 102 kg

c. 204 kg

d. 74 kg

12. The slenderness ratio of a vertical column of a square cross-section of 2.5 cm sides and 300 cm length, is

a. 240

b. 360

c. 416

d. 500

13. In a short cylindercial external mouthpiece, the venacontracta occurs at a distance from the outlet of orifice equal to

a. diameter of the orifice

b. one-fourth the diameter of the orifice

c. one-third the diameter of the orifice

d. two-third the diameter of the orifice

14. A closed cylindrical vessel of 100 cm diameter and 200 cm high is completely filled with a liquid (sp. weight 1600 kg/m3) when rotated about its vertical axis at 100 r.p.m. The total pressure on its lid, is

a. 549 kg

b. 945 kg

c. 954 kg

d. 95.4 kg.

15. Capillary rise of water is

a. directly proportional to surface tension

b. inversely proportional to water density

c. inversely proportional to diameter of the tube

d. All of these.

16. The thickness of a sharp crested weir is kept less than

a. one-half of the height of water on the sill

b. one-fourth of the height of water on the sill

c. two-third of the height of water on the sill

d. none of these.

17. The shear resistance of a soil is con-tituted basically of the following component.

a. The frictional resistance to translocation between the individual soil particles at their contact point

b. To the structural relation to displacement of the soil because of the interlocking of the particles

c. Cohesion and adhesion between the surfaces of the soil particles

d. All the above.

18. For determining the moisture content of a soil sample, the following data is available Weight of container = 260 g, Weight of soil sample and = 320 g container, Weight of soil sample (dried) and = 310 g container. The moisture content of the soil sample, is

a. 15%

b. 18%

c. 20%

d. 25%

19. The maximum load carried by a pile, when it continues to sink without further increase of load, is known as

a. ultimate load carrying capacity

b. ultimate bearing capacity

c. ultimate bearing resistant

d. all the above.

20. A pile is being driven with a drop hammer weighing 1800 kg and having a free fall of 1.00 m. If the penetration with last blow is 5 mm, the load carrying capacity of the pile, -according to the Engineering News formula, is

a. 100 tonnes

b. 50 tonnes

c. 20 tonnes

d. 10 tonnes.

21. An R.C.C. lintel is spanning an opening of 2 m span in a brick wall. The height of the roof is 2.9 m above the floor level and that of the opening is 2.1 m above the floor level. The lintel is to be designed for self weight plus

a. triangular load of the wall

b. UDL of wall

c. UDL of wall + load from the roof

d. triangular load + load from the roof.

22. For initial estimate for a beam design, the width is assumed

a. 1/20th of span

b. 1/25th of span

c. 1/30th of span

d. 1/40th of span.

23. If the tendon is placed at an eccentricity e below the centroidal axis of the lon-gitudial axis of a rectangular beam (sectional modulus Z and stressed load P in tendon) the stress at the extreme top edge

a. is increased by

b. is increased by

c. is decreased by

d. remains unchanged.

24. Thickened part of a flat slab over its supporting column, is technically known as

a. drop panel

b. capital

c. column head

d. none of these.

25. The wedge shaped bricks forming an arch ring, are called

a. Soffits

b. voussoirs

c. haunchs

d. spandrils.

26. For a rectangular foundation of width b, eccentricity of load should not exceed

a. b/3

b. b/4

c. b/5

d. b/6.

27. Pick up the incorrect statemnt from the following. For performing compressive strength test of cement

a. cement and standard sand mortar are used in the ratio of 1 : 3

b. water is added at the rate of  + 3.0 percentage of water where P is the percentage of water for standard consistency

c. A cube mould of 10 cm x 10 cm x 10 cm is used

d. The perpared moulds are kept in a atmosphere of 50% relative humidity

28. For an ordinary Portland cement

a. soundness varies from 5 to 10 mm

b. initial setting time is not less than 30 minutes

c. compressive stress after 7 days, is not less than 175 kg/cm2

d. all the above.

29. Specific capacity or yield of wells, is generally expressed, as

a. m3/hour

b. m3/hour/m2

c. m3/hour/m2/m

d. m3/hour/m2/sec.

30. Perched acquifers are generally found

a. on the surface of the ground

b. below the surface of the ground but above water table

c. below the water table

d. all the above.

31. Pathogens (or pathogentic bacterias) in water may cause

a. cholera

b. dysentery

c. infectious hepatitis

d. all the above.

32. Assertion (A) : Discharging the effluents from the oxidation ponds just up stream of lakes or reservoirs is undesirable. Reason (R) : The discharged algae get settled in the reservoirs and cause anaerobic decomposition and other water qualities.

a. Both A and R are true and R is the correct explanation of A

b. Both A and R are true but R is not a correct explanation of A

c. A is true but R is false

d. A is false but R is true.

33. Regime conditions in a channel may occur if

a. discharge is constant

b. channel flows uniformly in incoherent alluvium as that transported in suspension

c. silt grade and silt charge are constant

d. all the above.

34. For diversion of flood water of rivers, the type of canal constructed, is

a. ridge canal

b. perennial canal

c. inundation canal

d. canal.

35. If H and d are the water depth and drop in the bed level at a Sarda fall, the width B of the trapezoidal crest, is given by

a. B = 0.22 H + d

b. B = 0.33 H + d

c. B = 0.44 H + d

d. B = 0.55 H + d

36. Lane's weighted creep theory assumes

a. double weightage to horizontal creep and one weightage to vertical creep

b. triple weightage to horizontal creep and one weightage to vertical creep

c. triple weightage to vertical creep and one weightage to horizontal creep

d. double weightage to vertical creep and one weightage to horizontal creep.

37. The width of road pavements, depends upon

a. width of traffic lane

b. number of traffic

c. width of median strip

d. all the above.

38. Alignment of highways in hilly regions, is decided on

a. number of river crossings

b. natural unstable areas

c. saddles or passes

d. all the above.

39. In case of cement concrete pavements, pick up the incorrect statement

a. initial cost of construction is high

b. initial cost of construction is low

c. visibility during nights is high

d. none of these.

40. The maximum safe speed on roads, depends on the

a. type of road surface

b. type of curves

c. sight distance

d. all the above.

41. The brick work is measured in sq metre, in case of

a. Honey comb brick work

b. Brick flat soling

c. Half brick walls or the partition

d. All the above.

42. Pick up the excavation where measurements are made in square metres for payment.

a. surface dressing up to 15 cm depths

b. Surface excavation up to 30 cm depths

c. Both (b) and (c)

d. Both (a) and (b)

43. The area of a sloping surface of a protective embankment of mean height d, side slopes S : 1 and length L is

a. d x d x s

b. d2 x (ds)2

c. L.D 1 + s2

d. 2 LD 1 + s2

44. The expected out turn of cement concrete 1 : 2 : 4 per mason per day is

a. 1.5 m3

b. 2.5 m3

c. 3.5 m3

d. 5.0 m3

45. If D is the duration, ES and EF are the earliest start and finish, LS and LF are latest start and latest finish time, then the following relation holds good

a. LS = LF - D

b. LF = LS + D

c. D = EF - ES

d. all the above.

46. PERT analysis is based on

a. optimistic time

b. pessimistic time

c. most likely time

d. all the above.

47. The Overall in-charge of an organisation at the site responsible for the execution of the works, is

a. Engineer

b. Junior Engineer

c. Sub overseer

d. Assistant Engineer.

48. Pick up the correct network for the activities of pouring concrete, erection of form work, removal of form work and curing of concrete from the following:

a. 1. Pouring of Concrete  2. Erection of form work 3. Curing of concrete 4. Removal of form work

b. 1. Erection of form work 2. Pouring of concrete 3. Curing of concrete 4. Removal of form work

c. 1. Removal of form work  2. Erection of form work 3. Pouring of concrete 4. Curing of concrete

d. 1. Pouring of concrete 2. Curing of concrete 3. Erection of form work 4. Removal of form work

49. The maximum value of the angle of turning of the nose gear large jet aircrafts, is limited to

a. 20°

b. 30°

c. 45°

d. 60°

50. According to I.C.A.O. all markings on the runways are painted white and on taxiways

a. black

b. red

c. yellow

d. green