***Sub Engineer***

1. Plotting of inaccessible points on a plane table, is done by

a. intersection

b. traversing

c. radiation

d. none of these.

2. While setting a plane table at a station it was found that the error in centering was 30 cm away from the ray of length 40 m drawn from the station. If the scale of the plan is 1 cm = 2 cm, the displacement of the end of the ray in plan from the true position will be

a. 0.02 cm

b. 0.15 cm

c. 02 cm

d. 0.1 cm

3. If d is the distance between equidistant odd ordinates, the Simpson's rule for the areas, is

a. [h1 + hn + 2(h3 + h5 + ... + hn - 2) + 4(h2 + h4 + ... + hn - 1)]

b. [h1 + hn x 2(h2 + h4 + ... + hn - 1) + 4(h3 + h5 + ... + hn - 2)]

c. [h1 + hn + 2(h3 + h5 + ... + hn - 2) + 4(h2 + h4 + ... + hn - 1)]

d. none of these.

4. In a constant level tube, size of the bubble remains constant because upper wall is

a. of relatively larger radius

b. of relatively smaller radius

c. flat

d. convex downwards.

5. Plastics are compounds of carbon with element

a. hydrogen

b. nitrogen

c. oxygen

d. All of these.

6. Bitumen may be dissolved in

a. water

b. sodium chloride

c. carbon disulphide

d. none of these.

7. Pick up the rock which is not  a sedimentary rock from the following:

a. sand stone

b. gypsum

c. dolerite

d. lignite.

8. A volatile substance added to a paint to make its application easy and smooth, is known as

a. base

b. solvent

c. vehicle

d. none to these.

9. By applying the static equations i.e. ΣH = 0, ΣV = 0 and ΣM = 0, to a determinate structure, we may determine

a. shear forces only

b. bending moments only

c. internal forces only

d. all the above.

10. A rolled steel joist is simply supported at its ends and carries a uniformly distributed load which causes a maximum deflection of 10 mm and slope at the ends of 0.002 radian. The length of the joist will be,

a. 12 m

b. 14 m

c. 16 m

d. 18 m

11. A member is balanced at its end by two inclined members carrying equal forces. For equilibrium the angle between the inclined bars must be

a. 45°

b. 60°

c. 90°

d. 120°

12. A cantilever carrying a uniformly distributed load W over its full length is propped at its free end such that it is at the level of the fixed end. The bending moment will be zero at its free end also at

a. fixed point of the cantilever

b. 1/4th length from free end

c. 3/4th length from free end

d. half length from free end.

13. Flow in pipes is turbulent if Reynold number is

a. less than 2100

b. more than 3000

c. between 2100 and 3000

d. none of these.

14. While applying the Bernoulli's equation  = total head, the work any section done on the flow system, if any

a. is added on the right side of the equation

b. is added on the left side of the equation

c. is ignored

d. none of these.

15. In Newton's viscosity equation , the coefficient (μ) is known as coefficient of

a. absolute viscosity

b. dynamic viscosity

c. simple viscosity

d. all the above.

16. A fluid particle may possess the displacement of

a. translation

b. rotation

c. distortion

d. all the above.

17. If the back fill is having a uniform surcharge of intensity q per unit area, the lateral pressure will be

a. q times the lateral pressure within the surface

b. 1/q times the lateral pressure within the surface

c. equal to a fill of height Z equal to q/r, where r is the density of the backfill

d. none of these.

18. Pick up the correct statement from the following :

a. Liquid limit minus plastic limit, is known as plasticity index of the soil

b. Plastic limit minus shrinkage limit, is known as shrinkage index of the soil

c. The ratio of the plasticity index to the flow limit, is known as toughness index of the soil

d. All the above.

19. The intensity of active earth pressure at a depth of 10 metres in dry cohesionless sand with an angle of internal friction of 30° and with a weight of 1.8 t/m3, is

a. 5 t/m2

b. 6 t/m2

c. 7 t/m2

d. 8 t/m2

20. Geologic cycle for the formation of soil, is

a. Weathering → upheaval → transportation → deposition

b. Transportation → upheaval → weathering → deposition

c. Weathering → transportation → deposition → upheaval

d. None of these.

21. A part of the slab may be considered as the flange of the T-beam if

a. flange has adequate reinforcement transverse to beam

b. it is built integrally with the beam

c. it is effectively bonded together with the beam

d. all the above.

22. An R.C.C. column is treated as short column if its slenderness ratio is less than

a. 35

b. 40

c. 50

d. 60

23. The shear reinforcement in R.C.C. is provided to resist

a. vertical shear

b. horizontal shear

c. diagonal compression

d. diagonal tension.

24. If Ac, Asc and A are areas of concrete, longitudinal steel and section of a R.C.C. column and m and σc are the modular ratio and maximum stress in the configuration of concrete, the strength of column is

a. σcAc + m σcAsc

b. σc(A - Asc) + m σcAsc

c. σc[A + (m - 1)ASC]

d. all the above.

25. Dutch bond is a modification of

a. English bond

b. stretcher bond

c. header bond

d. single Flemish bond.

26. The strength of brick masonry in 1:6 cement mortar, is

a. 40 tonnes/m2

b. 50 tonnes/m2

c. 60 tonnes/m2

d. 75 tonnes/m2.

27. Pick up the correct statement from the following:

a. The quantity of water required for concreting, depends upon the grading of aggregate amd method of compaction

b. 10% excess of water reduces the strength of concrete by 15%

c. 30% excess of water reduces the strength of concrete by 50%

d. All the above.

28. Di-calcium silicate (C2S)

a. generates less heat of hydration

b. hardens rapidly

c. provides less ultimate strength to cement

d. has less resistance to sulphate attack.

29. A pressure conduit laid under ground, may not be subjected to

a. pressure due to external load

b. longitudinal temperature stress

c. longitudinal stresses due to unbalanced pressure to bends

d. none of these.

30. Pick up the correct statement from the following :

a. The internal pressure in a pipe running full is equal to vertical ordinate between hydraulic gradient line and centre of the pipe

b. In pressure pipes with water at rest, the pressure is equal to water head

c. Sudden closure of a valve, causes the water hammer pressure

d. All the above.

31. For trunk and out-fall, the type of sewers generally used, is

a. circular shaped

b. horse shoe shaped

c. parabolic shaped

d. semi-elliptical shaped.

32. The angle subtended by the surface of sewer water with partial flow, at sewer centre is 120°, the depth of sewerage is

a. 25 cm

b. 40 cm

c. 50 cm

d. 60 cm.

33. Pick up the correct statement from the following:

a. Gravity water is harmful to crops

b. Hygroscopic water remains attached to soil molecules by chemical bond

c. Capillary moisture held in the soil pores against gravity by surface tension, is utilised by plants

d. All the above.

34. A minimum of 90 cm free board is provided if the discharge in the canal is between

a. 30 to 33 cumecs

b. 30 to 60 cumecs

c. Over 60 cumecs

d. Over 100 cumecs

35. To hold hydraulic jumps, baffle walls are provided in

a. Sarda type falls

b. English type falls

c. Montague type falls

d. Vertical type falls.

36. According to Khosla, the exist gradient of surface flow

a. depends upon the b/d ratio

b. is independent of the b/d ratio

c. is independent of the depths of d/s cut off walls

d. none of these.

37. An Executive Engineer of roads, executes works under direct control of

a. Superintending Engineer

b. Secretary to the Govt

c. Chief Engineer

d. None of these.

38. When load is applied on concrete pavement

a. away from edges, the maximum bending moment is negative

b. away from edges, the maximum bending moment cause compression

c. on the edges, the maximum stress is parallel to the edge of the slab

d. none of these.

39. The normal road width of National and State highways

a. is kept 45 m

b. in plain and rolling terrain built-up area, is 30 m

c. in mountainous built-up area is 20 m

d. All the above.

40. For clear distinct vision, images of obstructions should fall on the retina with a cone of

a. 3°

b. 4°

c. 5°

d. 6°

41. For the construction of buildings, the subheads of the estimate are

a. Brickwork, Stone work, Roofing

b. Brickwork Flooring, Wood work, Steel work

c. Plastering or pointing, finishing, water supply and sanitary work

d. All the above.

42. While estimating the qualities for the construction of a building, the correct metric unit is

a. Cubic metre for area

b. Square metres for volume

c. Kilogram for weight

d. Litre for capacity

43. The minimum width of a septic tank is taken

a. 70 cm

b. 75 cm

c. 80 cm

d. 90 cm

44. The order of booking dimensions is

a. Length, breadth, height

b. Breadth, length, height

c. Height, breadth, length

d. None of these.

45. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

46. The main principle of an organisation, is

a. cohegency

b. effective control at all levels

c. delegation of authority

d. all the above.

47. The artificial activity which indicates that an activity following it, cannot be started unless the preceding activity is complete, is known as

a. event

b. free float

c. dummy

d. constant

48. A Milestone chart

a. shows the interdependencies of various jobs

b. depicts the delay of jobs, if any

c. points outgoing ahead of schedule of jobs, if any

d. none of these.

49. According to the International Civil Aviation Organisation (I.C.A.O.) the strength of runway pavements, have been coded by

a. Last Seven English alphabets

b. First Seven English alphabets

c. First seven numbers

d. None of these.

50. The reduced levels of the ends A and B of a runway are 3025 m and 3035 m and that of its mid-point is 3015 m. The reduced level of the horizontal surface, is

a. 3060 m

b. 3075 m

c. 3015 m

d. none of these.