***Sub Engineer***

1. If the radius of a simple curve is R, the length of the chord for calculating offsets by the method of chords produced, should not exceed.

a. R/10

b. R/15

c. R/20

d. R/25.

2. Ranging is an operation of

a. reconnaissance

b. judging the distance

c. determination of slope

d. establishing intermediate points between terminals.

3. The bearings of two traverse legs AB and BC are N52° 45' E and N34° 30' E respectively. The deflection angle is

a. 18° 15' N

b. 18° 15' W

c. 18° 15' R

d. 18° 15' L

4. The whole circle bearing of a line is 290°. Its reduced bearing is

a. N 20° E

b. N 20° W

c. N 70° W

d. S 70° E

5. Bullet proof glass is made of thick glass sheet sandwiched by a layer of

a. steel

b. stainless steel

c. high test plastic

d. chromium plate.

6. Acrylic sheets

a. are generally unaffected by most household detergents

b. possess the light transmission rate of 93%

c. are available in various shapes

d. all the above.

7. Pick up the plutonic rock from the following:

a. Granite

b. Dolerite

c. Basalt

d. All the above.

8. Apiece of sawn timber whose cross-sectional dimensions exceed 5 cm, in one direction and 20 cm in the other direction, is called a

a. cant

b. deal

c. baulk

d. strip.

9. A material is said to be perfectly elastic if

a. it regains its original shape on removal of the load

b. It regains its original shape partially on removal of the load

c. it does not regain its original shape at all

d. none of these.

10. Pick up the correct statement from the following:

a. Hoop strain of the walls of a cylinder due to liquid is

b. Longitudinal strain in the walls of a cylinder due to liquid is

c. Volumetric change in the cylinderdue to liquid is

d. All the above.

11. An arch with three hinges, is a structure

a. statically determinate

b. statically indeterminate

c. geometrically unstable

d. structurally sound but indeterminate

12. A cantilever beam rectangular in cross-section is subjected to an isolated load at its free end. If the width of the beam is doubled, the deflection of the free end will be changed in the ratio of

a. 1/8

b. 1/2

c. 2

d. 3

13. Flow in pipes is turbulent if Reynold number is

a. less than 2100

b. more than 3000

c. between 2100 and 3000

d. none of these.

14. In a centrifugual pump casing, the flow of water leaving the impeller, is

a. rectilinear flow

b. radial flow

c. free vortex motion

d. forced vertex.

15. A piezometer opening in pipes measures

a. velocity head

b. static pressure

c. total pressure

d. negative static pressure.

16. A pitot tube is used to measure

a. pressure

b. difference in pressure

c. velocity of flow

d. none of these.

17. The method of the slices is applicable to

a. homogenous soils

b. stratified soils

c. saturated soils

d. non-uniform slopes

18. Pick up the incorrect statement from the following:

a. Below the water table, the pore water may be static.

b. The hydrostatic pressure depends on the depth below the water level.

c. The attractive forces between the particles, caused due to negative pressure of water held above the water table is called soil suction.

d. None of these.

19. Pick up the correct statement from the following:

a. Failure plane carries maximum shear stress

b. Failure plane does not carry maximum shear stress

c. Failure plane carries shear stress equal to maximum shear stress

d. None of these.

20. The ratio of the volume of voids to the total volume of the given soil mass, is known

a. porosity

b. specific gravity

c. void ratio

d. water content.

21. Dimensions of a beam need be changed if the shear stress is more than

a. 10 kg/cm2

b. 15 kg/cm2

c. 20 kg/cm2

d. 25 kg/cm2

22. The radius of a bar bend to form a hook, should not be less than

a. twice the diameter

b. thrice the diameter

c. four times the diameter

d. five times the diameter

23. An R.C.C. beam of 25 cm width and 50 cm effective depth has a clear span of 6 metres and carries a U.D.L. of 3000 kg/m inclusive of its self weight. If the lever arm constant for the section is 0.865, the maximum intensity of shear stress, is

a. 8.3 kg/cm2

b. 7.6 kg/cm2

c. 21.5 kg/cm2

d. 11.4 kg/cm2

24. The allowable tensile stress in mild steel stirrups, reinforced cement concrete, is

a. 1400 kg/cm2

b. 190 kg/cm2

c. 260 kg/cm2

d. 230 kg/cm2

25. Raft foundation are generally preferred to when the area required for individual footing, is more than

a. 25% to total area

b. 30% of total area

c. 40% to total area

d. 50% of total area.

26. The thickness of a reinforced brick partition wall, is generally kept

a. 10 cm

b. 15 cm

c. 20 cm

d. 25 cm.

27. Di-calcium silicate (C2S)

a. generates less heat of hydration

b. hardens rapidly

c. provides less ultimate strength to cement

d. has less resistance to sulphate attack.

28. Internal friction between the ingredients of concrete, is decreased by using

a. fine aggregates

b. rich mix

c. more water and coarse aggregates

d. none of these.

29. Permanent hardness of water can be removed by

a. adding lime

b. adding chlorine

c. boiling

d. zeolite process.

30. Average annual rainfall at any station is the average of annual rainfall over a period of

a. 14 years

b. 21 years

c. 28 years

d. 35 years.

31. To maintain aerobic biological activity, the moisture content of the compost mass should be about

a. 45%

b. 50%

c. 55%

d. 60%.

32. The drop man holes are generally provided in sewers for

a. industrial areas

b. large town ships

c. hilly town ships

d. cities in plains.

33. Useful soil moisture for plant growth, is

a. capillary water

b. gravity water

c. hygroscopic water

d. chemical water

34. Pick up the correct statement from the following

a. The capacity of escapes should not be less than the capacity of the canal at its location

b. The escapes must lead the surplus water to natural drainages

c. The escapes are aligned to take advantage of contours of lower values

d. All the above.

35. According to G.W. Pickles the effect of confining the flood water of a river between levee, is to increase

a. water surface elevation during floods

b. maximum discharge

c. velocity and scouring action

d. surface slope of streams above the leveed portion.

36. For diversion of flood water of rivers, the type of canal constructed, is

a. ridge canal

b. perennial canal

c. inundation canal

d. canal.

37. Minimum number of 50 kg cement bags per cubic metre of concrete for a mix corresponding to crushing strength 280 kg/cm2 at 28 days, are

a. 6.5

b. 7.0

c. 7.5

d. 8.0

38. In case of a multi-lane road, overtaking is generally permitted

a. from right

b. from left

c. from both sides right and left

d. not at all.

39. If a Lemniscate curve of transition throughout is introduced to connect two parallel roads, the maximum polar angle of the curve, is

a. 15°

b. 20°

c. 30°

d. 45°

40. Pick up the correct statement from the following:

a. During reconnaissance, the general route of the alignment is selected

b. After reconnaissance, a trace is cut for the alignment

c. Last stage is the detailed surveys for desired geometries'of the highway

d. All the above.

41. In the mid-section formula

a. The area of mid-sections is calculated by using mean depth

b. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two sections

c. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two original sections

d. (a), (b) and (c) of the above.

42. The following item of earth work is not measured separately.

a. Site clearence

b. dead men

c. Steps in deep exavaction

d. All the above.

43. The concrete work for the following part of the building of specified thickness is measured in square metres

a. Floors

b. D.P.C.

c. Wall panels

d. All the above.

44. The brick work is measured in sq metre, in case of

a. Honey comb brick work

b. Brick flat soling

c. Half brick walls or the partition

d. All the above.

45. While scheduling a project by C.P.M.

a. required time for each activity is established

b. sequence of various activities is made according to their importance

c. net work is drawn by connecting the activities and the events

d. All the above.

46. Pre-tender stage requires

a. selection of site

b. formalisation of alignment of work

c. formalisation of designs and preparation of estimate

d. all the above.

47. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

48. Pick up the incorrect statement from the following:

a. The difference between the maximum time available for the job and actual time it consumes, is called total float

b. The difference between the latest start time and earliest start time of an activity, is called total float

c. The difference between the earliest finish time of an activity and the earliest start time of its successor activity, is called free float of the activity

d. None of these.

49. The depressions and undulations in the pavement, are caused due to

a. impact of heavy wheel loads

b. punching effect

c. flexible gears

d. all the above.

50. A gradient of + 0.08% is followed by a gradient of - 0.07%. If the permissible rate of change of grade is 0.003 per 30 metres, the length of the transition curve, is

a. 150 m

b. 140 m

c. 160 m

d. 175 m