***Sub Engineer***

1. If the smallest division of a vernier is longer than the smallest division of its primary scale, the vernier is known as

a. direct vernier

b. double vernier

c. retrograde vernier

d. simple vernier.

2. While measuring with a metallic tape of 30 m length pull should be applied

a. 1 kg

b. 2 kg

c. 3 kg

d. 4 kg

3. Resolving power of a telescope depends on

a. the diameter of the aperture

b. the pupil aperture of the eye

c. the diameter of the object glass

d. all the above.

4. A level when set up 25 m from peg A and 50 m from peg B reads 2.847 on a staff held on A and 3.462 on a staff held on B, keeping bubble at its centre while reading. If the reduced levels of A and B are 283.665 m and 284.295 m respectively, the collimation error per 100 m is

a. 0.015 m

b. 0.030 m

c. 0.045 m

d. 0.060 m

5. Gniess is obtained from

a. igneous rocks

b. metamorphic rocks

c. sedimentary rocks

d. sedimentary metamorphic rocks.

6. Quartzite, a metamorphic stone is

a. brittle

b. crystalline

c. compact

d. All the above.

7. A 1st class brick immersed in water for 24 hours, should not absorb water (by weight) more than

a. 15%

b. 20%

c. 25%

d. 5%.

8. A bull nose brick is not used for

a. rounding off sharp corners

b. pillars

c. decoration purpose

d. arches.

9. Principal planes are subjected to

a. normal stresses only

b. tangential stresses only

c. normal stresses as well as tangential stresses

d. none of these.

10. For calculating the allowable stress of long columns. The empirical formula , is known as

a. Straight line formula

b. Parabolic formula

c. Perry's formula

d. Rankine's formula.

11. In a continuous bending moment curve the point where it changes sign, is called

a. point of inflexion

b. point of contraflexture

c. point of virtual hinge

d. all the above.

12. The cross sections of the beams of equal length are a circle and a square whose permissible bending stress are same under same maximum bending. The ratio of their flexural weights is,

a. 1.118

b. 1.338

c. 1.228

d. 1.108

13. For a most economical rectangular channel, the hydraulic mean depth, is equal to

a. the depth of flow

b. half the depth of flow

c. one-third depth of flow

d. None of these.

14. If H is the difference of liquid levels on two sides of an orifice (width b, depth d), the discharge through the orifice will be

a. Cdbd22gH

b. Cdb2d2gH

c. Cdbd2gh

d. Cdbd.2g H1/2.

15. For steady flow in open channels, which one of the following does not change :

a. depth of flow

b. velocity of flow

c. rate of flow

d. All of these.

16. The flow is called rotational if its velocity normal to the plane of area is equal to

a. angular velocity vector

b. twice the angular velocity vector

c. thrice the angular velocity vector

d. none of these.

17. If drainage is permitted throughout the test, during the application of both normal, and shear stresses so that full consolidation occurs and no excess pore pressure is set up at any stage of the test, is known as

a. quick test

b. drained test

c. consolidated undrained test

d. none of these.

18. Failure of the stability of slopes, generally occurs along

a. slip plane

b. a horizontal surface

c. a curved surface

d. all the surfaces.

19. If G is specific gravity of sand particles, e is porosity, the critically hydraulic gradient

a. ic =

b. ic =

c. ic =

d. ic =

20. The ratio of the undrained strength in the undrained state to the undrained strength, at the same water content, in the remoulded state, is called the sensivity of the clay. Its value for quick clays is

a. 8

b. 12

c. 16

d. 20

21. The length of lap in tension reinforcement should not be less than the bar diameter x (actual tension / four times the permissible average bond stress) if it is more than

a. 18 bar diameters

b. 24 bar diameters

c. 30 bar diameters

d. 36 bar diameters

22. Pick up the incorrect statement from the following:

a. In the stem of a retaining wall, reinforcement is provided near the earth side

b. In the toe slab of a retaining wall, rein forcement is provided at the bottom of the slab

c. In the heel slab of a retaining wall, rein forcement is provided at the top of the slab

d. None of these.

23. As per I.S. 456 - 1978, the pH value of water shall be

a. less than 6

b. equal to 6

c. not less than 6

d. equal to 7

24. An R.C.C. column is treated as long if its slenderness ratio is greater than

a. 35

b. 40

c. 50

d. 60

25. A stair should not have pitch more than

a. 30°

b. 40°

c. 50°

d. 60°.

26. The floor is rubbed with oxalic acid, for making its surface

a. free from voids

b. glossy

c. durable

d. uniform.

27. If the effective working time is 7 hours and per batch time of concrete is 3 minutes, the output of a concrete mixer of 150 litre capacity, is

a. 15, 900 litres

b. 16, 900 litres

c. 17, 900 litres

d. 18, 900 litres

28. If the various concrete ingredients i.e. cement, sand and aggregates are in the ratio of 1:3:6, the grade of concrete, is

a. M 100

b. M 150

c. M 200

d. M 250

29. 'Shrouding' is essentially provided in

a. strainer type wells

b. cavity type wells

c. slotted type well

d. all the above.

30. Biochemical Oxygen Demand (B.O.D.) of safe drinking water must be

a. nil

b. 5

c. 10

d. 15

31. The sewers

a. must be of adequate size to avoid over flow

b. must flow under gravity  to  full

c. must be laid at least 2 to 3 m deep to collect /water from the basements

d. all the above.

32. 'Cowl' is provided at

a. lower end of ventilating column

b. upper end of ventilating column

c. upper end of the manhole

d. first step in manhole.

33. The field capacity of a soil depends upon

a. capillary tension in soil

b. porosity of soil

c. both (a) and (b)

d. neither (a) nor (b).

34. Lacy's regime condition is obtained if

a. silt grade in the channel is variable

b. discharge in the channel is variable

c. silt charge in the channel is variable

d. channel flows in unlimited, incoherent alluvium of the same character as that transported material.

35. For a standing crop, the consumptive use of water is equal to the depth of water

a. transpired by the crop

b. evaporated by the crop

c. transpired and evaporated by the crop

d. used by the crop in transpiration, evaporation and also the quantity of water evaporated from adjacent soil.

36. A counter berm is

a. a horizontal benching provided on the inside slope

b. a horizontal benching provided on the outside slope

c. a vertical benching provided on the outer edge of the bank

d. a vertical benching provided on the inner edge of the bank.

37. Formation width of a hill road is the distance between

a. outer end of side drain to outer end of parapet wall

b. outer end of side drain to inner end of parapet wall

c. inner end of side drain to outer end of parapet wall

d. inner end of side drain to inner end of parapet wall

38. Traffic census is carried out for

a. road parking study

b. traffic volume study

c. origin and destination study

d. all the above.

39. Indian Road Congress (I.R.C.) was founded and constituted with its head quarters at New Delhi, in

a. 1927

b. 1930

c. 1934

d. 1942

40. At intersection of roads, the traffic volume study is carried out to ascertain the number of vehicles

a. moving along straights

b. turning left

c. turning right

d. all the above.

41. The detention period in a septic tank is assumed

a. 20 minutes

b. 25 minutes

c. 30 minutes

d. 40 minutes

42. The cross-sections for a highway is taken at

a. 30 metres apart

b. intermediate points having abrupt change in gradient

c. the starting end points of the curves

d. All the above.

43. Pick up the correct statement from the following:

a. The actual expenditure involved to complete a work including incidental, establishment and travelling charges, is called actual cost

b. The formal acceptance by the administrative department for incurring an expenditure on the work, is called administrative approval

c. The order of a competent authority sanctioning a properly detailed estimate of the cost of a work of construction or repair is called technical sanction

d. All the above.

44. Pick up the incorrect statement from the following:

a. Dimensions are measured to the nearest 0.01 m

b. Areas are measured to the nearest 0.01 sq.m

c. Cubic contents are measured to the nearest 0.1 cm m

d. Weights are measured to the nearest 0.001 tonnes

45. A critical ratio scheduling

a. determines the status of each activity

b. adjusts automatically changes in activity progress

c. is a dynamic system

d. none of these.

46. An Executive Engineer may have powers upto

a. Rs. 25,000

b. Rs. 50,000

c. Rs. 100,000

d. Rs. 200,000

47. If TL is the latest allowable event occurrence time, total activity slack(s), is equal to

a. LST-EST

b. LFT-EFT

c. TL-EFT

d. all the above.

48. Pick up the correct statement from the following:

a. PERT analysis is event oriented

b. CPM does not make any allowance for the uncertainties in the duration of time

c. In CPM, the time is related to cost

d. All the above.

49. The landing and take off of the air craft is made against the direction of wind. In no case the centre line of the runway should make an angle with the wind direction exceeding

a. 10°

b. 20°

c. 30°

d. 40°

50. The reduced level of the proposed site of an air port is 2500 m above M.S.L. If the recommended length by I.C.A.O. for the runway at sea level is 2500 m, the required length of the runway is

a. 2500 m

b. 3725 m

c. 3000 m

d. 3250 m