***Sub Engineer***

1. In a constant level tube, size of the bubble remains constant because upper wall is

a. of relatively larger radius

b. of relatively smaller radius

c. flat

d. convex downwards.

2. Chain surveying is well adopted for

a. small areas in open ground

b. small areas with crowded details

c. large areas with simple details

d. large areas with difficult details.

3. During secular variation of magnetic meridian at different places

a. range of oscillations is constant

b. period of oscillation is constant

c. range and period of oscillation both vary

d. period of oscillation only varies.

4. Rankine's deflection angle in minutes is obtained by multiplying the length of the chord by

a. degree of the curve

b. square of the degree of the curve

c. inverse of the degree of the curve

d. none of these.

5. In stone masonry, if stones are so placed that their layers are parallel to the direction of load, they

a. split easily

b. are affected by moisture

c. both (a) and (b)

d. none of these.

6. If the ore impurities is

a. clay, lime stone is used as flux

b. lime stone, clay is used as flux

c. quartz, lime stone and argallaceous iron ores are used as flux

d. All the above.

7. Dorry's testing machine is used for

a. crushing test of stone

b. hardness test of stone

c. impact test of stone

d. water absorption test.

8. The process of decarbonising the pig iron completely and then adding proper percentage of carbon for manufacturing steel, is called

a. Cementation process

b. crucible process

c. Bessemer process

d. Open hearth process.

9. For beams breadth is constant,

a. depth d ∝ M

b. depth d ∝ M

c. depth d ∝ 3 M

d. depth d ∝

10. The horizontal thrust on the ends of a two hinged semicircular arch of radius R carrying

a. a uniformly distributed load ω per unit run over its right half span, is

b. a uniformly distributed load ω per unit run over its entire span is

c. a distributed load varying from zero at the left end to ω per unit horizontal run at the right end, is

d. all the above.

11. The deflection of any rectangular beam simply supported, is

a. inversely proportional to its width

b. inversely proportional to the cube of its depth

c. directly proportional to the cube of its length

d. none of these.

12. When equal and opposite forces applied to a body, tend to elongate it, the stress so produced, is called

a. shear stress

b. compressive stress

c. tensile stress

d. transverse stress.

13. The velocity distribution of viscous fluid through a circular/pipe is :

a. hyperbolic

b. circular

c. parabolic

d. elliptical.

14. Practical fluids possess

a. viscosity

b. surface tension

c. compressibility

d. all the above.

15. Mach number is the ratio of inertia force to

a. viscosity

b. surface tension

c. gravitational force

d. elasticity.

16. A jet projected at an angle of 45θ, 40 m from the foot of a vertical column, just reaches the top of the column. The height of the column is

a. 20 m

b. 30 m

c. 40 m

d. 60 m.

17. Negative skin friction on piles

a. is caused due to relative settlement of the soil

b. is caused in soft clays

c. decreases the pile capacity

d. all of the above.

18. A failure wedge develops if a retaining wall

a. moves away from the backfill

b. moves towards the backfill

c. sinks downwards

d. stresses equally by vertical and horizontal forces.

19. A phreatic line is defined as the line within a dam section below which there are

a. positive hydrostatic pressure

b. negative hydrostatic pressure

c. negative equipotential lines

d. none of these.

20. The shearing force acting along the slice of a curved surface of slippage, causes the soil to slide

a. down at the centre

b. down at the toe

c. upward at the centre

d. none of these.

21. The live load to be considered for an accessible roof, is

a. Nil

b. 75 kg/m3

c. 150 kg/m2

d. 200 kg/cm2

22. An R.C.C. roof slab is designed as a two way slab if

a. it supports live loads in both directions

b. the ratio of spans in two directions is less than 2

c. the slab is continuous over two supports

d. the slab is discontinuous at edges.

23. To ensure uniform pressure distribution, the thickness of the foundation, is

a. kept uniform throughout

b. increased gradually towards the edge

c. decreased gradually towards the edge

d. kept zero at the edge.

24. The diameter of main bars in R.C.C. columns, shall not be less than

a. 6 mm

b. 8 mm

c. 10 mm

d. 12 mm

25. Expansion joints in masonry walls are provided if length exceeds

a. 20 m

b. 30 m

c. 40 m

d. 50 m.

26. The dimensions of a half queen closer, are

a. 9 cm x 9 cm x 9 cm

b. 9 cm x 9 cm x 4.5 cm

c. 9 cm x 4.5 cm x 9 cm

d. 1.8 cm x 4.5 cm x 9 cm.

27. If fineness modulus of sand is 2.5, it is graded as

a. fine sand

b. medium sand

c. coarse sand

d. very coarse sand.

28. Pick up the correct statement from the following:

a. Insufficient quantity of water makes the concrete unworkable

b. Excess quantity of water makes the concrete segregated

c. Excess quantity of water causes bleeding in concrete

d. All the above.

29. Pick up the correct statement from the following :

a. Average consumption of commercial use of water is 25% of total consumption

b. Consumption of water on public use is 10% of total consumption

c. Waste water and leakage is 15% of total consumption

d. all the above.

30. In a rapid sand filter, air binding is caused due to excessive

a. negative pressure

b. pressure

c. turbidity

d. water pressure

31. For trunk and out-fall, the type of sewers generally used, is

a. circular shaped

b. horse shoe shaped

c. parabolic shaped

d. semi-elliptical shaped.

32. The ratio of maximum sewage flow to average sewage flow for mains up to 1 m in diameter, is

a. 1.5

b. 2.0

c. 3.0

d. 4.0

33. A water-logged land is found suitable for cultivation due to

a. ease of tillage for preparation of the field for optimum condition of germination

b. absence of aeration of soil from root zone of the plant

c. regular supply of water to plants from the water table by capillary action

d. none of these.

34. Disposal of extra excavated earth of canals, is utilised to provide a spoil bank on

a. left side

b. right side

c. both sides

d. all the above.

35. Pick up the incorrect statement from the following

a. A bank in the cross-section of a canal is provided on the right

b. A bank in the cross-section of a canal is provided on the left

c. A 'dowla' in the cross-section of a canal is provided on the left

d. None of these.

36. While deciding the alignment of a proposed canal taking off from a river at B, three alignments of approximately equal lengths are available. These cross a drainages at C1, C2 and C3 where drainage bed levels C1 > C2 > C3, you will provide at site C3

a. an aqueduct

b. a syphon aqueduct

c. a super passage

d. a syphon.

37. Retaining walls are generally constructed with dry rubble stones with 60 cm top width and

a. 1 : 2 front batter

b. 1 : 3 front batter

c. 1 : 4 front batter

d. 1 : 5 front batter

38. The total length of a valley formed by two gradients - 3% and + 2% curve between the two tangent points to provide a rate of change of centrifugal acceleration 0.6 m/sec2, for a design speed 100 km ph, is

a. 16.0 m

b. 42.3 m

c. 84.6 m

d. none of these.

39. Road width 8.8 m of two lane National highways or State highways in mountainous terrain

a. excludes the width of parapet (0.6 m)

b. excludes the width of side drain (0.6 m)

c. excludes the width of parapet and side drain

d. includes the width of parapet and side drain

40. Pick up the incorrect statement from the following:

a. Tresguet did not provide the top camber for the drainage of surface water

b. Tresguet provided the top camber for the drainage of surface water

c. Telford provided two layers of stones in the central 5.4 m width and one layer was provided on the sides

d. Macadam provided a camber to the formation at the dug-up state, to drain percolated water.

41. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

42. The expected out turn of cement concrete 1 : 2 : 4 per mason per day is

a. 1.5 m3

b. 2.5 m3

c. 3.5 m3

d. 5.0 m3

43. Pick up the correct statement from the following:

a. Additional supervising staff engaged at work site, is called work charged establishment

b. Detailed specifications specify qualities, quantities and the proportions of materials to be used for a particular item

c. The cost per unit at which the article can be procured, from the open market at a given time, is called 'market rate'

d. All the above.

44. For 100 sq. m cement concrete (1 : 2: 4) 4 cm thick floor, the quantity of cement required, is

a. 0.90 m3

b. 0.94 m3

c. 0.98 m3

d. 1.00 m3

45. PERT is

a. limited of event oriented diagrams

b. used for research and development projects

c. based on three time estimates for activities linking up two events

d. all the above.

46. Power stations are generally treated as

a. light construction

b. heavy construction

c. industrial construction

d. electrical construction.

47. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

48. If TL is the latest allowable event occurrence time, total activity slack(s), is equal to

a. LST-EST

b. LFT-EFT

c. TL-EFT

d. all the above.

49. The runway orientation is made so that landing and take off are

a. against the wind direction

b. along the wind direction

c. perpendicular to wind direction

d. none of these.

50. In instrument landing system

a. L.O.M. and L.M.M. are installed on one side and Localizer antenna on the other side

b. L.O.M. and Localizer are installed on one side and L.M.M. on the other side

c. Localizer and L.M.M. are installed on one side and L.O.M. on the other side

d. None of these.