***Sub Engineer***

1. If h1 and h2 are the differences in level between ground and the formation levels, m is the slope of the sloping sides. D is the distance between the cross sections then, prismoidal correction for a level section is

a. D/3m(h1 - h2)

b. D/6m(h1 - h2)2

c. D/6m(h1 - h2)3

d. D/6m(h1 + h2)2

2. The latitude of a traverse leg is obtained by multiplying its length by

a. tangent of its reduced bearing

b. sign of its reduced bearing

c. cosine of its reduced bearing

d. cosecant of its reduced bearing.

3. Pick up the correct statement from the following :

a. in Galileo's telescope, the rays from the object get refracted at the objective and are intercepted by the eyepiece before a real image is formed

b. a line passing through the optical centre of the objective traversing through the eyepiece, is called line of sight

c. the line of sight which passes through the intersection of cross-lines marked on a diaphragm fixed in front of the eyepiece in a plane at right-angles to the axis, is called the line of collimation

d. all the above.

4. In a perfect prismatic compass

a. ends of the needle and pivot are in same vertical and horizontal planes

b. pivot is vertically over the centre of the graduated circle

c. needle is always kept sensitive

d. all the above.

5. Pick up the most favourable condition for the rapid growth of fungus for dry rot from the following:

a. dampness

b. presence of sap

c. stagnant air

d. All the above,

6. Gniess is obtained from

a. igneous rocks

b. metamorphic rocks

c. sedimentary rocks

d. sedimentary metamorphic rocks.

7. The portion of the brick cut across its width and having its length equal to that of a full brick, is known as

a. queen closer

b. king closer

c. prince closer

d. none of these.

8. A stone is rejected if it absorbs water more than

a. 10%

b. 15%

c. 20%

d. 25%.

9. The equivalent length of a column of length L, having both the ends hinged, is

a. 2L

b. L

c.

d.

10. The point of contraflexure is the point where

a. B.M. changes sign

b. B.M. is maximum

c. B.M. is minimum

d. S.F. is zero.

11. The ratio of strengths of solid to hollow shafts, both having outside diameter D and hollow having inside diameter D/2, in torsion, is

a. 1/2

b. 1/16

c. 15/16

d. 3/8

12. The radius of gyration of a rectangular section is not proportional to

a. square root of the moment of inertia

b. square root of the inverse of the area

c. square root of the moment of inertia divided by area of the section

d. none of these.

13. For critical flow, the Froude number is :

a. 1.0

b. less than 1.0

c. more than 1.0

d. 2

14. If cohesion between the molecules of a fluid is more than adhesion between the fluid and glass, the free level of fluid in a dipped glass tube will be

a. higher than the surface of liquid

b. same as the surface of liquid

c. lower than the surface of liquid

d. none of these.

15. The dimensions MLT-2 refers to

a. specific weight

b. force

c. discharge

d. none of these.

16. To avoid an interruption in the flow of a syphon, an air vessel is provided

a. at the inlet

b. at the outlet

c. at the summit

d. at any point between intet and outlet.

17. The change of moisture content of soils, changes the

a. value of the angle of repose

b. amount of compaction required

c. cohesive strength of soil

d. all the above.

18. Through a point in a loaded soil mass, there exists n typical planes mutually orthogonal on which the stress is wholly normal and no shear stress acts, if n is

a. 1

b. 2

c. 3

d. 4

19. Terzaghi's analysis assumes :

a. soil is homogeneous and isotropic

b. elastic zone has straight boundaries inclined at ψ = φ to the horizontal and plastic zones fully developed

c. failure zones do not extend above the horizontal plane through the base of the footing

d. all the above.

20. The angle of internal friction, is least for

a. angular -grained dense sand

b. round-grained loose sand

c. round-grained loose sand

d. clays.

21. Pick up the correct statement from the following:

a. A pile is a slender member which transfers the load through its lower end on a strong strata

b. A pile is a slender member which transfers its load to the surrounding soil

c. A pile is a slender member which transfers its load by friction

d. A pile is a cylindrical body of concrete which transfers the load at a depth greater than its width.

22. The weight of a foundation is assumed as

a. 5% of wall weight

b. 7% of wall weight

c. 10% of wall weight

d. 12% of wall weight

23. Enlarged head of a supporting column of a flat slabs is technically .known as

a. top of the column

b. capital

c. drop panel

d. none of these.

24. The diameter of transverse reinforcement of columns should be equal to one-fourth of the diameter of the main steel rods but not less than

a. 5 mm

b. 6 mm

c. 7 mm

d. 8 mm

25. A wooden block hinged on post outside a door, is known

a. cleat

b. stop

c. horn

d. none of these.

26. The skirting/dado in a bath roof should be upto

a. ceiling

b. 15 cm above floor level

c. 200 cm

d. level of the tap.

27. Pick up the correct statement from the following:

a. The quantity of water required for concreting, depends upon the grading of aggregate amd method of compaction

b. 10% excess of water reduces the strength of concrete by 15%

c. 30% excess of water reduces the strength of concrete by 50%

d. All the above.

28. The dimensions of a 35 litre forma for measuring aggregates by volume, are :

a. length 39 cm, breadth 25 cm, height 32 cm

b. length 27 cm, breadth 27 cm, height 48 cm

c. length 220 cm, breadth 25 cm, height 40 cm

d. none of these.

29. The head against which the motor works for lifting water, is

a. maximum depression head

b. velocity head

c. frictional losses in the delivery pipe

d. all the above.

30. Total flow in stream is known

a. run off

b. stream flow

c. discharge

d. all the above.

31. If D.O. concentration falls down to zero in any natural drainage, it indicates the zone of

a. active decomposition

b. recovery

c. cleaner water

d. none of these.

32. Pick up the incorrect size of stone ware sewers

a. 67.5 cm

b. 82.5 cm

c. 90 cm

d. none of these.

33. A current meter measures the velocity of flow, if it is held

a. on the surface of water

b. at the bottom of channel

c. at any point within the cross-section

d. none of these.

34. The field capacity of a soil depends upon

a. capillary tension in soil

b. porosity of soil

c. both (a) and (b)

d. neither (a) nor (b).

35. The ratio of the discharge over a trapezoidal crest to a rectangular crest of Sarda falls of identical parameters, is

a. 1.084

b. 1.074

c. 1.064

d. 1.054

36. The optimum depth of kor watering for a rice crop, is

a. 19.0 cm

b. 17.5 cm

c. 13.5 cm

d. 12.0 cm

37. For a poorly graded sub-grade soil, thickness of sub-base, is

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm

38. The best compromise between the increase of the length of a highway and reduction in its load carrying capacity, is the ruling gradient

a. 1 in 15

b. 1 in 20

c. 1 in 25

d. l in 30

39. Pick up the correct statement from the following:

a. During reconnaissance, the general route of the alignment is selected

b. After reconnaissance, a trace is cut for the alignment

c. Last stage is the detailed surveys for desired geometries'of the highway

d. All the above.

40. Pick up the correct statement from the following:

a. The corss fall of the shoulder should be at least 0.5% steeper than camber

b. On superelevated sections, the shoulders should be provided a cross fall equal to camber

c. Earthern roads in general are provided steepest crossfall

d. All the above.

41. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

42. Pick up the incorrect statement from the following:

a. Dimensions are measured to the nearest 0.01 m

b. Areas are measured to the nearest 0.01 sq.m

c. Cubic contents are measured to the nearest 0.1 cm m

d. Weights are measured to the nearest 0.001 tonnes

43. Pick up the correct statement from the following:

a. The actual expenditure involved to complete a work including incidental, establishment and travelling charges, is called actual cost

b. The formal acceptance by the administrative department for incurring an expenditure on the work, is called administrative approval

c. The order of a competent authority sanctioning a properly detailed estimate of the cost of a work of construction or repair is called technical sanction

d. All the above.

44. The item of the brick structure measured in sq.m, is

a. Reinforced brick work

b. Broken glass coping

c. Brick edging

d. Brick work in arches.

45. PERT analysis is based on

a. optimistic time

b. pessimistic time

c. most likely time

d. all the above.

46. The main advantage of line organisation, is :

a. defined responsibilities at all levels

b. rigid discipline in the organisation

c. ability of quick decision at all levels

d. all the above.

47. The time which results in the leasi, possible construction cost of an activity, is known

a. normal time

b. slow time

c. crash time

d. standard time.

48. Pick up the correct statement from the following:

a. PERT analysis is event oriented

b. CPM does not make any allowance for the uncertainties in the duration of time

c. In CPM, the time is related to cost

d. All the above.

49. Pick up the correct statement from the following:

a. Approximate geometric centre of the landing area, is called air port reference point

b. The boundaries of horizontal surface and conical surface are marked with reference to air port reference point

c. The location of the air port on a map is the position of air port reference point

d. All the above.

50. The meterological condition which influences the size and location of an air port is

a. air density

b. reduced level

c. wind direction

d. all the above.