***Sub Engineer***

1. If the angular measurements of a traverse are more precise than its linear measurements, balancing of the traverse, is done by

a. Bowditch's rule

b. Transit rule

c. Empirical rule

d. all of the above.

2. Perpendicular offset from a tangent to the junction of a transition curve and circular curve is equal to

a. shift

b. twice the shift

c. thrice the shift

d. four times the shift.

3. A dumpy level was set up at the midpoint between two pegs A and B, 50 m apart and the staff readings at A and B were 1.22 and 1.06. With the level set up a A, the readings at A and B were 1.55 and 1.37. The collimation error per 100 m length of sight is

a. 0.02 m inclined upwards

b. 0.04 m inclined downwards

c. 0.04 m inclined upward

d. none of these.

4. The imaginary line passing through the intersection of cross hairs and the optical centre of the objective, is known as

a. line of sight

b. line of collimation

c. axis of the telescope

d. none of these.

5. The process of manufacturing steel by heating short lengths of wrought iron bars mixed with charcoal in fire clay crucibles and collecting the molten iron into moulds, is known as

a. Cementation process

b. Crucible process

c. Bessemer process

d. Open hearth process.

6. Varnish is a transparent or semi-transparent solution of resinuous substances in

a. alcohol

b. linseed

c. turpentine

d. all the above.

7. Quartzite, a metamorphic stone is

a. brittle

b. crystalline

c. compact

d. All the above.

8. The timber having maximum resistance against white ants, is obtained from

a. chir

b. shisham

c. sal

d. teak.

9. m1 and m2 are the members of two individual simple trusses of a compound truss. The compound truss will be rigid and determinate if

a. m = m1 + m2

b. m = m1 + m2 + 1

c. m = m1 + m2 + 2

d. m = m1 + m2 + 3

10. A steel rod 1 metre long having square cross section is pulled under a tensile load of 8 tonnes. The extension in the rod was 1 mm only. If Esteel = 2 x 106 kg/cm2, the side of the rod, is

a. 1 cm

b. 1.5 cm

c. 2 cm

d. 2.5 cm

11. In a three hinged arch, the third hinge is generally kept at

a. crown of the arch

b. midpoint of the crown and left support hinge

c. midpoint of the crown and right support hinge

d. none of these.

12. The reaction at the supports will be vertical to the plane of the support if the frame structure rests on

a. roller supports

b. free supports

c. hinged supports

d. all the above.

13. An independent mass of a fluid does not posses

a. elevation energy

b. kinetic energy

c. pressure energy

d. none of these.

14. The centre of pressure of a vertical plane immersed in a liquid is at

a. centre of higher edge

b. centre of lower edge

c. centroid of the area

d. none of these.

15. A pipe of 0.1 m2 cross sectional area suddenly enlarges to 0.3 m2 cross-sectional area. If the discharge of the pipe is 0.3 m3 /sec, the head loss is

a. 2/g m of water

b. g/2 m of water

c. 1g m of water

d. g m of water.

16. In an inclined position, a venturimeter records

a. more reading

b. less reading

c. same reading

d. none of these.

17. A moist soil sample weighing 108 g has a volume of 60 cc. If water content is 25% and value of G = 2.52, the void ratio is

a. 0.55

b. 0.65

c. 0.75

d. 0.80

18. A grillage foundation

a. is provided for heavily loaded isolated columns

b. is treated as spread foundation

c. consists of two sets of perpendicularly placed steel beams

d. all the above.

19. The consistency index of a soil is defined as the ratio of

a. liquid limit plus the natural water content to the plasticity index of the soil

b. liquid limit minus the natural water content to the plasticity index of the soil

c. natural water content of a soil minus plastic limit to the plasticity index of the soil

d. natural water content of a soil plus its plastic limit to the plasticity index of the soil.

20. The consolidation time for soils

a. decreases with increasing permeability

b. increases rapidly with increasing size of soil mass

c. is independent of the magnitude of the stress change.

d. All the above.

21. The transverse reinforcements provided at right angles to the main reinforcement

a. distribute the load

b. resist the temperature stresses

c. resist the shrinkage stress

d. all the above.

22. If the permissible compressive stress for a concrete in bending is C kg/m2, the modular ratio is

a. 2800/C

b. 2300/2C

c. 2800/3C

d. 2800/C2

23. In a simply supported slab the minimum spacing of distribution reinforcement, should be four times the effective thickness of the slab or

a. 30 cm

b. 40 cm

c. 50 cm

d. 60 cm

24. The length of lap in tension reinforcement should not be less than the bar diameter x (actual tension / four times the permissible average bond stress) if it is more than

a. 18 bar diameters

b. 24 bar diameters

c. 30 bar diameters

d. 36 bar diameters

25. Dutch bond is a modification of

a. English bond

b. stretcher bond

c. header bond

d. single Flemish bond.

26. In case of Raymond pile

a. diameter of top of piles varies from 40 cm to 60 cm

b. diameter of pile at bottom varies from 20 cm to 28 cm

c. thickness of outer shell depends upon pile diameter

d. all the above.

27. The rock which is not calcareous, is :

a. macl

b. chalk

c. laterite

d. none of these.

28. Hardening of cement occurs at

a. rapid rate during the first few days and afterwards it continues to increase at a decreased rate

b. slow rate during the first few days and afterwards it continues to increase at a rapid rate

c. uniform rate throughout its age

d. none of these.

29. Water supply system includes

a. digging a well for water

b. construction of dams

c. construction of canals

d. entire arrangement from source to distribution.

30. The detention period for plain sedimentation water tanks, is usually

a. 4 to 8 hours

b. 8 to 16 hours

c. 16 to 24 hours

d. 24 to 36 hours.

31. If γw is the unit weight of water, r the hydraulic mean depth of the sewer and S the bed slope of the sewer, then the tractive force exerted by flowing water, is

a. γw . r . S

b. γw r1/2 . S

c. γw r S

d. γw r2/3 S

32. A manhole is generally provided at each

a. junction

b. change of gradient

c. change of sewer diameter

d. all the above.

33. If the straight sides of a triangular section of a lined canal with circular bottom of radius R, make an angle θ with horizontal, the perimeter of the canal is

a. 2R(θ + tan θ)

b. R(θ + cos θ)

c. 2R(θ + cos θ)

d. none of these.

34. Retrogression of the bed level of a river downstream a weir, occurs due to

a. heavy impact of water

b. increase of the bed level

c. less percentage of silt

d. soft soil strata.

35. For a standing crop, the consumptive use of water is equal to the depth of water

a. transpired by the crop

b. evaporated by the crop

c. transpired and evaporated by the crop

d. used by the crop in transpiration, evaporation and also the quantity of water evaporated from adjacent soil.

36. The sensitivity of a rigid module, is

a. 1.50

b. 1.00

c. 0.50

d. zero

37. The width of road pavements, depends upon

a. width of traffic lane

b. number of traffic

c. width of median strip

d. all the above.

38. Length of vehicles does not affect

a. minimum radius of turning

b. passing sight distance

c. width of shoulders

d. none of these.

39. Thickness of broken line markings on multi-lane road for lanes is generally kept

a. 10 cm

b. 12 cm

c. 15 cm

d. 18 cm

40. If the radii of a compound curve and a reverse curve are respectively the same, the length of common tangent

a. of compound curve will be more

b. of reverse curve will be more

c. of both curves will be equal

d. none of these.

41. Pick up the incorrect statement regarding a master trap from the following :

a. It is provided a cleaning eye at the top of the trap

b. The height of fresh air inlet pipe fixed vertically with wall is 3 m

c. The mica flap valve which opens inwards only, is fitted at the top of the inlet pipe

d. The water seal is less than that of ordinary traps.

42. The area is measured correct to the nearest

a. 0.01 sqm

b. 0.02 sqm

c. 0.03 sqm

d. 0.04 sqm

43. The trap which is provided to disconnect the house drain from the street sewer is called

a. Intercepting trap

b. Interception manhole

c. Intercepterchamber

d. All the above.

44. The detention period in a septic tank is assumed

a. 20 minutes

b. 25 minutes

c. 30 minutes

d. 40 minutes

45. Railway projects are treated as

a. light construction

b. heavy construction

c. industrial construction

d. none of these.

46. Pick up the incorrect statement from the following:

a. The beginning and end of a job, are called events

b. The activity which consumes maximum time, is called a node

c. Logically and sequentially connected activities and events form a network

d. None of these.

47. Military organisation is known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. none of these.

48. Completion of an activity on CPM network diagram, is generally known

a. Event

b. Node

c. Connector

d. All the above.

49. Pick up the incorrect statement from the following:

a. In single engine aeroplanes, the engine is provided in the nose of the aircraft

b. In double engine aeroplanes, one engine on either wing is placed symmetrically

c. In three engine aeroplanes, two engines are placed on both wings and one engine is placed in the tail

d. None of these.

50. Pick up the correct statement from the following:

a. The speed of the aircraft relative to the ground, is called cruising speed

b. The speed of the aircraft relative to wind, is called air speed

c. When wind is blowing the direction of the flight, air speed is less than cruising speed

d. All the above.